mulators, A and B, and these are indicated by using . A or . B respectively.

Words are defined by the bracketing Hc: and ; Hc defining words, which basically perform the same functions as : and ; in Forth, or CODE and END-CODE in Forth assemblers. The assembly address of a FIC: name is left on the stack when the name is invoked. For subroutines, we have also developed the defining words SUB: and ; SUB which have the following characteristics:

; SUB compiles an RTS, instruction before doing the "enddefinition" work of ; Hc, and SUB: makes a header similar to HC: with the additional DOES> property of compiling a JSR to the name's address.

This means that a word defined by SUB: can be invoked simply by its name. Le., it does not need to be invoked by saying NAME JSR, which is what HC: names require. This has the net effect of making the code look very much like highlevel Forth. For example, if we have SUB: routines like:

SUB: OVER	code instructions	;SUB

SUB: SWAP	code instructions	;SUB

SUB: +	code instructions 	;SUB

51

... we can simply write, in a subsequent definition:

SUB: NEW-WORD	SWAP OVER + (etc.) 	SUB

In fact, the code becomes almost simultaneous Forth and assembler, in that it's possible to have a set of statements that look like Forth, and then have opcodes one after another as in assembler. For example, at the end of the word MAKE - U D P, we have:

CRC16 LDB, FF # B EOR, iBCRC LDA, FF # A EOR, PSHD SWAP ! "UDPBUFF" SWAP OVER C@ "X' + ;SUB

The first two lines are almost pure assembly, but the last is a lot like Forth.

2.2 The iButton Code-The Bits

With those preliminaries out of the way, we can now proceed to examine the code in detail, After the Forth kernel and the two CRC lookup tables, the iB code proper starts. First of all, we name the port line (an address, and a bit mask) and its associated Direction Register line (again, an address

Forth Dimensions XXI.1,2