All that is required now is to arrange to repeat the

two-instruction sequence the requisite number of times, For

7    6   5  4    3   2   1   0	CF	simplicity, let us do this using the most elementary looping

instruction, LOOP. We must place the number of times the

loop is to be executed in the register Cx, then at the end of the

loop issue the LOOP instruction which will decrement cx by 1

and loop back to the starting point (which we must label some

SHR		how-we will return to this point and describe how it is done) '

as long as ex is non-zero. That is, it will loop the number of

times specified by the integer in cx,

7   6    5   4   3   2    1   0	G		To assemble this subroutine using an assembler like

MASM@ or TASM@, we would prepare a text file of the form:

POP BX	; get n

PO P CX	; get # of iterations

XOR DX, DX	; set n' = 0

HERE:	; beginning of loop

SHR BX, 1	; send O'th bit of n to CF

RCL				and shift right 1 place

RCL DX, 1		shift n' left and

move	C17 into Oth bit of n'

Different Forths will require minor differences in how we		LOOP HERE		Cx=cX-1,	loop if CX > 0.

proceed. Several commercial Forths cache the top of the data		PUSH DX		leave	result on stack

stack in the register BX, thereby eliminating some pushes and

pops. The public-domain F-PC, on the other hand, leaves BX	(however, as we shall see below, there will need to be some

free. Since we are illustrating with F-PC, our first job will be to	necessary boilerplate lines that conform to the particular

obtain the argument n; we therefore BX it from the stack to BX:	assembler's conventions, as well as respecting the calling con

ventions of the high-level language we are going to use the

POP BX	subroutine with).

To test the assembly language program with F-PC's intrin

Next let us assign the (unused) DX register to the	sic assembler, we modify it slightly (to conform to the latter's

bit-reversed answer; we initialize DX to 0 quickly using bit-	notational conventions), obtaining [Listing Three].

wise exclusive-or [81		An assembler written in Forth is simple because the mne

monics are actually IMMEDIATE words that execute during

XOR DX, DX	assembly, placing the appropriate operation codes in the pa

rameter field of the word being defined. In the F-PC assem- 1

Now we shift BX one place to the right using SHR; the	bler, the LOOP mnemonics (LOOP, LOOPZ, LOOPNz, etc.) ex

rightmost bit, as the Figure suggests, moves from the register	pect a number on the stack, which is actually the address

to the Carry Flag. Then we RCL the DX register one place to	they loop back to (or not, depending whether an appropriate

the left; the bit formerly in the Carry Flag becomes the right-	condition is satisfied). This can be supplied by an explicit

most bit Of DX. The left-most bit (if any) of DX ends up in the	label or, as in the above example, we may simply say HERE,

CF. But that does not matter, because it will be replaced by	which places on the stack the address of the next piece of

the right-most bit of Bx when the sequence is repeated. So	code to be assembled; this is the very point we want to loop

the machine-language program (with comments) looks like	back to, hence LOOP enters the Intel opcode for LOOP, together 1

[Listing Two].	with that address.		We now enter the subroutine from

8, The instruction MOV DX, # 0 would also work, but requires one byte		the keyboard and test the result.

more storage.

Listing Three		CODE STIB <er> ok

POP BX <er> ok

CODE	STIB	reverse bit-order	POB CX <er> POB <-WHAT?

POP BX		get n

POP CX		get # of iterations

XOR DX, DX	\	set n' = 0	Oops! A typo, do it again. just in case,

HERE	\ beginning of loop	FORGET from STIB on:

SHR BX, # 1	send O'th bit of n to CF

and shift right 1 place	FORGET STIB <-WHAT?

RCL DX, # 1	shift n' left and

move CF into O'th bit of n'	CODE STIB <er> ok

LOOP	Cx=cX-1, loop if CX > 0.	POP BX <er> ok

PUSH DX	leave result on stack	POP CX <er> ok

NEXT		END-CODE	terminate definition	XOR DX, DX <er> ok

HERE <er> ok

Forth Dimensions XX11,2					7

SHR BX, # 1 <cr> ok	9 the comments would have to be preceded with a semico

RCL DX, # 1 <cr> ok		lon ; rather than Forth's traditional backslash \;

LOOP <or> ok	0 the word HERE must be converted to a loop label;

PUSH DX <cr> ok	0 a standard header must be added, and the definition

NEXT <or> ok		termination also changes.

END-CODE <cr> ok

[See Listing Four.]

This all looks like it entered correctly-at least the assem

bler did not burp. The proof of the pudding, however, is in	Case conversion

the eating:			Many languages contain a library function for converting

4 7 STIB	<or> 14 ok	a string to all upper-case letters or all lower-case ones, leav

4 14 STIB	<or> 7 ok	ing digits and punctuation alone. The new Forth ANS stan

dard [9] happens not to require such a routine, although most

Eureka! No warts this time.	Forths contain a word analogous to UCASE as part of the corn

If an assembly language version of STIB were needed for	9.	A copy of the final draft of the ANS Forth Standard document, X3114

linking with a BASIC or C program, some minor modifica-		dpANS-6 can be downloaded in several different machine-readable

tions would be necessary:		formats, including F-PC hypertext, Microsoft Word , or HTML, from the

Web site www.taygeta.com.

Listing Four

Code segment word public 'CODE'	define the code segment

assume cs: Code

public STIB	allow any routine to call it

STIB proc near	reverse bit-order

POP BX	get n

POP CX	; get # of iterations

XOR DX, DX ; zero n'

HERE: ; label beginning of loop

SHR BX, 1 ; 01th bit -> CF, shift right

RCL DX, I ; n' : shift left, CF -> 0 1 th bit

LOOP HERE		CX=CX-l, loop if CX > 0.

PUSH DX		leave result on stack

RET		return from function call

STIB	endp		terminate definition

Code	ends

end

Listing Five

Icase?	char -- flag) \ true if lower case

DUP (CHAR] a <	char fl)	\ true if char < "all

SWAP [CHAR] z >	fl f2)	\ true if char > "z"

OR	not[flag]	combine flags

NOT		logical not

UCASE	beg len)

0 DO		work from left	to right thru string

DUP C@	( -- adr char)	\ get character

DUP lcase?			adr char flag)

32 AND			adr char 32 if lcase 1 0 else)

subtract 32 from lcase letters only

OVER C!		replace modified character

1+	LOOP		increment address by 1 and loop

DROP ;	\	clean up stack

Listing Six

lcase?	( char	--	flag)

DUP	[CHAR]	a	<	char flagl)

SWAP	[CHAR]	z	>	flagl flag2)

OR		not[ flag]

NOT		flag)

8				Forth Dimensions XXI,1,2

piling mechanism.		ing. The actual switch from lower to upper case is accomplished

The first step is to choose our approach. In Microsoft Quick	by subtracting 32d from the ASCII character code of the letter,

Basic@ (QB), a string of N characters is stored in a contiguous	since the upper-case letters have codes 32d smaller than their

sequence of N bytes of memory in the default data segment.	corresponding lower-case values. It is worth noting that in the

It is referenced by a 4-byte string descriptor, with the first	words icase? and UCASE the programming style computes

two bytes containing the length as a signed 16-bit integer,	the result rather than deciding it [11]. That is, while it is not

and the second two bytes the offset of the beginning of the	always practical to avoid decisions [12], good style eschews

string in the data segment. That is, Quick Basic strings can be	branches wherever possible.

up to 32 Kb long. Microsoft C stores strings in contiguous		We test the Forth version using non-Standard but com

segments of N+1 bytes with theN+lst byte containing 0 (stan-	mon words $11 (save a string from the keyboard to temporary

dard terminator), strings being referenced by the address of	storage) and $ . (print a counted string at address adr to the

their first byte.		screen) [13]:

Forth, by contrast, usually deals in counted strings up to		$" this + is A % Icase STringl' COUNT UCASE

256 bytes long, whose count is contained in the first byte. These		'PAD $. THIS + IS A % LCASE STRING ok

differences between languages present a minor problem in

designing subroutines that manipulate strings, since they will		The assembly language version is easy to construct. Begin

not work the same in Forth as in QB or C. The easiest method	with Icase? [Listing Six] and recode directly in assembly lan

is to write the code in two pieces: a language-specific header	guage [141 [see Listing Seven].

and a universal body. We illustrate with headers for Forth, Quick		Test it. (Note: "true"-all bits set to 1- is assumed to be -1 in

Basic and C string-Storage conventions.	this example, an illustration of "environmental dependency,"

What of the body code? If we write it first in high-level	i.e., code that assumes two's-complement integer arithmetic.)

Forth the design becomes clear [101 [see Listing Five]. That is,		CHAR A DUP	lcase? 65 0 ok

we step through the string a byte at a time from beginning to		CHAR a DUP	lcase? 97 -1 ok

end, testing whether the character is a lower-case letter or		CHAR z DUP . lcase? . 122 -1 ok

"other." If lower case, change to upper case; otherwise do noth-		CHAR & DUP . lcase? . 38 0 ok

10. The ANS Forth word WITHIN could have been used for this test, as in		working with ASCII codes, i.e., integers in the range 0-255, we can save

lcase?	char --- flag=true if char is lower case)		register space by using eight-bit operations rather than 16- or 32-bit

[ CHAR] a	[ CHAR) z 1+ WITHIN ;		ones. (Of course, when PusHing or Popping to/from the data stack, the

operations appropriate to the cell size of the stack must be used.)

(The extra 1+ is required because ANS Forth defines WITHIN 50 that it	15. An alternate assembly definition of icase? employing jurrips is

returns TRUE if the limits satisfy n, s char < n2.) However, this would	CODE lcase?	char -- f)

not illustrate directly how to turn primitive Forth operations into CODE.	POP	BX	char -> BL

XOR	AX, AX	AX = false

11,		Note that, in this example, the use of a branching construct (IF

CMP	BL, # 97	< a ?

ELSE ... ENDIF) eliminates the need to store a byte if the character were

upper case or a non-letter. That is, we could say	JL	DONE

e	IF 32 -	SWAP C ! ELSE	DROP ENDIF	CMP BL, # 122	\ > Z ?

JG	DONE

Howev1ecr,atsests reveal that most text input is predominantly lower case;		NOT AX	AX = true

hence, the time consumed in the branch dominates the unnecessary	DONE:

store operations.				PUSH	AX	flag -> TOS

12.		J.V, Noble, Computers in Physics, Jul/Aug 1991, p. 386.		NEXT		END-CODE

13.		If your Forth lacks $'I and $. here are their definitions:	whereas we used no jumps to perform the tests in the previous version.

: $.,	( CHAR1 " WORD	PAD OVER C@ 1+ CMOVE PAD ;	The jumpless version assembles to 18 bytes, whereas the one with

Read text up to a terminating " then move it to PAD						i

jumps requires 18. That is, although the version with jump instructions i

\(temp storage); leave the string's new address.	looks much shorter than the branchless version, it is actually nearly the ~

same length. And eliminating jumps can reduce the likelihood of

$ .	adr-)	COUNT TYPE	having to dump the pipeline.

14. Several things to keep in mind about the translation: since we are

listing Seven

CODE Icase?			char -- flag)

POP BX		char -> BL

MOV AX, BX		copy to AL

SUB AL, # 96		AL = char - 96

CBW		sign AL -> AH	flagl

XCHG AX, BX		interchange registers BH	flagl

SUB AL, # 123		AL = char - 123

CBW		sign AL -> AH = flag2

OR AH, BH		AH = flagl or flag2

xe HG AL, AH		AL = -flag

NO T AL		AL = flag

CBW	\		convert B- to 16-bit flag

PUSH AX		flag -> TOS

NEXT END-CODE		terminate definition

Forth Dimensions M.1,2

The preceding test went well-we can test efficiently		Upon exiting, we restore DI with BX Di, as the last instruc

whether a character is lower case [15]. To proceed, we next	tion preceding NEXT END-CODE. Conversely, a header suitable

require a looping construct. The one we used in STIB will do	for Quick Basic would look like [16] [see Listing Eight] and the

fine, because once again the loop will execute a predetermined	corresponding QB footer (to exit gracefully) would be

number of times. Again we must provide header code that

places the count (string length, in bytes) in the CX register,		POP DI

and the address of its first byte in BX. This time, however, we		POP BP	; restore registers

identify the header code as a separate section of the assem

bler subroutine, in order to be able to replace it later with an		The complete program in F-PC assembler then becomes

appropriate equivalent that respects the conventions of a lan-	[Listing Nine].

guage other than Forth.		The subroutine is h , ard to read even with indented com

In F-PC the header will consist of the instructions:	ments (which is why we prefer high-level language to assem

bler), but it consists of the same parts as the high-level defi-

POP CX	\ count in CX	nition: a SETUP section that gets the count and origin of data;

POP BX	\ beginning of data in BX	a body that LOOPS through the string; a test that determines

PUSH DI	\ save DI (index) register	whether a character is a lower-case letter, and if so, modifies

MOV DI, BX	\ start-1 in DI	it to upper case; and a "footer" that restores whatever regis

ters have been saved on the stack and exits gracefully. Note

we were able to eliminate three redundant instructions:

1	16.	Note: if we were trying to generate the same function for linking to C,

we would have to take into account the zero-terminated structure of	XCHG AL, AH

strings in C, probably using a different looping method, since the	CBW

count would not be readily available.

16.	Abrash, op. cit., discusses in detail the pitfalls of assuming the	PUSH AX

instruction timings given by Intel.

17. Actually the test was performed on a 3086SX-33 machine.	whose only purpose in the CODE version of Icase?was to

Convert an 8-bit flag to a 16-bit inte

ger that could be left on the stack.The

code for UCASE is about as terse as such

Listing Eight					a routine can be made.

PUSH BP			; save BP		Since assembler supposedly provides

MOV BP, SP		use BP as a stack pointer	raw speed, it is interesting to examine

PUSH DI				save DI register	timings [16]. Looking up the number

of clock cycles per instruction for the

MOV BX, 6 BPI	; address of string descriptor to BX reg	Intel 80286 [171, we find [Listing Ten],

; Note: don't need to initialize CX		The instructions labeled "assembler

MOV CX, 0 BX]	; count in CX reg	directive" execute during compilation

ADD BX, 2	; offset to string origin in BX	and carry no run-time overhead. Since

the header and footer are executed

Listing Nine

once, their 25 clock cycles are imma- 1

CODE UCASE		start header	terial for reasonably long input strings.

POP CX			\ get count	Converting a lower-case to an upper

POP BX			\ get origin	case letter evidently requires 42 clock

PUSH DI			\ save DI	cycles, i.e., about 1.3 [tsec on a 33 MHz

MOV DI, BX	end header, start body	machine. The test loop

HERE		begin loop

INC DI			\ point to next byte		TESTO

MOV BL, 0	DI]	\ get byte	0 DO	PAD UCASE LOOP

MOV AX, # 96	test case	TEST1

SUB AL, BL		0 DO	10000 TESTO LOOP

CBW

XCHG AX, BX		allows us to iterate enough times to get

SUB AL, # 123		meaningful data: saying 10 TEST1 it

CBW		erates 105 times. The time to convert

AND AHf BH	AH = 171710	45 characters is 7 seconds, giving a

AND AH, # 32	\AH = 32 if lcase, 0 elSE per-character time of 1.5 ~tsec, in rea-

SUB 0 1 DII, AH	\ convert letter in $ 	sonable agreement with the estimate

LOOP			loop if CX > 0	from machine cycles. This is 24 times

end body	faster than the high-level Forth version;

footer	optimization is definitely worthwhile

POP DI		\ restore DI	when we have many strings to convert,

NEXT	end footer		For completeness, here is a version

END-CODE				that works with (zero-terminated)

10								Forth Dimensions XXI.1,2

strings in C. There are two obvious ways to approach the prob-	zero-terminated string, the revised upper-case function is vir

lem: first, modify the loop in UCAS E so it terminates when the	tually identical to its predecessor [Listing Twelve].

byte fetched is 0 (not ASCII 0). Alternatively, if we had a fast		To test these, we need words that input and print C-like

way to determine the string's length, we could use the preced-	strings [Listing Thirteen].

ing code unmodified. Now, we know only the beginning ad-		A quick interactive test [is shown in Listing Fourteen].

dress of a C string, so to determine its length we must search it		Of course, if we had to replicate the steps and tests of

until we find the terminating character, incrementing a counter	GET - LEN in assembler, it would obviously be better to rewrite

as we go. In high-level Forth, the subroutine is [Listing Eleven]	UCASE. C entirely, Fortunately, the 80x86 chips have a spe

and is very slow. Given a function to compute the length of a	cial instruction pair, SCASB and REPNZ, that speed up certain

Listing Ten

CODE UCASE

0 (assembler directive)

POP CX		5

POP BX		5

PUSH DI		3

MOV DI, BX		2

total		15 for header

HERE	0 (assembler directive)

INC DI		2

MOV BL, 0 [ DII		5

MOV AXf # 96		2

SUB AL, BL		2

CBW		2

XCHG AX, BX		3

SUB AL, # 123		3

CBW		2

AND AH, BH		2

AND AH, # 32		3

SUB 0 [DI], AH		7

LOOP		9

total	= 42 for body

POP DI		5

NEXT		5 (depends on the Forth)

total	= 10 for footer

END-CODE	0 (assembler directive)

Listing Eleven

GET_LEN		beg -- len)

DUP		beg	beg)

BEGIN		\ start indefinite loop

DUP C@	\ get	char

0<>	( beg	adr flag)

WHILE	1+	( beg adr+l)

REPEAT		(beg end+l)

loop until character is 0

SWAP				len)	compute length

Listing Twelve

UCASE.C	beg

DUP	GET_LEN

0 DO			work left to right thru string

DUP C@		(	--- adr char)	\ get char

DUP	lease?	adr char flag)

32 AND		adr char	32 if lease 1 0 else)

subtract 32 from lease letters only

OVER C!	replace modified character

1+	LOOP	increment address by 1 and loop

DROP ;	\ clean up stack

Forth Dimensions M.1,2

1

string operations. The Forth assembler definition using these	zero-terminated strings (and, for all 1 know, one may be avail

instructions would then be [Figure Fifteen],	able), there does not seem to be any reason to factor out this

This is a bit long and complicated, and no doubt will get	functionality, merely to re-use the code designed for counted

longer and more complex when the boilerplate headers and	strings. Here is a situation where recoding UCASE. C from

footers that respect C conventions are added. Unless there is	scratch is the more efficient approach.

a specific need for a function that determines the lengths of		Once again we begin by prototyping in high-level Forth,

Listing Thirteen

$011		( -- adr)		input 0-terminated string

[CHAR]	WORD	get input

DUP		$adr $adr)

1+	PAD		ROT	C@	get length

DUP >R			save it temporarily

CMOVE		\	move text to scratchpad

PAD R>	OVER +	( -- beg end+l)

0 SWAP	C!	\	terminate with 0

$0.		adr			print 0-terminated string

DUP	GET LEN 	TYPE

Listing Fourteen

$T' Here is a test string of 37 characters!" ok GET LEN . 37 ok PAD $0. Here is a test string of 37 characters! ok

Listing Fifteen

HEX

CODE	GET_LEN	adr	len) \	get length of 0-terminated $

POP BX			adr -> BX

PUSH DI		save state

PUSH ES		save "extra" segment descriptor

MOV AX, DS

\ there is no MOV ES, DS instruction

MOV ES,	AX		point to data segment

MOV CX, # FFFF		largest possible string

MOV DI,	BX		load offset

XOR AL,	AL		AL = 0

REPNZ		SCASB		go thru $ until 0 byte found

SUB BX,	DI		compute length

NEG BX

POP ES					restore state

POP DI

PUSH BX			result on stack

NEXT END-CODE

DECIMAL

Listing Sixteen

UCASE.C			beg

BEGIN						start indefinite loop

DUP C@				( -- adr char)

DUP 0<>					-- adr char flag)

WHILE							haven't reached end

lease?						adr flag)

~32	AND		adr	-32 1 0)

OVER		+C!	modify char in place

1+	-- adr+l)

REPEAT	loop until char = 0

DROP		clean up stack12				Forth Dimensions XX1.1,2