Power PC as a Forth EngineC. H. Ting1.	IntroductionPowerPC architecture was developed in IBM for the mainframe computers and the personal computers.  It was adopted by Apple to run in the newer Macintosh computers, and was also manufactured by Motorola.  In the brief time I worked at the UMAX Computer Corp, I was given the opportunity to study the OpenFirmware system which boots the Macintosh OS system, and also evaluate the Forth code development environment, which compiles the OpenFirmware and generates a target image for the boot ROM.  I found that PowerPC is an interesting chip and serves very well as a Forth engine.Apple chose to use Wil Baden's ThisForth as the Forth code develop environment, mainly because ThisForth is a public domain Forth system and also because it was written in C and was easily ported over to the Macintosh-PowerPC system.  However, most of the code in ThisForth was produced by the PDM4 macro language, which is very cryptic, and nobody knows how to maintain it.  UMAX was quite ambitious in deciding to build its own Forth environment to replace ThisForth.I took the 32-bit 386 eForth and ported it to Macintosh Programmer Workshop, which is the standard software development system provided by Apple.  It has C and C++ compilers, PowerPC assembler, debugger, and all the tools for software development.  I first brought up the  elementary eForth system with the small machine dependent kernel, as Version 1.01.  Later, I enhanced the system by replacing all the tool words by PowerPC code, and added all the utilities ThisForth provided.  The latest version was 2.07, just before UMAX laid off all the Mac programmers, thanks to Mr. Steve Jobs' decision to close down all the Mac clone shops.PowerPC eForth version 2.07 is quite powerful and very fast.  It includes all the functionality of ThisForth and many of the features required to compile PowerPC OpenFirmware.  However, the closing of the UMAX firmware group was so sudden that I didn't have time to really exercise it in compiling OpenFirmware.  I had test-compiled most of the OpenFirmeare system, but didn't try to part which produces the ROM image.  Maybe a couple of weeks would be sufficient to complete the task.In this paper I will discuss the implementation of eForth on PowerPC chip and show that PowerPC is a very nice host for Forth, in spite of the fact that it is not a stack machine.2.	Forth Virtual Engine on Power PCThe register assignments are:Name		PPC Register		FunctionrTOS		r20			Top of data stackrNOS		r21			Next of data stackrDP		r31			Data stack pointerrLR		r18			Shadow of link registerrTOR		r19			Top of return stackrRP		r30			Return stack pointer rBase		r25			Base address for relocationrUP		r29			User area pointert1:		r13			scratch pad registerst2:		r14			scratch pad registersBeing a RISC machine, PowerPC does not have CALL and RET instructions.  Instead, the Branch-Link (BL) instruction saves the address of the next instruction in the Link Register (LR), and the Branch-Thru-Link (BLR) instruction returns to this location after the subroutine is completed.  The address field in the BL instruction is 26 bits wide, and is quite adequate to reach the entire dictionary space in the relative addressing mode.It we consider the BL instruction as a CALL with a one level return stack, we can use the Subroutine-Threaded-Model to implement Forth on PowerPC.  Thus in the list of a colon definition, all the token in the list are BL  instructions of Power PC microprocessor.  NEXT is thus a single machine instruction BLR.  In this sense, PowerPC is a Forth engine.The LR register in PPC branch processor is thus the IP register of the eForth virtual machine.One important advantage of the Subroutine-Thread-Model is that the tokens in the colon definitions are machine instructions, and consequently one can mix other machine instructions in-line with the tokens.  This feature allows Forth words to be easily optimized by inserting machine code directly into colon definitions.3.	Stack CacheTo implement the data stack and the return stack, we need at least two CPU registers as stack pointers.  Here we use r31 is the stack pointer and r30 as the return stack pointer.  As PowerPC has 32 registers in the Integer ALU unit, much more than what is required in a minimal Forth virtual machine, we have the freedom of using other registers to enhance the performance of PowerPC as a Forth engine.Both the top of the data stack (rTOS) and the top of the return stack (rTOR) are cached in CPU registers: r20 and r19.  r21 is used to hold temporarily the contents of the next item on the data stack as rNOS.  Having the top stack items cached in CPU registers optimizes the performance of the Forth Virtual Engine and reduces the stack pushing and popping of stacks in run-time.To access stack items, PowerPC can post-increment or post-decrement the stack pointer register in a single memory accessing instruction.  However, it cannot pre-increment of pre-decrement.  It is thus only half of a good stack machine.  I choose to post-decrement the stack pointer on pushing.  On popping, the stack pointer has to be incremented before doing memory read or write.4.	Memory AddressingThe Forth Engine accesses memory using addresses relative to the origin of the eForth image in memory.  The absolute address of the origin is stored in rBase register (r25).  Therefore, this eForth is totally relocatable, because all the tokens in the list of colon words are relative branch-link instructions with 26-bit address offsets in the address fields.Upon entering into a colon words, the address pushed on the return stack is an absolute address.  Addresses of data visible to the eForth system are relative addresses from the origin of eForth as placed in the memory.  The absolute address of the origin is stored in register rBase.  When coding in assembly, you have to be aware of the two distinct addressing systems.  Normal Forth operations assumes the relative addresses.  Only when you pop address back from the return stack, you have to convert the absolute address to relative address before you can you @ or ! to access the memory.Relative address has the advantage of relocation.  eForth thus can be loaded into any memory region and the code will run without any modification.5.	OS InterfaceeForth is entered via a small C program, which posts a sign-on message and then calls eForth as a subroutine.  This C calling program is very useful in developing the Forth words which must request services from the operating system.  These Forth functions were first written in C, tested, and then the functions were recoded in Forth.The only input/output functions eForth need are the terminal input ?RX and terminal output TX!.  eForth only assumes one serial port for user interaction.  KEY and EMIT are vectored through ?RX and TX! to get characters from the user and display characters on the appropriate display device.  In PPC eForth, ?RX calls 'read' to get characters from the Mac OS, and TX! calls 'write' to send characters to the Mac OS.  This scheme works fine under MPW. The C program CALLING.C is compile by the C compiler MRC and then linked with the eForth program for execution.  CAALING.C is my playground to test many different OS interface like files, time/date,memory allocation, etc.6.	File InterfaceThe most important file related function in eForth is the loading of source files.  It is desirable to be able to load files from other files, recursively, and without limits on the number of files to be loaded and the levels of file loading.  The operating system generally allows an application to open many files, and each file is associated with a file handle.  I found that with a very simple mechanism, one can achieved these goals.  This mechanism was implemented in the MIPS eForth. and was ported over to PowerPC eForth.The mechanism is to make the terminal input buffer as large as possible.  The INCLUDE commands opens a file, loads the source at the end of the text in the terminal buffer, and then closes the file immediately.  The source text of the file is interpreted.  When the entire text of the file is processed, the interpreter continues to process the text received from the terminal.If we encounter another INCLUDE command in the file, the new file is opened and loaded at the end of the old file.  The new file is closed and its text is interpreted.  After the new file is finished, the rest of the old file is interpreted from the point after the INCLUDE command.  Files can thus be loaded recursively, as long as the terminal input buffer is large enough to hold all the files for interpreting.  If we structure the loading process by loading all the files from one file, the terminal input buffer needs only to be big enough to hold the loading file and the largest of the loaded files.7.	DebuggingMacintosh has a powerful debugging tool MacsBug.  It is invoked whenever an illegal instruction is executed or when some system errors occur.  It is smart enough to know when to enter the PowerPC mod or the 68000 mode.  In the PowerPC mode, you can see the contends of all the CPU registers, and can inspect memory, change memory, disassemble PowerPC machine instructions, setting break points, and single stepping through code anywhere in the memory.  It was my best friend when I brought the eForth system up.PPC eForth provides several entry points to the code image for the convenience of debugging the system when there is trouble.  The entry point DEBUG has an invalid PPC instruction, which will trap to Macsbug.  Set PC to 4 bytes passing DEBUG will start eForth under Macsbug.  There you can use the full suite of Macbug commands to debug eForth and its applications.The command:	0  EXECUTEwill invoke the Macsbug and then you can debug the

application using Macsbug.8.	Local VariablesLocal variables are conveniences for people who hate stacks.  OpenFirmware uses local variables extensively, and I have to implement them according to the OpenFirmware style.  The syntax of local variables is:	:  <name>  {  ...  stack3 stack2 stack1 ; local1 local2 local3 ... }		<ForthWordList>		;Inside the curly brackets, items before semi-colon are input parameters taken from the data stack; and items after the semi-colon are names of local variable.  Stack items and local items can be referred in <ForthWordList> by names to get their values on to the data stack.  The TO command is used to store new values into the stack or local items.  Input stack items are removed from the data stack and stored into local variable.  Items left on the data stack are the stack output from this word.  In a word, up to 10 local variables are allowed.  Local variables are expensive, no matter how they are implemented.  In this implementation, a special register is designated as the local variable frame pointer, and a fairly large memory area must be allocated for the local variable frames.  Local variable frames are stacked up during execution.  Each word with local variables must push a local variable frame, move the stack input items to the frame, read values from the frame, store new values into the frame, and finally must pop the frame.Only after implementing the local variables, one can appreciate the simplicity of Forth in passing all parameters through a single stack.9.	ANS Forth StandardI tried to build this PowerPC eForth aligned to ANS Forth Standard.  However, the primary objective was to replace ThisForth and standard compliance was not pursued with vengeance.  All words with standard names have standard behavior, but not all the standard and extension words were implemented.10.	Executable StructuresOpenFirmware has an interesting feature that most control and loop structures can be interpreted as well as compiled in colon words.  The structure building words like IF, ELSE, THEN, BEGIN, UNTIL, DO, and LOOP are smart so that when they are invoked while interpreting, they build the necessary structures in a small buffer.  These structures are executed and then forgotten.I was very interested in the interpreting of structures, to the point that I tried to discard the interpreter completely.  In version 2.05, everything is compiled.  After a line of commands is accepted, the commands are compiled into a command buffer as a new colon word, only without an header.  A DOCOL starts the buffer and an EXIT is attached to the end of the buffer.  This headerless word is executed and the buffer is used to compile the next line of commands from the keyboard.Since all commands are compiled, all the structures are compiled and not special attention is necessary for interpreting structures.This scheme works well in most cases.  The only problem is CREATE.  Here all the defining words must be IMMEDIATE, because they have to build new words and cannot be themselves compiled into colon words.  The problem with CREATE is that when we have to define a defining word, CREATE must be compiled into the definition of the defining word.  Its immediate behavior has to be POSTPONEd to the time when the defining word is executed.Defining words were used very extensively in OpenFirmware, and it was inappropriate for me the change all the source code to postpone CREATE.  Eventually, I abandoned the compiler-only approach and dutifully implemented the smart control words so that we can interpret the control structures.11.	BenchmarkI tested the eForth against the ThisForth.  Timing for the two systems on the following testing routines are:Test Routines							 ThisForth (sec)      eForth 	: LOOPTEST 10000000 0 DO LOOP ;				4		1: +TEST    10000000 0 DO R@ DUP + DROP LOOP ;		11		2: *TEST    10000000 0 DO R@ DUP * DROP LOOP ;		11		2: /TEST    10000000 0 DO 10000000 R@ / DROP LOOP ;		12		3eForth is about 4-5 times faster than ThisForth.  this is not surprising, because ThisForth was implemented in C, and eForth is fully optimized in PowerPC assembler.  From these tests, I can conclude that eForth is doing about 40 million primitive Forth instructions per seconds, on an 150 MHz 604e PowerPC microprocessor, a very respectful performance for a Forth engine.12. 	ConclusionPowerPC is a very good microprocessor, in spite of its RISC ancestry.  The Branch-Link mechanism in the RISC architecture is very useful in the Forth Virtual Machine because it is exactly what is needed to implement the Forth inner interpreter NEXT, if we use the Branch-Link instructions as Forth tokens.  The post-incrementing and post-decrementing features in memory accessing instructions give us a half-decent stack mechanism, and under the circumstances, we really do not have too much to complain.  An efficient Forth implementation can bring out very good performance from this microprocessor.  At 40 million Forth instructions per second, this PowerPC eForth is indeed the fastest Forth I have ever seen.