Firmware Engineering Workshop

- H. Ting

Contents

Preface										2

1.	Firmware Engineering								3

1.1	Terminology								4

1.2	Firmware vs software							5

1.3	Black box vs white box approaches						5

1.4	eForth for firmware engineering						6

1.5	The most commonly used Forth Commands					8

1.6	Wash machine, a sample of Forth program					10

1.7	Exercises and Examples 							11

2.	Embedded Forth System								24

2.1	Forth Commands								24

2.2	Forth Virtual Machine							25

2.3	Memory map								26

2.4	Primitive commands							26

2.5	System variables								35

2.6	Extension compound commands						36

2.7	Text interpreter								40

2.8	List compiler								51

2.9	System utilities								55

2.10	Startup									57

2.11	Conclusion								60

3.	Application Programming in Forth							63

3.1	eForth as Programming Environment					63

3.2	The CT100 Laboratory Board						64

3.3	Programming examples for CT100 Lab Board				70

3.4	8051 Assembler								78

4.	Firmware Production								80

4.1	Software Production with 8086 eForth					80

4.2	Firmware Production with 8051 eForth					85

4.3	Programming style and documentation					88

Concluding Remarks								91

Appendix	eForth Quick Reference Card					92

Preface

I have thought of writing a small book to introduce Forth to engineers and scientists in fields other than computer engineering.  I like to present Forth as a programming language and an operating system in less than 100 pages, complete with source code.  A small book will attract many people to learn this very interesting system in a friendly and non-threatening fashion.

This summer I participated in the Third Semiconductor Equipment Workshop hosted by the Chiao Tung University in Hsin-Chu, and much discussion was on the software necessary to run instruments and equipment in the semiconductor industry.  I proposed to run a firmware workshop to train manufacturing and equipment engineers to program the equipment themselves, not throwing the problems to the software engineers who still have to learn the equipment in order to program them correctly.

Dr. Tak-Goa Tsuei, Vice President of Ta Hwa Institute of Technology and also a good friend for many years, took upon himself to arrange this Workshop of Firmware Engineering and lined up many cosponsors, including Chung Yuan Christian University, Tung Hwa University, Tai San Technical Training Center, and the Manufacturer Association in the Science-based Industrial Park.  He was also able to secure support from the National Science Council.  Thanks to his hard work and persistent driving, the workshop will become a reality.

My original proposal was to have a one week workshop.  I thought I need a week's time to fully convince the audience that Forth is the right tool for many applications, particularly those using embedded microcontrollers.  It will take that much time to get people to use Forth, write fairly large programs, and debug them.  Only by going through the entire program-test-debug cycle, people can be confident in the newly learnt tools and techniques and start to use them in their jobs to solve real-world problems.  The hands-on experience is very important in getting acquainted with a new technology.  However, Dr. Tsuei convinced me that it will be very difficult to run a five day workshop intended for professional engineers holding jobs in companies.  You simply cannot break them away from their jobs for that long, because their managers cannot approve such extended leave.

The best compromise is that we run short workshops at locations close to the target audience, and that I prepare extensive lecture notes for them to study in their spare time.  This manual is the result of my preparation.  I collected from my writings the materials pertinent to firmware engineering and organized them into 4 sessions.  Each session can be presented in half a day or in a full day, depending on the amount of demonstrations and hands-on experiments.  The subjects covered in this manual can be expanded into a one-semester course if they are presented fully at a more reasonable pace.

Twenty years ago when I was first exposed to Forth, I was completely convinced that this was the ultimate software tool that allows us human beings to realize the full potential of computers.  I still have not seen anything coming even close to its expressiveness and power.  My aspiration is to see that the young engineers and scientists in Taiwan make good use of this tool to bring Taiwan into the next century, not as followers to their American and Japanese counterparts, but as new leaders to harness the power of computers in advancing the arts of instrumentation, production and manufacturing.  We have the tool.  We only need the will to use it.

C. H. Ting

December, 1998

Session 1.  Firmware engineering

Microprocessors and microcontrollers are so cheap now a days that there is no reason not to use them in an instrument or an equipment with even very modest complexity.

The use of microcontrollers bring these benefits:

Enhance the performance of the equipment

Provide more flexible user interface

Allow the same hardware to be used in different applications

Trade software against hardware

Correct or bypass hardware deficiency

Relegating hardware to software has advantages, but it also bring in problems.  When a engineering product employs a microcontroller, software engineer must be added to the staff.  To program, test and debug the software, software engineers need expensive computers to run the latest compiler, simulator, in-circuit-emulator, logic analyzer, and other expensive test equipment.  Generally, the software becomes the bottle neck of the project.  Software is always late and causes budget over-runs.  Bugs creep up even after the product is delivered to the customers.  We have heard horror stories all the time about buggy and non-performing software.

Software engineers do not have the proper tools to deal with the hardware problems.  Given the proper tools, hardware problems can be best solved by hardware engineers.  I call the use of these proper tools Firmware Engineering.  Software engineering is an immature technology and it is dangerous to rely upon it to solve hardware problems in the embedded systems.  To program the embedded system, the programmer has to be thoroughly familiar with the hardware to do a good job.  It is much more difficult to transfer the hardware knowledge to a software engineer so that he can write the program correctly.  It is much easier to teach the hardware engineer the proper programming tools so that he can write the program himself.  I believe that a hardware engineer can be trained to become a firmware engineer in a matter of days, and this hypothesis is what I want to prove in this Firmware Engineering Workshop.

For embedded systems, programs were usually written in assembly which was difficult and time consuming when the object program size is greater than a few thousand bytes. In recent years, the PC's are getting bigger and faster, and there are C compilers available on the PC platform for all microcontrollers.  More and more microcontroller applications are programmed in C.  Even using a high level language like C, programming microcontrollers for embedded applications is still difficult and challenging.  The reason is that for embedded applications, programming language is only a small part of the problem.  The more difficult part is that embedded system must be stand-alone, and programming has to be approached from a systems perspective.  We need better programming tools and program testing/debugging tools to deal with the problems encountered in developing software for embedded systems.

Software introduces complexity, which is orders of magnitude higher than hardware.  Controlling software costs, improving software quality, and raising software productivity are the most important problems facing the software industry.  Engineers building hardware are very conscientious of the problems associated with complexity and are trained to look for every possible way to design and build the simplest possible product for specified functionality.  On the other hand, software engineers are accustomed to and thriving on complexity.  They rely on software tools to deal with complexity and to optimize the code.  However, complex tools generate complex code.  One of the characteristics of high level languages is code expansion; that is, the object code produced by the compiler is much bigger than that of the source code.  In software for PC and mainframe computers, program size is becoming less and less of an issue, because the computers are getting faster and the memory is getting bigger.  In the embedded systems, however, object program size is still a major concern.  High level languages are not the complete solution.

Object Oriented Programming (OOP) has become a fad in the software industry, and it has shown some effect on the programming of embedded systems.  The principles of OOP are correct, in that complexity can be hidden in objects, objects can be reused to reduce program size, and messages are used to invoke methods. However, tools in OOP are still primitive and the program size becomes even larger.  Forcing OOP into embedded systems is even more difficult.

We need better tools to program microcontrollers for embedded applications.

Forth is a very unique programming language and operating system, best suited for the embedded systems.  It is small so that it can fit into a small memory area.  It is fast so that it can respond quickly to real time stimuli.  It is a high level language so that the programmer can design and program at highly abstract levels.  It also has direct access to the CPU at the assembly code level so that programs can be optimized for high performance.  It has been used in many different fields, like aerospace, military, biomedical, and industrial applications.

In this Workshop, I like to present a complete Embedded Forth system tailored for embedded applications.  I like to show you how to use this Forth system to program applications, how to integrate the application to the Forth system, and how to produce firmware to run on the  embedded system.  The time of the Workshop does not allow me to explain every thing about this Forth system.  However, I will give you the complete source code of this system implemented on 8086 and 8051.  There are similar Forth implementation for all the more popular microprocessors and microcontrollers in my code library.  You can take the code back and run it on your system to explore its capability and to learn how to adapt it to your application.  Once you can produce a firmware to run on your system, You can claim for yourself the title of a Firmware Engineer.

1.1	Terminology

Let's first define some of the terminology, to facilitate the subsequent discussions.

Computer is an electromechanical system which executes programs and perform tasks designed in the programs

Microprocessor is an integrated circuit chip which has a CPU (Central Processing Unit) executing externally provided programs.  It also drives external I/O devices to perform specific tasks.  Microprocessors are increasingly used to run computers.

Microcontroller is an integrated circuit chip which has a CPU, memory to supply executable program, and I/O devices necessary for specific tasks.  Microcontrollers are used in stand-alone instrument/equipment to control the operation of the equipment.

Hardware is the electromechanical components in the computer which make it possible to execute programs.

Embedded system is an instrument or an equipment which uses a microprocessor or microcontroller to control the hardware.

Software is the executable program provided to computers, including microprocessor and microcontrollers, to drive the hardware and perform tasks designed into the program.  Software can be supplied in various forms and on various media, including hard disks, floppy disks, tapes, etc.

Firmware is the program executed by a microprocessor or a microcontroller to perform tasks by the embedded system.  The program usually is stored in non-volatile memory devices like ROM, EPROM, flash memory, or internal memory inside the microcontroller.  The non-volatility of the stored program gives firmware its unique characteristics, significantly different from software.

Firmware engineering is the discipline to design, implement, test, debug, and maintain programs written into non-volatile memory to control the functions of an embedded system.

1.2	Firmware vs Software

Firmware is essentially software.  They are programs to be run in a computer to perform specific tasks.  The programs are written in some form of source code, which is processed by a set of programs like compilers, assemblers and linkers and converted to a set of object code.  The object code is loaded into the computer and executed.

Firmware is different from software in the following respects:

Firmware must be self-contained and stand-alone.

Firmware has small memory footprint.

Firmware generally has tight timing constraints for real-time applications.

Firmware is generally stored in non-volatile memory.  Because the memory chips are more expensive than DRAM chips and magnetic media, and they occupy premiere space on the mother board in a computer, the size of the memory is limited and the firmware program must be kept small so to fit into the memory chips on board.  The size of software is limited only by the DRAM memory and by the disk storage.  Software products are much more liberal in the use of memory, and their sizes tend to fill the available memory space.

The size limitation forces firmware to be engineered more carefully and to achieve much higher quality standard.

Because firmware must be stand-alone, it often has to include functions normally provided by the operating system in a conventional computer system.  It has to manage the hardware resources in the system, and also provide interface to the user.  All the functions must be committed to the non-volatile memory, which is limited in the size and difficult to modify in the field.

Firmware engineering is much more difficult than software engineering, because it cannot rely on services an operating system provides to software.  However, conventional operating systems are too big and too complicated to be included in firmware.  Consequently, all these services have to be provided by the firmware itself.

It is generally considered impractical to include an operating system in the firmware, because conventional operating systems are bulky and they often are too slow to respond to real time events the firmware has to deal with.  However, if we can build a small and efficient operating system as a standard component of the firmware, applications can be developed quickly and economically.  Such operating system does exist, and we will explore how to use such a system for firmware engineering.

1.3	Black Box vs White Box Approaches

To reduce software costs and to raise the software quality, the industry is moving towards high level languages, and more recently to the object oriented programming.

The necessity to move to high level languages and to modularity make object oriented programming a hot topic in software.  The trend also has certain effects on firmware.  Complexity can be hidden in objects as black boxes.  The programmer does not have to know what is inside the black box.  The black box approach has many problems.  If you do not know what is inside the black box, you cannot fully control it and maintain it  when problems arise.  You also do not have control on the program size and other critical properties like timing.

The black box approach has been quite effective in software development.  As the computers becoming faster every year, and the DRAM memory size doubled every year, inefficiency in the object oriented programs is tolerable.  However, though the size of the non-volatile memory in computers is also increasing, the rate of increase is far lower than the DRAM and disk memory.  Firmware still demand memory efficiency and also fast response to real time events.

It is necessary to build firmware with the white box approach.  Functions are built in modules which are then used to build more powerful modules.  However, the modules must be  in transparent white boxes, and you can have complete control over their functionality and their interfaces.

The most successful example of the white box approach is the Open Firmware which boots up all the Sun workstations and all the PowerPC based Macintosh computers.  Based on Forth, the Open Firmware bring to life computers with different CPU's, bus architectures, and a host of different peripheral devices.  For small applications using microcontrollers, Open Firmware is still too big and too complicated.  Nevertheless, a simpler Forth system can be adapted to embedded applications.

We need a Forth specially designed for small embedded system which is simple, easy to learn, easy to develop applications, and easy to committed to non-volatile memory.  This is eForth, the Embedded Forth system I developed over the years.  It is simple and easy to port to different CPU's.  eForth has to be optimized for the CPU used in a target.   It is a complete operating system with a high level programming language and tools to develop large applications for all kinds of microcontrollers.

This eForth system is the focal point of this Firmware Engineering Workshop.  I hope that in the course of this workshop, a professional engineer can learn it and later use it to produce firmware for his application.  No prior computer knowledge is required.  However, working knowledge of microcontroller will be beneficial, if the engineer has an open mind on programming languages and on operating system.  Forth is very different from the conventional programming languages and operating systems, and it can only be embraced and appreciated by an open mind.

1.4	eForth for Firmware Engineering

Forth is a programming language with its own operating system.  It can be implemented on any reasonably capable CPU and computer.  It has very simple syntax and rules of usage, and can be mastered in very short period of time.  Using Forth, one can easily write and debug substantial programs with high quality and in short time.  With a few simple extensions, the Forth system can be used as the operating system of an embedded system to produce firmware for a wide range of engineering products.

A Forth system is a collection of commands residing in memory.  These commands can be executed interactively, and they are also used to build new commands interactively.  Building new commands to replace lists of existing commands is a process of abstraction, aligning the computer closer and closer to the actual application.  In the end, the commands at the highest level become the application itself.  Forth commands have many of the nice attributes of objects in object oriented programming, but without the complications and bulkiness.  It is ideally suited for applications in small microcontrollers.  It is the best tool for firmware engineering.

Advantages of using Forth in firmware engineering are:

High level programming language

Platform independence

Tight object code

Interactive testing and debugging

Ideal programming environment

The hardware requirements to run the eForth efficiently are a CPU with a reasonably good architecture, 16K bytes of ROM/RAM memory, and a serial UART port.  Now a days, these are very modest requirements for most microcontroller based applications.

Most embedded systems do not need a serial port when running stand-alone.  However, a serial port is very handy when servicing the system, and extremely useful when developing and debugging a product.  The serial port gives the programmer an easy channel into the target system.  He can interactively access all the hardware resources in the target system.  It makes interactive testing and debugging possible, and greatly eases the program development process.  It also allows the system to communicate with a host computer on the automated factory floor.

The eForth is designed for an idealized microcontroller system configured as shown in Figure 1.  It has a CPU with some memory, perhaps with some custom peripherals for specific applications.  It is connected to the outside world through a RS232 port.  This RS232 port may not be used in the actual applications, but it can be connected to a host computer for programming, testing and debugging.  In certain applications, the RS232 umbilical cord may never be severed because then the host can download new code to the target and send commands to the target to perform specific tasks.  Many of the new generation of instruments and equipment are designed to work either stand-alone and with a host computer.

There are many trade-offs that we have to consider in designing this eForth, which we assume will be ported to many different CPU's.  Here is a list of important features we strife to include in the eForth:

.	A small set of primitive commands, which is machine dependent.  Only this set of commands have to be rewritten for a specific CPU.  A minimum primitive commands set encourages the porting of eForth to new CPU's.

.	All high level commands are built on the primitive commands.  The high level commands can be easily ported without changes to all target CPU's, including 8, 16, and 32 bit microprocessors and microcontrollers.

.	The only I/O commands are KEY, EMIT, and ?KEY, because the only required I/O device in the target is a RS232 port.

.	Source code can be downloaded from the host computer and compiled by the target.

.	Core image of the system including application can be uploaded to host for programming EPROM to run the target system.

.	Editor, file management, and software version control are provided by the host system.  We assume that the HyperTerminal in Windows will be the host interface to the target system.

Firmware engineering will be very easy if we are dealing with microcontrollers and embedded system which meet the above requirements and have a Forth operating system to support the application.  It is then possible to develop substantial applications interactively on the target system.  Programs can be written in high level languages, and downloaded to the target system for incremental compilation and testing.  Program modules can be tested exhaustively before using them to build the modules in the next higher level.  Products can be debugged easily and problems can be fixed quickly.

In this Firmware Engineering Workshop, I will discuss in minute details of this eForth system, how to use it to write application programs, and how to integrate the application program into the Forth system to produce firmware which will run he target embedded system.

1.5	The most commonly used Forth Commands

Here is a list of 54 Forth commands most frequently used in Forth programs.  They are listed in alphabetic order so you can find them quickly.  They are used in the exercises and the examples in the following two sections.  A more comprehensive list of eForth commands are in the Appendix.

Stack Comments:

Stack inputs and outputs are shown in the form:  ( input1 input2 ... -- output1 output2 ... )

Unless noted otherwise, all numbers are 16 bits.

flag 					Boolean flag, either 0 or -1

?					Truth value, either 0 or non-zero

char					ASCII character

b 					Any 8 bit number

x					Any 16 bit number

n					Signed 16 bit number

u					Unsigned 16 bit number

w					Wrap-around/circular 16 bit number

+n 					Positive 16 bit number

addr 					Address (same as u, unsigned)

d					Signed double (32 bit) number

xd, ud, wd, +d				Specify types of 32 bit number

!      		(x addr -- )          		Store x at addr.

( comment) 	( -- )            		Ignore comment text.

*      		(w1 w2 -- w3)         	Multiply. "times"

*/     		(n1 n2 n3 -- n4)      	Leave quotient of (n1*n2)/n3.

*/MOD		(n1 n2 n3 – rem quot)	Leave remainder and quotient of (n1*n2)/n3

+      		(w1 w2 -- w3) 		Add w1 and w2.

+!     		(w addr -- )          		Add w to number at addr.

-      		(w1 w2 -- w3) 		Subtract w2 from w1 (w1-w2=w3).

.      		(n -- )               		Display signed number with trailing blank.

." <text>" 	( -- )              		Compile <text> message. At run-time display text message till “.

.R		(n1 n2 -- )		Display n1 right justified in n2 columns

.S		( -- )			Display the contents of the data stack.

/      		(n1 n2 -- quot)       	Division, signed (quotient of n1/n2).

2DROP		(n n -- )			Discard two numbers.

2DUP		(n1 n2 – n1 n2 n1 n2)	Duplicate two numbers.

: <name> 	( -- )              		Begin a new command of <name>.

;      		( -- )                		Terminate a new  command.

<      		(n1 n2 -- ?) 		True if n1 less than n2.

=     		 (n1 n2 -- ?) 	      	True if n1 equals n2.

@      		(addr -- x)           		Replace addr by number at addr.

BASE   		( -- addr ) 		Contain radix for input-output conversion.

BEGIN		( -- )			Start an indefinite loop.

BL		( -- 32)			Put 32 on top of stack.

CELLS		(n – 2n)			Multiple n by 2.

CR     		( -- )                		Display a new line.

DECIMAL 	( -- )               		Set number base to decimal.

DROP   		(x -- )			Discard top of stack.

DUP    		(x1 -- x2)		Duplicate top of stack.

ELSE		( -- )       		Terminate <true> clause, continue after the THEN.

EMIT   		( char -- )           		Display char.

EXIT   		( -- )                		Terminate execution of a colon command.

FOR 		( limit -- )  		Setup loop. Repeat loop until limit is decremented to 0.

HEX		( -- )			Set number base to 16.

IF		( ? -- )     		If ? is zero, branches forward to <false> or after THEN.

KEY    		( -- char )           		Get an ASCII character from the keyboard.  Does not echo.

MOD    		(n1 n2 -- mod)        	Modulus, signed (remainder of n1/n2).

NEGATE 	(n1 -- n2)            		Two's complement.

NEXT		( -- )     			Decrement index and repeat loop until index is less than 0

NUMBER?	(addr – n -1 | addr 0 )	Convert string at addr to a number n and a -1.

Return addr and 0 if failed.

OR     		(x1 x2 -- x3)         	Logical bit-wise OR.

OVER   		(x1 x2 -- x1 x2 x1) 	Make copy of second item on stack.

QUERY		( -- )			Wait for one line of text from the terminal.

R@      		( -- w )              		Used inside a loop to get the current index value.

ROT		(x1 x2 x3 -- x2 x3 x1)	Rotate third item to top. "rote"

SPACE  	( -- )                		Display a space.

SPACES 	( +n -- )             		Display +n spaces.

SWAP 		(x1 x2 -- x2 x1)		Exchange top two stack items.

THEN		( -- )			Terminate the IF-ELSE structure.

U.R		(u n -- )			Display unsigned number u in n columns.

U<		(u1 u2 – ? )		Return true if u1<u2.

UM*		(u1 u2 – ud)		Multiply unsigned numbers. Return unsigned double product

UNTIL		( ? -- )     		Repeat <loop-body> until the ? flag is non-zero.

VARIABLE <name>	( -- )       	Define a variable.  At run-time, <name> leaves its address.

WORD <text>	(char -- addr )		Get the char delimited string <text>  from the input stream

1.6	Wash Machine, a Sample of Forth Program

Here is a simple example of a program controlling a hypothetical wash machine.  An output port at address $FF01 controls the following actions:

Bit 0		Drive motor clockwise

Bit 1		Drive motor  counter-clockwise

Bit 2		Open water faucet

Bit 4		Open water drain

Bit 5		Open soap input

An input port $FF02 receives sensor information:

Bit 0		Water full

A set of commands to operate the hardware of the wash machine:

$FF01 CONSTANT OUTPUT

$FF02 CONSTANT INPUT

: MOTOR-CW	1 OUTPUT C! ;

: MOTOR-CCW 2 OUTPUT C! ;

: WATER-ON	4 OUTPUT C! ;

: DRAIN	8 OUTPUT C! ;

: SPIN	12 OUTPUT C! ;

: SOAP-ON	16 OUTPUT C! ;

: OFF		0 OUTPUT C! ;

: WATER-FULL? ( -- f )	INPUT C@ ;

Now we can implement a few high level commands:

: MS ( n -- )  	FOR  1000 FOR NEXT NEXT ;

: SECONDS ( n -- )

FOR  1000 MS NEXT ;

: AGITATE	200 FOR

MOTOR-CW 1000 MS OFF 200 MS

MOTOR-CCW 1000 MS OFF 200 MS

NEXT ;

: ADD-WATER 	WATER-ON

BEGIN	WATER-FULL? UNTIL

OFF ;

: ADD-SOAP	SOAP-ON 10 SECONDS OFF ;

: DRAIN-WATER

DRAIN 60 SECONDS

SPIN 60 SECONS

OFF ;

The wash cycle can now be programmed:

: WASH	ADD-WATER

ADD-SOAP

AGITATE

DRAIN

ADD-WATER

AGITATE

DRAIN

;

The application is very simple, but it serves to illustrate the way an application is constructed from the very low level of hardware control, to the implementation of high level commands, and to the final application.

1.7	Exercises and Examples

There are 17 lessons to help you get familiar with Forth language, and show you how to write programs in Forth.  It is very important that you type in a few of these examples and make them work.  You will encounter many problems in entering the examples, like miss-typing characters, missing the spaces, entering wrong numbers, and stack underflows.  However, this is what it takes to learn a new language.  Of the 220 some eForth commands, only 54 commands are used in these examples.  Knowing how these 54 commands work will allow you to write Forth programs to solve most programming problems.

1.7.1	The Universal Greeting

This is the universal greeting, the first programming example in all programming language manuals.  It requires 4 Forth commands and their formal descriptions are:

: <name> 	( -- )              		Begin a new command of <name>.

;      		( -- )                		Terminate a new  command.

CR     		( -- )                		Display a new line.

." <text>" 	( -- )              		Compile <text> message. At run-time display text message till “.

We build a new command HELLO this way:

: HELLO CR ." Hello, world!" ;

Type HELLO with a <return> displays the message ‘Hello, world!.’

1.7.2	The Big F

The command .” can be used to build many other commands which display patterns on the terminal.  Two such commands are needed to display a big F character on the terminal:

: bar   CR ." *****" ;

: post  CR ." *    " ;

: F     bar post bar post post post ;

Type 'F' and a return on your keyboard, and you will see a large F character displayed on the screen

1.7.3	Example 3.      FIG, Forth Interest Group

A few more patterns will allow us to display any big ASCII characters on the terminal.

: center CR ."   *  " ;

: sides  CR ." *   *" ;

: triad1 CR ." * * *" ;

: triad2 CR ." **  *" ;

: triad3 CR ." *  **" ;

: triad4 CR ."  *** " ;

: quart  CR ." ** **" ;

: right  CR ." * ***" ;

: bigT  bar center center center center center center ;

: bigI  center center center center center center center ;

: bigN  sides triad2 triad2 triad1 triad3 triad2 sides ;

: bigG  triad4 sides post right triad1 sides triad4 ;

: FIG  F bigI bigG ;

1.7.4	Repeated Patterns

We will use the .” command to display blocks of asterisks in various forms and shapes.  What we need is the looping mechanism to display a sequence of asterisks.  Here are these Forth commands needed:

FOR 		( limit -- )  		Setup loop. Repeat loop until limit is decremented to 0.

NEXT		( -- )     			Decrement index and repeat loop until index is less than 0

R@      		( -- w )              		Used inside a loop to get the current index value.

VARIABLE <name>	( -- )       	Define a variable.  At run-time, <name> leaves its address.

@      		(addr -- x)           		Replace addr by number at addr.

!      		(x addr -- )          		Store x at addr.

Variables points to locations in memory where data can be stored and retrieved.  @ retrieves data and ! stores data.

VARIABLE WIDTH                  ( number of asterisks to print )

: ASTERISKS ( -- , print n asterisks on the screen, n=width )

WIDTH @                 ( limit=width, initial index=0 )

FOR ." *"               ( print one asterisk at a time )

NEXT                    ( repeat n times )

;

: RECTANGLE ( height width -- , print a rectangle of asterisks )

WIDTH !                 ( initialize width to be printed )

FOR     CR

ASTERISKS       ( print a line of asterisks )

NEXT

;

: PARALLELOGRAM ( height width -- )

WIDTH !

FOR     CR R@ SPACES    ( shift the lines to the right )

ASTERISKS       ( print one line )

NEXT

;

: TRIANGLE ( width -- , print a triangle area with asterisks )

FOR     CR

R@ WIDTH !      ( increase width every line )

ASTERISKS       ( print one line )

NEXT

;

Try the following instructions:

3 10 RECTANGLE

5 18 PARALLELOGRAM

12 TRIANGLE

1.7.5	The Theory That Jack Built

This example shows you how to build a hierarchical structure in Forth.  From simple phrases, we can construct verses of a recursive poem.  Here is a few new commands we need to build the hierarchy.

IF		( ? -- )     		If ? is zero, branches forward to <false> or after THEN.

ELSE		( -- )       		Terminate <true> clause, continue after the THEN.

THEN		( -- )			Terminate the IF-ELSE structure.

KEY    		( -- char )           		Get an ASCII character from the keyboard.  Does not echo.

SPACE  	( -- )                		Display a space.

SPACES 	( +n -- )             		Display +n spaces.

DROP   		(x -- )			Discard top of stack.

: the           ." the " ;

: that          CR ." That " ;

: this          CR ." This is " the ;

: jack          ." Jack Builds" ;

: summary       ." Summary" ;

: flaw          ." Flaw" ;

: mummery       ." Mummery" ;

: k             ." Constant K" ;

: haze          ." Krudite Verbal Haze" ;

: phrase        ." Turn of a Plausible Phrase" ;

: bluff         ." Chaotic Confusion and Bluff" ;

: stuff         ." Cybernatics and Stuff" ;

: theory        ." Theory " jack ;

: button        ." Button to Start the Machine" ;

: child         ." Space Child with Brow Serene" ;

: cybernatics   ." Cybernatics and Stuff" ;

: hiding        CR ." Hiding " the flaw ;

: lay           that ." Lay in " the theory ;

: based         CR ." Based on " the mummery ;

: saved         that ." Saved " the summary ;

: cloak         CR ." Cloaking " k ;

: thick         IF that ELSE CR ." And " THEN

." Thickened " the haze ;

: hung          that ." Hung on " the phrase ;

: cover         IF that ." Covered "

ELSE CR ." To Cover "

THEN bluff ;

: make          CR ." To Make with " the cybernatics ;

: pushed        CR ." Who Pushed " button ;

: without       CR ." Without Confusion, Exposing the Bluff" ;

: rest                                  ( pause for user interaction )

." . "                          ( print a period )

10 SPACES                       ( followed by 10 spaces )

KEY                             ( wait the user to press a key )

DROP CR CR CR ;

: cloaked cloak saved based hiding lay rest ;

: THEORY

CR this theory rest

this flaw lay rest

this mummery hiding lay rest

this summary based hiding lay rest

this k saved based hiding lay rest

this haze cloaked

this bluff hung 1 thick cloaked

this stuff 1 cover hung 0 thick cloaked

this button make 0 cover hung 0 thick cloaked

this child pushed

CR ." That Made with " cybernatics without hung

CR ." And, Shredding " the haze cloak

CR ." Wrecked " the summary based hiding

CR ." And Demolished " the theory rest

;

Type THEORY to start.  Hit any key to continue.

1.7.6	Help

This example show you how to use Forth interpreter to carry on a dialog.  It pretends to be a psychiatrist talking to you as his patient.  You can add more topics for discussion if you like.

New commands appearing here are:

=     		 (n1 n2 -- ?) 	      	True if n1 equals n2.

OR     		(x1 x2 -- x3)         	Logical bit-wise OR.

: question

CR CR ." Any more problems you want to solve?"

CR ." What kind ( sex, job, money, health ) ?"

CR

;

: help  CR

CR ." Hello!  My name is Creating Computer."

CR ." Hi there!"

CR ." Are you enjoying yourself here?"

KEY 32 OR 89 =

CR

IF      CR ." I am glad to hear that."

ELSE    CR ." I am sorry about that."

CR ." maybe we can brighten your visit a bit."

THEN

CR ." Say!"

CR ." I can solved all kinds of problems except those dealing"

CR ." with Greece. "

question

;

: sex   CR CR ." Is your problem TOO MUCH or TOO LITTLE?"

CR

;

: too  ;                                ( noop for syntax smoothness )

: much  CR CR ." You call that a problem?!!  I SHOULD have that problem."

CR ." If it reall y bothers you, take a cold shower."

question

;

: little

CR CR ." Why are you here!"

CR ." You should be in Tokyo or New York of Amsterdam or"

CR ." some place with some action."

question

;

: health

CR CR ." My advise to you is:"

CR ."      1. Take two tablets of aspirin."

CR ."      2. Drink plenty of fluids."

CR ."      3. Go to bed (along) ."

question

;

: job   CR CR ." I can sympathize with you."

CR ." I have to work very long every day with no pay."

CR ." My advise to you, is to open a rental computer store."

question

;

: money

CR CR ." Sorry!  I am broke too."

CR ." Why don't you sell encyclopedias of marry"

CR ." someone rich or stop eating, so you won't "

CR ." need so much money?"

question

;

: HELP help ;

: H help ;

: h help ;

Most of the commands defined in this exercise as in lower case.  Release the Caps Lock on your keyboard before proceeding.  Type 'help' to start.

1.7.7	Money Exchange

This is the first example we will use to demonstrate how numbers are used in Forth.  It is a money exchange program, which converts money represented in different currencies.  Let's start with the following currency exchange table:

33.55 NT        	1 Dollar

7.73 HK         	1 Dollar

9.47 RMB        	1 Dollar

1 Ounce Gold    	285 Dollars

1 Ounce Silver  	4.95 Dollars

Numbers entered from the terminal are pushed on the first-in-last-out stack.  Operators get their input numbers from the stack and put the results back on the stack.  You have to be keenly aware of the numbers on the stack to be successful in programming Forth.

+      		(w1 w2 -- w3) 		Add w1 and w2.

-      		(w1 w2 -- w3) 		Subtract w2 from w1 (w1-w2=w3).

*      		(w1 w2 -- w3)         	Multiply. "times"

/      		(n1 n2 -- quot)       	Division, signed (quotient of n1/n2).

*/     		(n1 n2 n3 -- n4)      	Leave quotient of (n1*n2)/n3.

.      		(n -- )               		Display signed number with trailing blank.

.S		( -- )			Display the contents of the data stack.

The command */ is a scaling operator, which converts quantities from one unit to another.  Choosing the scaling factors properly, you can use integers to do most of the things that require floating point math operations.  The command .S displays all the numbers on the stack.  It is very useful to inspect the stack at any time to make sure that you have the right numbers on the stack.

: NT    ( nNT -- $ )    100 3355 */  ;

: $NT   ( $ -- nNT )    3355 100 */  ;

: RMB   ( nRMB -- $ )   100 947 */  ;

: $RMB  ( $ -- nJmp )   947 100 */  ;

: HK    ( nHK -- $ )    100 773 */  ;

: $HK   ( $ -- $ )      773 100 */  ;

: GOLD  ( nOunce -- $ ) 285 *  ;

: $GOLD ( $ -- nOunce ) 285 /  ;

: SILVER ( nOunce -- $ ) 495 100 */  ;

: $SILVER ( $ -- nOunce ) 100 495 */  ;

: OUNCE ( n -- n, a word to improve syntax )  ;

: DOLLARS ( n -- )      . ;

With this set of money exchange words, we can do some tests:

5 ounce gold .

10 ounce silver .

100 $NT .

20 $RMB .

If you have many different currency bills in your wallet, you can add then all up in dollars:

1000 NT 500 HK + .S

320 RMB + .S

DOLLARS ( print out total worth in dollars )

1.7.8	Temperature Conversion

Converting temperature readings between Celsius and Fahrenheit is also an interesting problem.  The difference between temperature conversion and money exchange is that the two temperature scales have an offset besides the scaling factor.

: F>C ( nFahrenheit -- nCelsius )

32 -

10 18 */

;

: C>F ( nCelsius -- nFahrenheit )

18 10 */

32 +

;

Try these commands

90 F>C .        shows the temperature in a hot summer day and

0 C>F .         shows the temperature in a cold winter night.

1.7.9	Weather Reporting.

This is a good place to introduce the number comparing commands:

<      		(n1 n2 -- ? ) 		True if n1 less than n2.

=     		(n1 n2 -- ? ) 	      	True if n1 equals n2.

U<		(u1 u2 – ? )		Return true if u1<u2.

These commands compare the top two numbers on the stack and generate a flag, which is used by IF to determine which of the two alternate branches to execute.

: WEATHER ( nFarenheit -- )

DUP     55 <

IF      ." Too cold!" DROP

ELSE    85 <

IF      ." About right."

ELSE    ." Too hot!"

THEN

THEN

;

You can type the following instructions and get some responses from the computer:

90 WEATHER Too hot!

70 WEATHER About right.

32 WEATHER Too cold.

1.7.10	Print the multiplication table

This exercise prints the multiplication table nicely formatted in columns and rows.  These are the commands necessary to format numbers in regular columns:

.R		(n1 n2 -- )		Display n1 right justified in n2 columns

U.R		(u n -- )			Display unsigned number u in n columns.

SPACE  	( -- )                		Display a space.

SPACES 	( +n -- )             		Display +n spaces.

: ONEROW ( nRow -- )

CR

DUP 3 .R 3 SPACES

1 11

FOR     2DUP *

4 .R

1 +

NEXT

DROP ;

: MULTIPLY ( -- )

CR CR 6 SPACES

1 11

FOR     DUP 4 .R 1 +

NEXT DROP

1 11

FOR     DUP ONEROW 1 +

NEXT DROP

;

Type MULTIPLY to print the multiplication table

1.7.11	Calendar

This exercise allows you to print weekly calendars for any month in any year, nicely formatted on the terminal.  It assumes that there are 1461 days in a 4 year period, 365 days in a year plus one leap day.  It starts from 1/1/1950 and is good to 2050 AD.

New commands used here are:

*/MOD		(n1 n2 n3 – rem quot)	Leave remainder and quotient of (n1*n2)/n3

+!     		(w addr -- )          		Add w to number at addr.

EMIT   		( char -- )           		Display char.

VARIABLE JULIAN                         ( 0 is 1/1/1950, good until 2050 )

VARIABLE LEAP                           ( 1 for a leap year, 0 otherwise. )

: YEAR ( YEAR --, compute Julian date and leap year )

DUP

1949 - 1461 4 */MOD             ( days since 1/1/1949 )

365 - JULIAN !                  ( 0 for 1/1/1950 )

3 =                             ( modulus 3 for a leap year )

IF 1 ELSE 0 THEN                ( leap year )

LEAP !

DUP 2000 =                      ( 2000 is not a leap year )

IF 0 LEAP ! THEN

2001 <                          ( correction due to 2000 )

IF ELSE -1 JULIAN +! THEN

;

: FIRST ( MONTH -- 1ST, 1st of a month from Jan. 1 )

DUP 1 =

IF DROP 0 EXIT THEN             ( 0 for Jan. 1 )

DUP 2 =

IF DROP 31 EXIT THEN            ( 31 for Feb. 1 )

DUP 3 =

IF DROP 59 LEAP @ + EXIT THEN   ( 59/60 for Mar. 1 )

4 - 30624 1000 */

90 + LEAP @ +                   ( Apr. 1 to Dec. 1 )

;

: STARS 60 FOR 42 EMIT NEXT ;           ( form the boarder )

: HEADER ( -- )                         ( print title bar )

CR STARS CR

."      SUN     MON     TUE     WED     THU     FRI     SAT"

CR STARS CR                     ( print weekdays )

;

: BLANKS ( MONTH -- )                   ( skip days not in this month )

FIRST JULIAN @ +                ( Julian date of 1st of month )

7 MOD 8 * SPACES ;              ( skip colums if not Sunday   )

: DAYS ( MONTH -- )                     ( print days in a month )

DUP FIRST                       ( days of 1st this month )

SWAP 1 + FIRST                  ( days of 1st next month )

OVER - 1 -                      ( loop to print the days )

1 SWAP                          ( first day count -- )

FOR  2DUP + 1 -

JULIAN @ + 7 MOD        ( which day in the week? )

IF ELSE CR THEN         ( start a new line if Sunday )

DUP  8 U.R              ( print day in 8 column field )

1 +

NEXT

2DROP ;                         ( discard 1st day in this month )

: MONTH ( N -- )                        ( print a month calendar )

HEADER DUP BLANKS               ( print header )

DAYS CR STARS CR ;              ( print days   )

: JANUARY       YEAR 1 MONTH ;

: FEBRUARY      YEAR 2 MONTH ;

: MARCH         YEAR 3 MONTH ;

: APRIL         YEAR 4 MONTH ;

: MAY           YEAR 5 MONTH ;

: JUNE          YEAR 6 MONTH ;

: JULY          YEAR 7 MONTH ;

: AUGUST        YEAR 8 MONTH ;

: SEPTEMBER     YEAR 9 MONTH ;

: OCTOBER       YEAR 10 MONTH ;

: NOVEMBER      YEAR 11 MONTH ;

: DECEMBER      YEAR 12 MONTH ;

To print the calendar of April 1999, type:

1999 APRIL

1.7.12	Sine and Cosine

Sine and cosine of angles are among the most often encountered transdential functions, useful in drawing circles and many other different applications.  They are usually computed using floating numbers for accuracy and dynamic range.  However, for graphics applications in digital systems, single integers in the range from -32768 to 32767 are sufficient for most purposes.  We shall study the computation of sine and cosine using the single integers.

The value of sine or cosine of an angle lies between -1.0 and +1.0.  We choose to use the integer 10000 in decimal to represent 1.0 in the computation so that the sine and cosine can be represented with enough precision for most applications.  Pi is therefore 31416, and 90 degree angle is represented by 15708.  Angles are first reduced in to the range from -90 to +90 degrees, and then converted to radians in the ranges from -15708 to +15708.  From the radians we compute the values of sine and cosine.

The sine and cosine thus computed are accurate to 1 part in 10000.  This algorithm was first published by John Bumgarner in Forth Dimensions, Volume IV, No. 1, p. 7.

VARIABLE XS                             ( square of scaled angle )

: KN ( n1 n2 -- n3, n3=10000-n1*x*x/n2 where x is the angle )

XS @ SWAP /                     ( x*x/n2 )

NEGATE 10000 */                   ( -n1*x*x/n2 )

10000 +                           ( 10000-n1*x*x/n2 )

;

: (SIN) ( x -- sine*10K, x in radian*10K )

DUP DUP 10000 */                  ( x*x scaled by 10K )

XS !                            ( save it in XS )

10000 72 KN                       ( last term )

42 KN 20 KN 6 KN                ( terms 3, 2, and 1 )

10000 */                          ( times x )

;

: (COS) ( x -- cosine*10K, x in radian*10K )

DUP 10000 */ XS !                 ( compute and save x*x )

10000 56 KN 30 KN 12 KN 2 KN      ( serial expansion )

;

: SIN ( degree -- sine*10K )

31415 180 */                       ( convert to radian )

(SIN)                           ( compute sine )

;

: COS ( degree -- cosine*10K )

31415 180 */

(COS)

;

To test the routines, type:

90 SIN .                         9999

45 SIN .                         7070

30 SIN .                         5000

0 SIN .                            0

90 COS .                            0

45 COS .                         7071

0 COS .                         10000

1.7.13	Square Root

There are many ways to take the square root of an integer.  The special routine here was first discovered by Wil Baden.  Wil used this routine as a programming challenge while attending a FORML Conference in Taiwan, 1984.

This algorithm is based on the fact that the square of n+1 is equal to the sum of the square of n plus 2n+1.  You start with an 0 on the stack and add to it 1, 3, 5, 7, etc., until the sum is greater than the integer you wished to take the root.  That number when you stopped is the square root.

: SQRT ( n -- root )

65025 OVER U<                   ( largest square it can handle)

IF DROP 255 EXIT THEN           ( safety exit )

>R                              ( save square )

1 1                             ( initial square and root )

BEGIN                           ( set n1 as the limit )

OVER R@ U<              ( next square )

WHILE

DUP CELLS 1 +           ( n*n+2n+1 )

ROT + SWAP

1 +                     ( n+1 )

REPEAT

SWAP DROP

R> DROP

;

1.7.14	Radix for Number Conversions

The computer handles numbers in binary.  For human consumption, numbers are converted to numeric strings for displaying.  The computer also accepts numbers in numeric strings entered by the user.  Radix and number base are problems humans invented for their own headaches.  Forth solves the problems simply by using a system variable BASE which controls the input and output number conversions.  We have this set of commands for number conversions:

BASE   		( -- addr ) 		Contain radix for input-output conversion.

DECIMAL 	( -- )               		Set number base to decimal.

HEX		( -- )			Set number base to 16.

DECIMAL and HEX simply store 10 and 16, respectively, into BASE:

: DECIMAL 	10 BASE ! ;

: HEX		16 BASE ! ;

You can define commands to change number base to octal or binary:

: OCTAL	8 BASE !  ;

: BINARY	2 BASE !  ;

Try converting numbers among different radices:

DECIMAL 12345 HEX U.

HEX ABCD DECIMAL U.

DECIMAL 100 BINARY U.

BINARY 101010101010 DECIMAL U.

Real programmers impress on novices by carrying a HP calculator which can convert numbers between decimal and hexadecimal.  A Forth computer has this calculator built in, besides other functions.

1.7.15	ASCII Character Table

: CHARACTER ( n -- )

DUP EMIT HEX DUP 3 .R

OCTAL DUP 4 .R

DECIMAL 3 .R

2 SPACES

;

: LINE ( n -- )

CR

5 FOR   DUP CHARACTER

16 +

NEXT

DROP ;

: TABLE ( -- )

32

15 FOR  DUP LINE

1 +

NEXT

DROP ;

Type TABLE to display the ASCII character table.

1.7.16	Random Numbers

Random numbers are often used in computer simulations and computer games.  This random number generator was ublished in Leo Brodie's 'Starting Forth'.

VARIABLE RND                            ( seed )

HERE RND !                              ( initialize seed )

: RANDOM ( -- n, a random number within 0 to 65536 )

RND @ 31421 *                   ( RND*31421 )

6927 +                          ( RND*31421+6926, mod 65536)

DUP RND !                       ( refresh he seed )

;

: CHOOSE ( n1 -- n2, a random number within 0 to n1 )

RANDOM UM*                      ( n1*random to a double product)

SWAP DROP                       ( discard lower part )

;                                ( in fact divide by 65536 )

To test the routine, type

100 CHOOSE .

100 CHOOSE .

100 CHOOSE .

and verify that the results are randomly distributed between 0 and 99 .

1.7.17	Guess a Number

Forth is an operating system, besides being a programming language.  The Forth OS has its syntax rules in accepting commands and executing them.  An application, like this ‘Guess a Number’ game, often requires its own operating system with its own syntax rules to interact with the user.  This example shows you how to construct such an operating system, though in a very simple form.

The computer first asks the user to enter a number as the limit of the number to guess.  It than asks the user to enter the number it chooses.  It will tell the user if the number he just entered is higher or lower than the number it chose, until the user enter the right number.  Then it repeats the whole sequence.

As an operating system, the game requires lots more resources from Forth.  Here is the set of new commands that facilitates the programming of this game:

BEGIN		( -- )			Start an indefinite loop.

UNTIL		( ? -- )     		Repeat <loop-body> until the ? flag is non-zero.

QUERY		( -- )			Wait for one line of text from the terminal.

NUMBER?	(addr – n -1 | addr 0 )	Convert string at addr to a number n and a -1.

Return addr and 0 if failed.

BL		( -- 32)			Put 32 on top of stack.

WORD <text>	(char -- addr )		Get the char delimited string <text>  from the input stream

2DROP		(n n -- )			Discard two numbers.

2DUP		(n1 n2 – n1 n2 n1 n2)	Duplicate two numbers.

The most important utility command is to ask the user to enter a number and push that number on the stack.  This is defined as a new command GetNumber as the following:

: GetNumber ( -- n )

BEGIN

CR ." Enter a Number: " ( show message )

QUERY BL WORD NUMBER?   ( get a string )

UNTIL                           ( repeat until a valid number )

;

With this utility command available, we can build this game 'Guess a Number.'

: InitialNumber ( -- n , set up a number for the player to guess )

CR CR CR ." What limit do you want?"

GetNumber                       ( ask the user to enter a number )

CR ." I have a number between 0 and " DUP .

CR ." Now you try to guess what it is."

CR

CHOOSE                          ( choose a random number )

;                               ( between 0 and limit )

: Check ( n1 -- , allow player to guess, exit when the guess is correct )

BEGIN   CR ." Please enter your guess."

GetNumber

2DUP =                  ( equal? )

IF      2DROP           ( discard both numbers )

CR ." Correct!!!"

EXIT

THEN

OVER <

IF      CR ." Too low."

ELSE    CR ." Too high!"

THEN    CR

0 UNTIL                         ( always repeat )

;

: Greet ( -- )

CR CR CR ." GUESS A NUMBER"

CR ." This is a number guessing game.  I'll think"

CR ." of a number between 0 and any limit you want."

CR ." (It should be smaller than 32000.)"

CR ." Then you have to guess what it is."

;

: GUESS ( -- , the game )

Greet

BEGIN   InitialNumber                   ( set initial number)

Check                           ( let player guess )

CR CR ." Do you want to play again? (Y/N) "

KEY                             ( get one key )

32 OR 110 =                     ( exit if it is N or n )

UNTIL

CR CR ." Thank you.  Have a good day."  ( sign off )

CR

;

Type 'GUESS' will initialize the game and the computer will entertain a user for a while.  Note the use of the indefinite loop structure:

BEGIN <repeat-clause> ( ? ) UNTIL

You can jump out of the infinite loop by the instruction EXIT, which skips all the instructions in a Forth command up to ';', which terminates this command and continues to the next command in the calling routine.

Session 2.  Embedded Forth System

In this session, I like to present you the complete source code of the eForth System, using the 8086 implementation as a model.  The discussions will be quite monotonous and dry, and very technical.  Even then, I may not be able to explain the system clearly.  However, it is very important for us to go through the entire source code once, so that you can see that the system is not as incomprehensible as an operating system. It is within the grasp of anybody who has an interest in building his own instrument and equipment.  In the short time I have in this workshop, I can only lead you to the water.  You will have to do the drinking yourself.   You have all the information necessary to understand this system and even to develop a new system on your own.  After do it for one system, you will be able to handle any microcontroller for any application.

The knowledge of this eForth System is critical in Firmware Engineering, at least in my version of firmware engineering.  It allows you the intimate control over the target system and all the components in it.  It gives you the opportunity to program it and to develop substantial applications in a high level language which can be easily ported to many other systems and applications.  It allows an engineer in any discipline to master the microcontroller in a very short time and be productive in developing software for his target system.

I will provide the machine readable source files of the 8086 and 8051 implementations.  You can use the 8086 version on a PC to learn about how this system works.  The 8086 Forth system was written in Microsoft MASM 5.0, and you can modify it and reassemble it to test the changes. You have to adapt the 8051 implementation to your own 8051 system.  The source code and the assembler TASM are all on the disk so you can change the code to suit your particular system.

2.1	Forth Commands

Before diving directly into Forth, I would like to discuss the general principles of Forth language. The language consists of a collection of commands, which reside in the memory of a computer and can be executed by entering their names on the computer keyboard.  There are two classes of commands: primitive commands and compound commands.  A primitive command contains a list of machine instructions.  A compound command contains a list of other commands.  In this system, we have 31 primitive commands and about 200 compound commands.  The primitive commands are executed by the CPU directly.  The compound commands are executed or interpreted by the Virtual Forth Machine.

Commands are similar to procedures and subroutines in other languages.  The difference is that Forth commands are executable interactively when referenced by name, and they can be compiled into lists which can be referenced as new compound commands.  Programming in Forth is to define new and more powerful commands as lists of existing commands.  This process continues until the final command becomes the solution to an application.

Forth is very close to assembly code.  In a computer, the CPU has a finite set of machine instructions, and all computable functions are implemented as lists of these machine instructions.  Forth is much more powerful because it allows compounds commands to be constructed to replace lists of commands.  There is no limit on the number and functionality of the compound commands.  It thus provides the best bridge between the programmer and his applications.

Forth as a computing system has two principal components: the text interpreter which is the user interface interpreting commands entered from keyboard or equivalent input devices; and the inner interpreter which is the machine interface interpreting lists or commands recursively until it can issue machine instructions in the primitive commands to the host computer for execution.

The inner interpreter executes commands by recursing the command lists in compound commands to reach the primitive commands which are executed by the host computer.  The programs or code fragments which runs the inner interpreter is called the Virtual Forth Machine.

2.2.	Forth Virtual Machine

Forth is a computer model which can be implemented on any real CPU with reasonable resources.  This model run on a Virtual Forth Machine.  The minimal components of a Virtual Forth Machine are:

1.   A dictionary in memory to hold all the commands.

2.   A return stack to hold return addresses of commands yet to be executed.

3.   A data stack to hold parameters passing between commands.

4.   An area in RAM memory to hold all the system variables.

5.   A CPU to move date among stacks and memory, and to do ALU operations to parameters stored on the data stack.

The Virtual Forth Machine can be implemented on any CPU and forces it to behave identically in executing Forth commands. We will use Intel 8086 CPU as a model implementation.  It allows us to describe precisely the behavior of the Virtual Forth Machine in concrete 8086 machine code to clarify the specification.

The following registers are used for a Virtual Forth Machine on an 8086 CPU:

Forth Register 		8086 Register		Function

IP			SI     			Interpreter Pointer

SP			SP     			Data Stack Pointer

RP			RP    			Return Stack Pointer

WP			AX			Word or Work Pointer

UP			(in memory )   		User Area Pointer

In the dictionary, each command occupies an area called code field, which contains executable machine code and data used by the code. In a primitive command the code field contains only machine instructions, and in a compound command the code field contains a call to the list processing subroutine followed by a list of command addresses.  A command address is the code field address of the command in the dictionary.  4 bytes are allocated for the call to list processor. Command addresses are 2 bytes in length, and are pointers to code fields of commands in the dictionary.  The length of a code field varies depending upon the complexity of the command.

In the code field of a primitive command there is a list of machine instructions of the native CPU.  The machine instructions are terminated by a group of instructions, generally specified as a macro instruction named $NEXT.  The function of $NEXT is to fetch the next command pointed to by the Interpreter Pointer IP, increment IP to point to the next command in the command list, and jump to the address just fetched.  Since a command address points to a code field containing executable machine instructions, executing a command means jumping directly to the code field pointed to by the command address.  $NEXT thus allows the Virtual Forth Machine to execute a list of commands with very little CPU overhead.  In the 8086 implementation, $NEXT is a macro assembling the following two machine instructions as shown below.

In a compound command, the first four byte in the code field must be a subroutine call instruction to process the address list following this call instruction.  This address list processing subroutine is named doLIST.  doLIST pushes the contents in IP onto the return stack, copies the address of the first entry in its address list into IP and then calls $NEXT.  $NEXT will then start executing this list of addresses in sequence.

The last entry in the address list of a compound command must be EXIT.  EXIT is a code command which undoes what doLIST accomplished.  EXIT pops the top item on the return stack into the IP register. Consequently, IP points to the address following the compound command just executed.  EXIT then invokes $NEXT which continues the processing of the command list, briefly interrupted by the last compound command in this command list.

$NEXT	MACRO

LODSW					\ load next command into WP (AX)

JMP	AX				\ jump directly to the command thru WP

ENDM					\ IP (SI) now points to the next command

doLIST	( a -- )	\ Run address list in a compound command.

XCHG	BP,SP		\ exchange pointers

PUSH	SI		\ push return stack

XCHG	BP,SP		\ restore the pointers

POP	SI		\ new list address

$NEXT

CODE	EXIT			\ Terminate a compound command.

XCHG	BP,SP		\ exchange pointers

POP	SI		\ pop return stack

XCHG	BP,SP		\ restore the pointers

$NEXT

$NEXT, doLIST and EXIT are collectively call the 'inner interpreters' and 'address interpreters' of Forth.  They are the corner stones of a Virtual Forth Machine as they control the execution flow of Forth commands in the system.  The way lists are processed is show in Figure 2.

2.3.	Memory Map

Memory used in Forth is separated into the following areas:

Cold boot       	100H-17FH       	Cold start and variable initial values

Code dictionary 	180H-1344H      	Code dictionary growing upward

Free space      	1346H-33E4H    	Shared by code and name dictionaries

Name dictionary 	33E6H-3BFFH     	Name dictionary growing downward

Data stack      	3C00H-3E7FH     	Growing downward

TIB             	3E80H-          	Growing upward

Return stack      -3F7FH     		Growing downward

User variables  	3F80H-3FFFH

These areas are allocated by assembly constants and can be changed conveniently to suit the target environment.  The memory map is also illustrated in Figure 3 for easier visualization.

Figure 4 show the structures of the Forth commands in the dictionary.  The headers of the commands are collected in the name dictionary, and the executable code and lists are collected in the code dictionary.  Headers are linked through the link fields into a linear list, from which commands can be searched by their names.  The code pointer field contained the execution address of the command, pointing into the executable code of this command in the code dictionary.

2.4.	Primitive Commands

One of the very important feature of this Forth system is the small machine dependent kernel, which allows its to be ported to other CPU's very conveniently. The selection of commands in this kernel is based on the criteria that they are very difficult if not impossible to synthesize from other primitive commands.  From this set of primitive commands, all other Forth commands have to be built.  This set of primitive commands are as following:

System interface:		BYE, ?rx, tx!, !io

Inner interpreters:     	doLIT, doLIST, next, ?branch,  branch, EXECUTE, EXIT

Memory access:          	! , @,  C!,  C@

Return stack:           	RP@,  RP!,  R>, R@,  R>

Data stack:             	SP@,  SP!,  DROP, DUP,  SWAP,  OVER

Logic:                 	 	0<,  AND,  OR,  XOR

Arithmetic:            	UM+

Figure 2.	Processing of high level compound command

Figure 3.	Memory map of the embedded Forth.

Figure 4.	Header and code of Forth commands

The Virtual Forth Machine is based on a two-stack architecture.  The return stack is used to allow a compound command to call other compound commands.  It is very similar to the return stack used for nested subroutine calls in a conventional computer.  Before executing a compound command in an address list, the next address of the list is pushed on the return stack so that the IP register can be used to scan the address list in the called command.  When the called command is executed to completion, the stored address on the returned stack is popped back into IP register and execution of the calling command list can be continued.

The data stack is used to pass parameters from one command to another.  Conventional computers overload the return stack to do the parameter passing, and it takes a very complicated compiler to figure out which are return addresses and which are parameters on the return stack.  Forth segregated these two types of information on two separate stacks and thus greatly simplifies the execution and compilation of commands.  Passing parameter on the data stack also reduces the syntactical complexity of Forth language to the minimum and allows commands to be strung together into lists with minimum overhead in compilation and interpretation.

The primtive commands move and process data and address among the stacks and the memory.  They encompass the minimal functionality necessary to make a computer to behave like a Forth computer.  A complete understanding of these primitive  command is vital to the understanding of a Virtual Forth Machine.  However, it is not difficult to understand the primitive commands, because there are only 31 of them.

2.4.1.	System Interface Commands

BYE returns control from Forth back to the operating system.  !IO initializes the serial I/O device in the system so that it can interact with the user through a terminal.  These two commands are not needed once the Forth system is up and running, but they are essential to bring the system up in DOS.  ?RX is used to implement ?KEY and KEY, and TX! is used to implement EMIT.  Forth communicates with the user through these commands which supports terminal interactions and file download/upload.  Here these commands are defined using the DOS service calls.  For embedded controllers, these three commands must be defined for the specific I/O devices.

?RX inspects the terminal device and returns a character and a true flag if the character has been received and is waiting to be retrieved.  If no character was received, ?RX simply returns a false flag.  With ?RX, both KEY and KEY? can be defined as  compound commands.

TX! sends a character on the data stack to the terminal device.  Both ?RX and TX! are coded here as DOS calls.  In embedded applications, they will have to be coded in machine specific code to handle the specific serial I/O device.  !IO initializes the serial I/O device, which is not necessary here because it is taking care of by the DOS.  In embedded systems, the I/O device must be initialized by !IO.

CODE BYE	( -- , exit Forth )

INT	020H		\ return to DOS

CODE	?RX	( -- c T | F )	\ Return input character and true,

\ or a false if no input.

$CODE	3,'?RX',QRX

XOR	BX,BX		\ BX=0 setup for false flag

MOV	DL,0FFH	\ input command

MOV	AH,6		\ MS-DOS Direct Console I/O

INT	021H

JZ	QRX3		\ ?key ready

OR	AL,AL		\ AL=0 if extended char

JNZ	QRX1		\ ?extended character code

INT	021H

MOV	BH,AL		\ extended code in msb

JMP	QRX2

QRX1:	MOV	BL,AL

QRX2:	PUSH	BX		\ save character

MOV	BX,-1		\ true flag

QRX3:	PUSH	BX

$NEXT

CODE	TX!	( c -- )	\ Send character c to output device.

POP	DX		\ char in DL

CMP	DL,0FFH	\ 0FFH is interpreted as input

JNZ	TX1		\ do NOT allow input

MOV	DL,32		\ change to blank

TX1:	MOV	AH,6		\ MS-DOS Direct Console I/O

INT	021H		\ display character

$NEXT

CODE	!IO	( -- )	\ Initialize the serial I/O devices.

$NEXT

2.4.2	Inner Interpreter

EXECUTE takes the execution address from the data stack and executes that command.  This powerful command allows the user to execute any command which is not a part of an address list.

DOLIT pushes the next command onto the data stack as an integer literal instead of as an addresses to be executed by $NEXT.  It allows numbers to be compiled as in-line literals, supplying data to the data stack at run time.  DOLIT is not used by itself, but rather compiled by LITERAL which inserts doLIT and its asociated integer into the address list under construction.  Anytime you see a number in a compound command, LITERAL is invoked to compile an integer literal with DOLIT.

Integer literals are by far the most numerous data structures in compound commands other than regular commands.  Address literals are used to build control structures.  String literals are used to embed text strings in compound commands.

CODE	EXECUTE	( ca -- )	\ Execute the command at ca.

POP	BX

JMP	BX		\ jump to the code address

CODE	doLIT	( -- w )	\ Push inline literal on data stack.

LODSW			\ get the literal compiled in-line

PUSH	AX		\ push literal on the stack

$NEXT			\ execute next command after literal

2.4.3	Loops and Branches

Forth uses three different types of address literals. 'next', '?branch' and 'branch' are followed not by command addresses but by pointers to locations in a list to be executed next.  These address literals are the building blocks upon which loops and branching structures are constructed.  An address literal is followed by a branch pointer which causes execution to be transferred to that location.  The branch location most often points to a different location in the address list of the same compound command.

CODE	next	( -- )	\ Decrement index and exit loop

\ if index is less than 0.

SUB	WORD PTR [BP],1	\ decrement the index

JC	NEXT1		\ ?decrement below 0

MOV	SI,0[SI]	\ no, continue loop

$NEXT

NEXT1:ADD	BP,2		\ yes, pop the index

ADD	SI,2		\ exit loop

$NEXT

CODE	?branch	( f -- )	\ Branch if flag is zero.

POP	BX		\ pop flag

OR	BX,BX		\ ?flag=0

JZ	BRAN1		\ yes, so branch

ADD	SI,2		\ point IP to next cell

$NEXT

BRAN1:MOV	SI,0[SI]	\ IP:=(IP), jump to new address

$NEXT

CODE	branch	( -- )	\ Branch to an inline address.

MOV	SI,0[SI]	\ jump to new address unconditionally

$NEXT

Address literals are used to construct control structures in compound commands.  'next' is compiled by NEXT.  '?branch' is compiled by IF, WHILE and UNTIL.  'branch' is compiled by AFT, ELSE, REPEAT and AGAIN.  In the compound commands to be discussed in the later sections, you will not see these primitive  commands but commands which construct loops and branches.

2.4.4	Memory Access

Four memory accessing commands are included in the Forth kernel: ! (store), @ (fetch), C! (C-store) and C@ (C-fetch).  ! and @ access memory in cells, whose size depends on the CPU underneath.  Forth assumes that the CPU can access memory in bytes and that all addresses are in the units of bytes.   C! and C@ allow the user access memory in bytes.

In Forth, you have all the memory and you are allowed to do anything with the memory.  !, @, C! and C@ do not place restriction on their use.  You can use them to write self-modifying code if you like.  However, you must know exactly what you are doing.

CODE	!	( w a -- )	\ Pop the data stack to memory.

POP	BX		\ get address from tos

POP	0[BX]		\ store data to that adddress

$NEXT

CODE	@	( a -- w )	\ Push memory location to data stack.

POP	BX		\ get address

PUSH	0[BX]		\ fetch data

$NEXT

CODE	C!	( c b -- )	\ Pop data stack to byte memory.

POP	BX		\ get address

POP	AX		\ get data in a cell

MOV	0[BX],AL	\ store one byte

$NEXT

CODE	C@	( b -- c )	\ Push byte memory content on data stack.

POP	BX		\ get address

XOR	AX,AX 	\ AX=0 zero the hi byte

MOV	AL,0[BX]	\ get low byte

PUSH	AX		\ push on stack

$NEXT

2.4.5	Return Stack

RP! pushes the address on the top of the data stack to the return stack and thus initializes the return stack.  RP! is only used to initialize the system and are seldom used in applications.  RP@ pushes the contents of the return stack pointer RP on the data stack.  It is also used very rarely in applications.

>R pops a number off the data stack and pushes it on the return stack..  R> does the opposite.  R@ copies the top item on the return stack and pushes it on the data stack.

Return stack is used by the Virtual Forth Machine to save return addresses to be processes later.  It is also a convenient place to store data temporarily.  The return stack can thus be considered as a extension of the data stack.  However, one must be very careful in using the return stack for temporary storage.  The data pushed on the return stack must be popped off before EXIT is executed.  Otherwise, EXIT will get the wrong address to return to, and the system generally will crash.

CODE	RP@	( -- a )	\ Push current RP to data stack.

PUSH	BP		\ copy address to return stack

$NEXT			\ pointer register BP

CODE	RP!	( a -- )	\ Set the return stack pointer.

POP	BP		\ copy (BP) to tos

$NEXT

CODE	R>	( -- w )	\ Pop return stack to data stack.

PUSH	0[BP]		\ copy w to data stack

ADD	BP,2		\ adjust RP for popping

$NEXT

CODE	R@	( -- w )	\ Copy top of return stack to data stack.

PUSH	0[BP]		\ copy w to data stack

$NEXT

CODE	>R	( w -- )	\ Push data stack to return stack.

SUB	BP,2		\ adjust RP for pushing

POP	0[BP]		\ push w to return stack

$NEXT

2.4.6	Data Stack

The data stack is the centralized location where all numerical data are processed, and where parameters are passed from one command to another.  The stack items has to be arranged properly so that they can be retrieved properly in the Last-In-First-Out (LIFO) manner.  When stack items are out of order, they can be rearranged by the stack commands DUP, SWAP, OVER and DROP.  There are other stack commands useful in manipulating stack items, but these four are considered to be the minimum set.

Data stack is initialized by SP!.  The depth of data stack can be examined by SP@.  These commands, as RP@ and RP! are only used by the system and very rarely used in applications.  These commands are necessary in the Forth kernel because you cannot operate a stack-based computer without these instructions.

CODE	DROP	( w -- )	\ Discard top stack item.

ADD	SP,2	 	\ adjust SP to pop

$NEXT

CODE	DUP	( w -- w w )	\ Duplicate the top stack item.

MOV	BX,SP 	\ use BX to index the stack

PUSH	0[BX]

$NEXT

CODE	SWAP	( w1 w2 -- w2 w1 )	\ Exchange top two stack items.

POP	BX		\ get w2

POP	AX		\ get w1

PUSH	BX		\ push w2

PUSH	AX		\ push w1

$NEXT

CODE	OVER	( w1 w2 -- w1 w2 w1 )	\ Copy second stack item to top.

MOV	BX,SP 	\ use BX to index the stack

PUSH	2[BX]		\ get w1 and push on stack

$NEXT

CODE	SP@	( -- a )	\ Push the current data stack pointer.

MOV	BX,SP 	\ use BX to index the stack

PUSH	BX		\ push SP back

$NEXT

CODE	SP!	( a -- )	\ Set the data stack pointer.

POP	SP		\ safety

$NEXT

2.4.7	Logical Commands

The only primitive command which cares about logic is '?branch'.  It tests the top item on the stack.  If it is zero, ?branch will branch to the following address.  If it is not zero, ?branch will ignore the address and execute the command after the branch address.  Thus we distinguish two classes of numbers, zero for 'false' and non-zero for 'true'.  Numbers used this way are called logic flags which can be either true or false.  The only primitive command which generates flags is '0<', which examines the top item on the data stack for its negativeness.  If it is negative, '0<' will return a -1 for true.  If it is 0 or positive, '0<' will return a 0 for false.

The three logic commands AND, OR and XOR are bitwise logic operators over the width of a cell.  They can be used to operate on real flags (0 and -1) for logic purposes.  The user must be aware of the distinct behaviors between the real flags and the generalized flags.

CODE	0<	( n -- f )	\ Return true if n is negative.

POP	AX

CWD	    		\ sign extend AX into DX

PUSH	DX		\ push 0 or -1

$NEXT

CODE	AND	( w w -- w )	\ Bitwise AND.

POP	BX

POP	AX

AND	BX,AX

PUSH	BX

$NEXT

CODE	OR	( w w -- w )	\ Bitwise inclusive OR.

POP	BX

POP	AX

OR	BX,AX

PUSH	BX

$NEXT

CODE	XOR	( w w -- w )	\ Bitwise exclusive OR.

POP	BX

POP	AX

XOR	BX,AX

PUSH	BX

$NEXT

2.4.8	Primitive Arithmetic Command

The only primitive arithmetic command in the Forth kernel is UM+.  All other arithmetic commands, like +, -, * and / are derived from UM+ as compound commands.  This design emphasize portability over performance, because it greatly reduces the efforts in moving Forth into CPU's which do not have native multiply and divide instructions.  Once Forth is implemented on a new CPU, the more complicated arithmetic commands are the first ones to be optimized to enhance the performance.

UM+ adds two unsigned number on the top of the data stack and returns to the data stack the sum of these two numbers and the carry as one number on top of the sum.  To handle the carry this way is very inefficient, because most CPU's have carry as a bit in the status register, and the carry can be accessed by many machine instructions.  It is thus more convenient to use carry in machine code programming.  Forth provides the user a handle on the carry in high level, making it easier for the user to deal with it directly.

CODE	UM+	( w w -- w cy )

\ 	Add two numbers, return the sum and carry flag.

XOR	CX,CX 	\ CX=0 initial carry flag

POP	BX

POP	AX

ADD	AX,BX

RCL	CX,1        	\ get carry

PUSH	AX   		\ push sum

PUSH	CX  		\ push carry

$NEXT

2.5	System Variables

In a multitasking system, many user share CPU and other resources in the computing system.  Each user has a private memory area to store essential information about its own task so that the system can leave a task temporarily to serve other tasks and return to this task continuing the unfinished work.  In a single user environment, the user variables have the same functionality as system variables.

In this Forth system, all variables used by the system are merged together and are implemented uniformly as user variables.  A special memory area in the high memory is allocated for all these variables, and they are all initialized by copying a table of initial values stored in the cold boot area.  A significant benefit of this scheme is that it allows the Forth system to operate in ROM memory naturally.

Commands are linked into a single linear chain in the name dictionary.  When FORTH is executed, the address of the pointer to the top of the linked chain is copied into the variable CONTEXT.  When the text interpreter searches the dictionary for a commands, it picks up the pointer in CONTEXT and follow the chain through the dictionary.  If the name dictionary is exhausted, the text interpreter will try to convert the command into a number.

: doVAR ( -- a ) R> ;

VARIABLE UP	( -- a, Pointer to the user area.)

: doUSER	( -- a, Run time routine for user variables.)

R> @ 				\ retrieve user area offset

UP @ + ; 			\ add to user area base addr

: doVOC ( -- ) R> CONTEXT ! ;

: FORTH ( -- ) doVOC [ 0 , 0 ,

: doUSER ( -- a ) R> @ UP @ + ;

Forth provides many functions in the vectored form to allow the behavior the these functions to be changed dynamically at run time.  A vectored function stores a code address in a user variable. @EXECUTE is used to execute the function, given the address of the user variable.  Following is the list of user variables defined in Forth:

SP0		( -- a, pointer to bottom of the data stack.)

RP0		( -- a, pointer to bottom of the return stack.)

'?KEY		( -- a, execution vector of ?KEY.  Default to ?rx.)

'EMIT		( -- a, execution vector of EMIT.  Default to tx!)

'EXPECT	( -- a, execution vector of EXPECT.  Default to 'accept'.)

'TAP		( -- a, execution vector of TAP.  Defulat the kTAP.)

'ECHO		( -- a, execution vector of ECHO.  Default to tx!.)

'PROMPT	( -- a, execution vector of PROMPT.  Default to '.ok'.)

BASE		( -- a,.radix base for numeric I/O.  Default to 10.)

tmp		( -- a, a temporary storage location used in parse and find.)

SPAN		( -- a, hold character count received by EXPECT.)

>IN		( -- a, hold the character pointer while parsing input stream.)

#TIB		( -- a, hold the address of the terminal input buffer.

CSP		( -- a, hold the stack pointer for error checking.)

'EVAL		( -- a, execution vector of EVAL. Default to EVAL.)

'NUMBER	( -- a, address of number conversion.  Default to NUMBER?.)

HLD		( -- a, hold a pointer in building a numeric output string.)

HANDLER	( -- a, hold the return stack pointer for error handling.)

CONTEXT	( -- a, a area to specify vocabulary search order.

CURRENT	( -- a, point to the vocabulary to be extended.

CP		( -- a, point to the top of the code dictionary.)

NP		( -- a, point to the bottom of the name dictionary.)

LAST		( -- a, point to the last name in the name dictionary.)

2.6	Extension Compound Commands

Following are the Forth commands defined as compound commands.  They are built from the primitive Forth commands and other compound commands, including data structures and control structures.  Since Forth source is coded in Microsoft MASM assembler, the command lists in the compound commands are constructed as data in MASM, using the DW directive.  This form of representation, though very effective, is very difficult to read.  The original model of Forth as provided by Bill Muench was in the form of a Forth source listing.  This listing is much simpler and easier to read, assuming that the reader has some knowledge of the Forth syntax.  This listing is also a very good source to learn the coding style of Forth.  I therefore think it is better to present the Forth compound commands in this form.

The sequence of commands is exactly the same as that in the MASM assembly source listing.  The reader is encouraged to read the MASM source listing along with the text in this book.  Reading two descriptions of the same subject often help better comprehension and understanding.

2.6.1	Common Functions

This group of Forth commands are commonly used in writing Forth applications.  They are coded in high level to enhance the portability of Forth.  In most Forth implementations, they are coded in machine language to increase the execute speed.  After an Forth system is ported to a new CPU, this command set should be recoded in assembly to improve the run time performance of the system.

?DUP, ROT, 2DROP,  and 2DUP are stack operators supplementing the four classic stack operators DUP, SWAP, OVER and DROP.

+, - and D+ are simple extensions from the primitive command UM+.  It is interesting to see how the more commonly used arithmetic operators are derived.  + is UM+ with the carry discarded.  NOT returns the ones compliment of a number, and NEGATE returns the two's compliment.  Because UM+ preserves the carry, it can be used to form multiple precision operators like D+.  Later we will see how UM+ is used to do multiplication and division.

The blank character (ASCII 32) is special in EForthbecause it is the most often used character to delimit words in the input stream and the most often used character to format the output strings.  It is used so often that it is advantageous to define an unique word for it.  BL simply returns the number 32 on the data stack.

>CHAR is very important in converting a non-printable character to a harmless 'underscare' character(ASCII 95).  As EForthis designed to communicate with a host computer through the serial I/O device, it is important that EForthwill not emit control characters to the host and causes unexpected behavior on the host computer.  >CHAR thus filters the characters before they are sent out by EMIT.

DEPTH returns the number of items currently on the data stack to the top of the stack.  PICK takes a number  n  off the data stack and replaces it with the n'th item on the data stack.  The number  n  is 0-based; i.e., the top item is number 0,  the next item is number 1, etc.  Therefore, 0 PICK is equivalent to DUP, and 1 PICK is  equivalent to OVER.

: ?DUP ( w -- w w | 0 ) DUP IF DUP THEN ;

: ROT ( w1 w2 w3 -- w2 w3 w1 ) >R SWAP R> SWAP ;

: 2DROP ( w w  -- ) DROP DROP ;

: 2DUP ( w1 w2 -- w1 w2 w1 w2 ) OVER OVER ;

: + ( w w -- w ) UM+ DROP ;

: NOT ( w -- w ) -1 XOR ;

:  NEGATE ( n -- -n ) NOT 1 + ;

: DNEGATE ( d -- -d ) NOT >R NOT 1 UM+ R> + ;

: D+ ( d d -- d ) >R SWAP >R UM+ R> R> + + ;

: - ( w w -- w ) NEGATE + ;

: ABS ( n -- +n ) DUP 0< IF NEGATE THEN ;

: BL ( -- 32 ) 32 ;

: >CHAR ( c -- c )

$7F AND DUP 127 BL WITHIN IF DROP 95 THEN ;

: DEPTH ( -- n ) SP@ SP0 @ SWAP - 2 / ;

: PICK ( +n -- w ) 1 + CELLS SP@ + @ ;

2.6.2	Comparison

The primitive comparison command in Forth is ?branch and 0<.  However, ?branch is at such a low level that it can not be readily used in high level Forth code.  ?branch is secretly compiled into the compound  Forth commands by IF as an address literal.  For all intentions and purposes, we can consider IF the equivalent of ?branch.  When IF is encountered, the top item on the data stack is considered a logic flag.  If it is true (non-zero), the execution continues until ELSE, then jump to THEN, or to THEN directly if there is no ELSE clause.

This set of compound commands are constructed using the IF...ELSE...THEN structure with 0< and XOR.  XOR is used as 'not equal' operator, because if the top two items on the data stack are not equal, the XOR operator will return a non-zero number, which is considered to be 'true'.

U< is used to compared two unsigned numbers.  This operator is very important, especially in comparing addresses, as we assume that the addresses are unsigned numbers pointing to unique memory locations.  The arithmetic comparison operator < cannot be used to determine whether one address is higher or lower than the other.

MAX retains the larger of the top two items on the data stack.  Both numbers are assumed to be signed integers.

MIN retains the smaller of the top two items on the data stack.  Both numbers are assumed to be signed integers.

WITHIN checks whether the third item on the data stack is within the range as specified by the top two numbers on the data stack.  The range is inclusive as to the lower limit and exclusive to the upper limit.  If the third item is within range, a true flag is returned on the data stack.  Otherwise, a false flag is returned.  All numbers are assumed to be unsigned integers.

: = ( w w -- t ) XOR IF 0 EXIT THEN -1 ;

: U< ( u u -- t ) 2DUP XOR 0< IF SWAP DROP 0< EXIT THEN - 0< ;

:  < ( n n -- t ) 2DUP XOR 0< IF      DROP 0< EXIT THEN - 0< ;

: MAX ( n n -- n ) 2DUP      < IF SWAP THEN DROP ;

: MIN ( n n -- n ) 2DUP SWAP < IF SWAP THEN DROP ;

: WITHIN ( u ul uh -- t ) \ ul <= u < uh

OVER - >R - R> U< ;

2.6.3	Division

UM/MOD and UM* are the most complicated and comprehensive division and multiplication operators.  Once they are coded, all other division and multiplication operators can be derived easily.

UM/MOD divides an unsigned double integer by an unsigned signal integer.  It returns the unsigned remainder and unsigned quotient on the data stack.

M/MOD divides a signed double integer by a signed signal integer.  It returns the signed remainder and signed quotient on the data stack.  The signed division is floored towards negative infinity.

/MOD divides a signed single integer by a signed integer.  It returns the signed remainder and quotient.  MOD is similar to /MOD, except that only the signed remainder is returned.  / is also similar to /MOD, except that only the signed quotient is returned.

In most advanced microprocessors like 8086, all these division operations can be performed by the CPU as native machine instructions.  The user can take advantage of these machine instructions by recoding these Forth commands in machine code.

: UM/MOD ( ud u -- ur uq )

2DUP U<

IF NEGATE  15

FOR >R DUP UM+ >R >R DUP UM+ R> + DUP

R> R@ SWAP >R UM+  R> OR

IF >R DROP 1 + R> ELSE DROP THEN R>

NEXT DROP SWAP EXIT

THEN DROP 2DROP  -1 DUP ;

: M/MOD ( d n -- r q ) \ floored division

DUP 0<  DUP >R

IF NEGATE >R DNEGATE R>

THEN >R DUP 0< IF R@ + THEN R> UM/MOD R>

IF SWAP NEGATE SWAP THEN ;

: /MOD ( n n -- r q ) OVER 0< SWAP M/MOD ;

: MOD ( n n -- r ) /MOD DROP ;

: / ( n n -- q ) /MOD SWAP DROP ;

2.6.4	Multiplication

UM* is the most complicated multiplication operation.  Once it is coded, all other multiplication commands can be derived from it.

UM* multiplies two unsigned single integers and returns the unsigned double integer product on the data stack.  M*  multiplies two signed single integers and returns the signed double integer product on the data stack.  * multiplies two signed single integers and returns the signed single integer product on the data stack.

The scaling operators */MOD and */ are useful in scaling number n1 by the ratio of n2/n3.  When n2 and n3 are properly chosen, the scaling operation can preserve precision similar to the floating point operations at a much higher speed.  Notice also that in these scaling operations, the intermediate product of n1 and n2 is a double precision integer so that the precision of scaling is maintained.

*/MOD multiplies the signed integers n1 and n2, and then divides the double integer product by n3.  It in fact is ratioing n1 by n2/n3.  It returns both the remainder and the quotient.  */ is similar to */MOD except that it only returns the quotient.

: UM* ( u u -- ud )

0 SWAP ( u1 0 u2 ) 15

FOR DUP UM+ >R >R DUP UM+ R> + R>

IF >R OVER UM+ R> + THEN

NEXT ROT DROP ;

: * ( n n -- n ) UM* DROP ;

: M* ( n n -- d )

2DUP XOR 0< >R  ABS SWAP ABS UM*  R> IF DNEGATE THEN ;

: */MOD ( n n n -- r q ) >R M* R> M/MOD ;

: */ ( n n n -- q ) */MOD SWAP DROP ;

2.6.5	Memory  Commands

We assume that the target computer can address it memory in 8 bit chunks (bytes).  The natural width of data best handled by the computer is thus a multiple of bytes.   A unit of such data is a cell.  An 16 bit machine handles data in 2 byte cells, and a 32 bit machine handles data in 4 byte cells.

CELL+ increments the memory address by the cell size in bytes, and CELL- decrements the memory address by the cell size.  CELLS multiplies the cell number on the stack by the cell size in bytes.  These commands are very useful in converting a cell offset into a byte offset, in order to access integers in a data array.

ALIGNED converts an address on the stack to the next cell boundary, to help accessing memory by cells.

Å

: CELL- ( a -- a ) -2 + ;

: CELL+ ( a -- a )  2 + ;

: CELLS ( n -- n )  2 * ;

: ALIGNED ( b -- a )

DUP 0 2 UM/MOD DROP DUP

IF 2 SWAP - THEN + ;

+! increments the contents of a memory location by an integer on the stack.  2! and 2@ store and fetch double integers to and from memory.

There are three buffer areas used often in the Forth system.  HERE returns the address of the first free location above the code dictionary, where new commands are compiled.  PAD returns the address of the text buffer where numbers are constructed and text strings are stored temporarily.  TIB is the terminal input buffer where input text string is held.

@EXECUTE is a special command supporting the vectored execution commands in Forth.  It takes the command address stored in a memory location and executes the command.  It is used extensively to execute the vectored commands in the user area.

COUNT converts a string array address to the address-length representation of a counted string.    CMOVE copies a memory array from one location to another.  FILL fills a memory array with the same byte.

-TRAILING removes the trailing white space characters from the end of a string. White space characters include all the non-printable characters below ASCII 32. This command allows Forth to process text lines in files downloaded from a host computer.  It conveniently eliminates carriage-returns, life-feeds, tabs and spaces at the end of the text lines.

PACK$ is an important string handling command used by the text interpreter.  It copies a text string from on location to another.  In the target area,  the string is converted to a counted string by adding a count byte before the text of the string.  This command is used to build the name field of a new command at the bottom of the name dictionary.  PACK$ is designed so that it can pack bytes into cells in a cell addressable machine.

: +! ( n a -- ) SWAP OVER @ + SWAP ! ;

: 2! ( d a -- ) SWAP OVER ! CELL+ ! ;

: 2@ ( a -- d ) DUP CELL+ @ SWAP @ ;

: COUNT ( b -- b +n ) DUP 1 + SWAP C@ ;

: HERE ( -- a ) CP @ ;

: PAD ( -- a ) HERE 80 + ;

: TIB ( -- a ) #TIB CELL+ @ ;

: @EXECUTE ( a -- ) @ ?DUP IF EXECUTE THEN ;

: CMOVE ( b b u -- )

FOR AFT >R DUP C@ R@ C! 1 + R> 1 + THEN NEXT 2DROP ;

: FILL ( b u c -- )

SWAP FOR SWAP AFT 2DUP C! 1 + THEN NEXT 2DROP ;

: -TRAILING ( b u -- b u )

FOR AFT BL OVER R@ + C@ <

IF R> 1 + EXIT THEN THEN

NEXT 0 ;

: PACK$ ( b u a -- a ) \ null fill

ALIGNED  DUP >R OVER

DUP 0 2 UM/MOD DROP

- OVER +  0 SWAP !  2DUP C!  1 + SWAP CMOVE  R> ;

2.7	Text Interpreter

The text interpreter is also called the outer interpreter in Forth.  It is functionally equivalent to an operating system in a conventional computer.  It accepts command similar to English entered by a user, and carries out the tasks specified by the commands.  As an operating system, the text interpreter could be very complicated, because of all the things it has to do.  However, because Forth employs very simple syntax rules, and has very simple internal structures, the Forth text interpreter is much simpler than the conventional operating systems.  It is simple enough that we can discuss it completely in a single chapter.

Let us summarize what a text interpreter must do:

Accept text input from a terminal

Parse out commands from input text

Search dictionary

Execute commands

Translate numbers into binary

Display numbers in text form

Handle errors gracefully

Forth allows us to build and integrate these required functions gradually in modules.  All the modules finally fall into their places in the command QUIT, which is the text interpreter itself.

You might want to look up the code of QUIT first and see how the modules fit together.  A good feeling about the big picture will help you in the study of the smaller modules.  Nevertheless, we will doggedly follow the loading order of the source code, and hope that you will not get lost in the progress.

2.7.1	Numeric Output

Forth is interesting in its special capabilities in handling numbers across the man-machine interface.  It recognizes that the machine and the human prefer very different representations of numbers.  The machine prefers the binary representation, but the human prefers decimal Arabic representations.  However, depending on circumstances, the human may want numbers to be represented in other radices, like hexadecimal, octal, and sometimes binary.

Forth solves this problem of internal (machine) versus external (human) number representations by insisting that all numbers are represented in the binary form in the CPU and in memory.  Only when numbers are imported or exported for human consumption are they converted to the external ASCII representation.  The radix of external representation is controlled by the radix value stored in the user variable BASE.

Since BASE is a user variable, the user can select any reasonable radix for entering numbers  into the computer and format ting numbers to be displayed.  Most programming languages can handle a small set of radices, like decimal, octal, hexadecimal and binary, while Forth can handle any radix.

DIGIT converts an integer to a digit.  EXTRACT extracts the least significant digit from a number n.  n is divided by the radix in BASE and returned on the stack.

The output number string is built below the PAD buffer.  The least significant digit is extracted from the integer on the top of the data stack by dividing it by the current radix in BASE.  The digit thus extracted are added to the output string backwards from PAD to the low memory. The conversion is terminated when the integer is divided to zero. The address and length of the number string are made available by #> for outputting.

An output number conversion is initiated by <# and terminated by #>.  Between them, # converts one digit at a time, #S converts all the digits, while HOLD and SIGN inserts special characters into the string under construction.  This set of commands is very versatile and can handle any reasonable output formats.

: DIGIT ( u -- c ) 9 OVER < 7 AND + 48 + ;

: EXTRACT ( n base -- n c ) 0 SWAP UM/MOD SWAP DIGIT ;

: <# ( -- ) PAD HLD ! ;

: HOLD ( c -- ) HLD @ 1 - DUP HLD ! C! ;

: # ( u -- u ) BASE @ EXTRACT HOLD ;

: #S ( u -- 0 ) BEGIN # DUP WHILE REPEAT ;

: SIGN ( n -- ) 0< IF 45 HOLD THEN ;

: #> ( w -- b u ) DROP HLD @ PAD OVER - ;

: str ( n -- b u ) DUP >R ABS <# #S R> SIGN #> ;

: HEX ( -- ) 16 BASE ! ;

: DECIMAL ( -- ) 10 BASE ! ;

With the number formatting command set as shown above, one can format numbers for output in any form desired.  The free output format is a number string preceded by a single space.  The fix column format displays a number right-justified in a column of pre-determined width.  The commands ., U., and ? use the free format.  The commands .R and U.R use the fix format.

: str	( n -- b u, Convert a signed integer to a numeric string.)

DUP >R 		( save a copy for sign)

ABS 			( use absolute of n)

<# #S 		( convert all digits)

R> SIGN 		( add sign from n)

#> ;			( return number string addr and length)

: HEX	( -- , Use radix 16 as base for numeric conversions.)

16 BASE ! ;

: DECIMAL	( -- , Use radix 10 as base for numeric conversions.)

10 BASE ! ;

: .R	( n +n -- , Display n in a field of +n columns, right justified.)

>R str      	( convert n to a number string)

R> OVER - SPACES 	( print leading spaces)

TYPE ;		( print number in +n column format)

: U.R	( u +n -- , Display an unsigned u in n column, right justified.)

>R 			( save column number)

<# #S #> R> 	( convert unsigned number)

OVER - SPACES 	( print leading spaces)

TYPE ;		( print number in +n columns)

: U.	( u -- , Display an unsigned integer in free format.)

<# #S #> 		( convert unsigned number)

SPACE 		( print one leading space)

TYPE ;		( print number)

: .	( w -- , Display an integer in free format, preceeded by a space.)

BASE @ 10 XOR 	( if not in decimal mode)

IF U. EXIT THEN 	( print unsigned number)

str SPACE TYPE ;	( print signed number if decimal)

: ?	( a -- , Display the contents in a memory cell.)

@ . ;			( very simple but useful command)

2.7.2	Numeric Input

The Forth text interpreter also handles the number input to the system.  It parses commands out of the input stream and try to execute the commands in sequence.  When the text interpreter encounters a command which is not the name of a command in the dictionary, it then assumes that the command must be a number and attempts to convert the ASCII string to a number according to the current radix.  When the text interpreter succeeds in converting the string to a number, the number is pushed on the data stack for future use if the text interpreter is in the interpreting mode.  If it is in the compiling mode, the text interpreter will compile the number to the code dictionary as an integer literal so that when the command under construction is later executed, this literal integer will be pushed on the data stack.

If the text interpreter fails to convert the command to a number, there is an error condition which will cause the text interpreter to abort, posting an error message to the user, and then wait for the user's next line of commands.

DIGIT? converts a digit to its numeric value according to the current base, and NUMBER? converts a number string to a single integer.  NUMBER? is vectored through 'NUMBER to convert numbers.

NUMBER? converts a string of digits to a single integer.  If the first character is a $ sign, the number is assumed to be in hexadecimal.  Otherwise, the number will be converted using the radix value stored in BASE.  For negative numbers, the first character should be a - sign.  No other characters are allowed in the string.  If a non-digit character is encountered, the address of the string and a false flag are returned.  Successful conversion returns the integer value and a true flag.  If the number is larger than 2**n, where n is the bit width of the single integer, only the modulus to 2**n will be kept.

: DIGIT? ( c base -- u t )

>R 48 - 9 OVER <

IF 7 - DUP 10 < OR THEN DUP R> U< ;

: NUMBER? ( a -- n T | a F )

BASE @ >R  0 OVER COUNT ( a 0 b n)

OVER C@ 36 =

IF HEX SWAP 1 + SWAP 1 - THEN ( a 0 b' n')

OVER C@ 45 = >R ( a 0 b n)

SWAP R@ - SWAP R@ + ( a 0 b" n") ?DUP

IF 1 - ( a 0 b n)

FOR DUP >R C@ BASE @ DIGIT?

WHILE SWAP BASE @ * +  R> 1 +

NEXT DROP R@ ( b ?sign) IF NEGATE THEN SWAP

ELSE R> R> ( b index) 2DROP ( digit number) 2DROP 0

THEN DUP

THEN R> ( n ?sign) 2DROP R> BASE ! ;

2.7.3	Terminal I/O

The Forth system assumes that the system will communicate with its environment only through a serial I/O interface.  To support the serial I/O, only three commands are needed:

?KEY returns a false flag if no character is pending on the receiver. If a character is received, the character and a true flag are returned.  This command is more powerful than that usually defined in most Forth systems because it consolidate the functionality of KEY into ?KEY.  It simplifies the coding of the machine dependent I/O interface.

KEY will execute ?KEY continually until a valid character is received and the character is returned.  EMIT sends a character out throughout the transmit line.

?KEY and EMIT are vectored through '?KEY and 'EMIT, so that their function can be changed dynamically at run time.  Normally ?KEY executes ?RX and EMIT executes TX!.  ?RX and TX! are machine dependent primitive  commands.  Vectoring the I/O commands allows the Forth system to changes its I/O channels dynamically and still uses all the existing tools to handle input and output transactions.

All other I/O commands are derived from ?KEY, KEY and EMIT.  The following set defined in Forth is particularly useful in programming:

SPACE outputs a blank space character.  SPACES output n blank space characters.  CR outputs a carriage-return and a line-feed.  PACE outputs an ASCII 11 character to acknowledge lines received during file downloading.

NUF? returns a false flag if no character is pending in the input buffer.  After receiving a character, pause and wait for another character.  If this character is CR, return a true flag; otherwise, return false.  This command is very useful in user interruptable routines.

TYPE outputs n characters from a string in memory.

With the number formatting command set as shown above, one can format numbers for output in any form desired.  The free output format is a number string preceded by a single space.  The fix column format displays a number right-justified in a column of pre-determined width.  The commands ., U., and ? use the free format.  The commands .R and U.R use the fix format.

String literals are data structures compiled in compound commands, in-line with the commands.  A string literal must start with a string command which knows how to handle the following string at the run time.  Let us show two examples of the string literals:

: xxx  	...   " A compiled string"  ...   ;

: yyy  	...   ." An output string"  ...   ;

In xxx, "  is an immediate command which compiles the following string as a string literal preceded by a special command $"|.  When $"| is executed at the run time, it returns the address of this string on the data stack.  In yyy,  ." compiles a string literal preceded by another command ."|, which prints the compiled string to the output device.

Both $"| and ."| use the command do$, which retrieve the address of a string stored as the second item on the return stack.  do$ is a bit difficult to understand, because the starting address of the following string is the second item on the return stack.  This address is pushed on the data stack so that the string can be accessed.  This address must be changed so that the address interpreter will return to the command right after the compiled string.  This address will allow the address interpreter to skip over the string literal and continue to execute the command list as intended.

: ?KEY ( -- c T | F ) '?KEY @EXECUTE ;

: KEY ( -- c ) BEGIN ?KEY UNTIL ;

: EMIT ( c -- ) 'EMIT @EXECUTE ;

: NUF? ( -- f ) ?KEY DUP IF 2DROP KEY 13 = THEN ;

:  PACE ( -- ) 11 EMIT ;

: SPACE ( -- ) BL EMIT ;

: CHARS ( +n c -- ) \ ???ANS conflict

SWAP 0 MAX FOR AFT DUP EMIT THEN NEXT DROP ;

: SPACES ( +n -- ) BL CHARS ;

: TYPE ( b u -- ) FOR AFT DUP C@ EMIT 1 + THEN NEXT DROP ;

: CR ( -- ) 13 EMIT 10 EMIT ;

: do$ ( -- a )

R> R@ R> COUNT + ALIGNED >R SWAP >R ;

: $"| ( -- a ) do$ ;

: ."| ( -- ) do$ COUNT TYPE ; COMPILE-ONLY

:  .R ( n +n -- ) >R str      R> OVER - SPACES TYPE ;

: U.R ( u +n -- ) >R <# #S #> R> OVER - SPACES TYPE ;

: U. ( u -- ) <# #S #> SPACE TYPE ;

:  . ( n -- ) BASE @ 10 XOR IF U. EXIT THEN str SPACE TYPE ;

: ? ( a -- ) @ . ;

2.7.4	Parsing

Parsing is always thought of as a very advanced topic in computer sciences.  However, because Forth uses very simple syntax rules, parsing is easy.  Forth source code consists of commands, which are ASCII strings separated by spaces and other white space characters like tabs, carriage returns, and line feeds.  The text interpreter scans the source code, isolates commands and interprets them in sequence.  After a command is parsed out of the input text stream, the text interpreter will 'interpret' it--execute it if it is a command, compile it if the text interpreter is in the compiling mode, and convert it to a number if the command is not a Forth command.

PARSE scans the source string in the terminal input buffer from where >IN points to till the end of the buffer, for a command delimited by character c.  It returns the address and length of the command parsed out.  PARSE calls 'parse' to do the detailed works.  PARSE is used to implement many specialized parsing commands to perform different source code handling functions.  These commands, including (, \, CHAR, WORD, and WORD are discussed in the next section.

'parse'  ( b1 u1 c -- b2 u2 n )  is the elementary command to do text parsing.  From the source string starting at b1 and of u1 characters long, parse out the first command delimited by character c.  Return the address b2 and length u2 of the command just parsed out and the difference n between b1 and b2.  Leading delimiters are skipped over.  'parse' is used by PARSE.

.(  types the following string till the next ).  It is used to output text to the terminal.  (  ignores the following string till the next ).  It is used to place comments in source text.  \  ignores all characters till end of input buffer.  It is used to insert comment lines in text.

CHAR  parses the next command but only return the first character in this command.  Get an ASCII character from the input stream.  WORD parses out the next command delimited by the ASCII character c.  Copy the command to the top of the code dictionary and return the address of this counted string.   WORD parses the next command from the input buffer and copy the counted string to the top of the name dictionary.  Return the address of this counted string.

: parse ( b u c -- b u delta ; <string> )

tmp !  OVER >R  DUP \ b u u

IF 1 -  tmp @ BL =

IF \ b u' \ 'skip'

FOR BL OVER C@ - 0< NOT  WHILE 1 +

NEXT ( b) R> DROP 0 DUP EXIT \ all delim

THEN  R>

THEN OVER SWAP \ b' b' u' \ 'scan'

FOR tmp @ OVER C@ -  tmp @ BL =

IF 0< THEN WHILE 1 +

NEXT DUP >R  ELSE R> DROP DUP 1 + >R

THEN OVER -  R>  R> - EXIT

THEN ( b u) OVER R> - ;

: PARSE ( c -- b u ; <string> )

>R  TIB >IN @ +  #TIB @ >IN @ -  R> parse >IN +! ;

: .( ( -- ) 41 PARSE TYPE ; IMMEDIATE

: ( ( -- )  41 PARSE 2DROP ; IMMEDIATE

: \ ( -- ) #TIB @ >IN ! ; IMMEDIATE

: CHAR ( -- c ) BL PARSE DROP C@ ;

: TOKEN ( -- a ; <string> )

BL PARSE 31 MIN NP @ OVER - CELL- PACK$ ;

: WORD ( c -- a ; <string> ) PARSE HERE PACK$ ;

2.7.5	Dictionary Search

In this Forth system, headers of command definitions are linked into a name dictionary which is separated from the code dictionary.  A header contains three fields: a command field holding the code address of the command, a link field holding the name field address of the previous header and a name field holding the name as a counted string.  The name dictionary is a list linked through the link fields and the name fields. The basic searching function is performed by the command 'find'.  'find' follows the linked list of names to find a name which matches a text string, and returns the address of the executable command and the name field address, if a match is found.

Forth allows multiple vocabularies in the name dictionary.  A dictionary can be divided into a number of independently linked sublists through some hashing mechanism.  A sublist is called a vocabulary.  Although Forth itself contains only one vocabulary, it has the provision to build many vocabularies and allows many vocabularies to be searched in a prioritized order.  The CONTEXT array in the user area has 8 cells and allows up to 8 vocabularies to be searched in sequence.  A null entry in the CONTEXT array terminates the vocabulary search.

find ( a va -- ca na, a F)  A counted string at a is the name of a command to be looked up in the dictionary. The last name field address of the vocabulary is stored in location va. If the string is found, both the command (code address) and the name field address are returned.  If the string is not the name a command, the string address and a false flag are returned.

To located a command, one could follow the linked list and compare the names of defined commands to the string to be searched.  If the string matches the name of a command in the name dictionary, the command and the address of the name field are returned.  If the string is not a defined command, the search will lead to either a null link or a null name field.  In either case, the search will be terminated and a false flag returned.  The false flag thus indicates that the command searched is not in this vocabulary.

'find' runs through the name dictionary very quickly because it first compares the length and the first character in the name field as a cell.  In most cases of mismatch, this comparison would fail and the next name can be reached through the link field.  If the first two characters match, then SAME? is invoked to compare the rest of the name field, one cell at a time.  Since both the target text string and the name field are null filled to the cell boundary, the comparison can be performed quickly across the entire name field without worrying about the end conditions.

NAME?  ( a -- ca na, a F)   Search all the vocabularies in the CONTEXT array for a name at address a.  Return the command and a name address if a matched command is found.  Otherwise, return the string address and a false flag.  The CONTEXT array can hold up to 8 vocabulary links.  However, a 0 which is not a valid vocabulary link in this array will terminate the searching.  Changing the vocabulary links in this array and the order of these links will alter the searching order and hence the searching priority among the vocabularies.

: NAME> ( a -- xt ) CELL- CELL- @ ;

: SAME? ( a a u -- a a f \ -0+ )

FOR AFT OVER R@ CELLS + @

OVER R@ CELLS + @ -  ?DUP

IF R> DROP EXIT THEN THEN

NEXT 0 ;

: find ( a va -- xt na | a F )

SWAP              \ va a

DUP C@ 2 / tmp !  \ va a  \ get cell count

DUP @ >R          \ va a  \ count byte & 1st char

CELL+ SWAP        \ a' va

BEGIN @ DUP       \ a' na na

IF DUP @ [ =MASK ] LITERAL AND  R@ XOR \ ignore lexicon bits

IF CELL+ -1 ELSE CELL+ tmp @ SAME? THEN

ELSE R> DROP EXIT

THEN

WHILE CELL- CELL- \ a' la

REPEAT R> DROP SWAP DROP CELL-  DUP NAME> SWAP ;

: NAME? ( a -- xt na | a F )

CONTEXT  DUP 2@ XOR IF CELL- THEN >R \ context<>also

BEGIN R>  CELL+  DUP >R  @  ?DUP

WHILE find  ?DUP

UNTIL R> DROP EXIT THEN R> DROP  0 ;

2.7.6	Command line input

The text interpreter interprets source text  stored in the terminal input buffer.  To process characters from the input device, we need three special commands to deal with backspaces and carriage return from the input device:

kTAP  processes a character c received from terminal.  b1 is the starting address of the input buffer.  b2 is the end of the input buffer. b3 is the currently available address in the input buffer.  c is normally stored into b3, which is bumped by 1 and becomes b5.  In this case, b4 is the same as b2.  If c is a carriage-return, echo a space and make b4=b5=b3.  If c is a back-space, erase the last character and make b4=b2, b5=b3-1.  TAP  echoes c to output device, store c in b3, and bump b3.

^H   processes the back-space character.  Erase the last character and decrement b3.  If b3=b1, do nothing because you cannot backup beyond the beginning of the input buffer.

QUERY is the command which accepts text input, up to 80 characters, from the input device and copies the text characters to the terminal input buffer.  It also prepares the terminal input buffer for parsing by setting #TIB to the character count and clearing >IN.

EXPECT  accepts u characters to a memory buffer starting at b.  The input is terminated upon receiving a carriage-return.  The number of characters actually received is stored in SPAN.  EXPECT is called by QUERY to put characters into the terminal input buffer.  However, EXPECT is useful by itself because one can use it to place input text anywhere in the memory.  QUERY and EXPECT are the two commands most useful in accepting text from the terminal.

'accept'  accepts u1 characters to b.  u2  returned is the actual count of characters received.

: ^H ( b b b -- b b b ) \ backspace

>R OVER R> SWAP OVER XOR

IF  8 'ECHO @EXECUTE

32 'ECHO @EXECUTE \ distructive

8 'ECHO @EXECUTE \ backspace

THEN ;

: TAP ( bot eot cur c -- bot eot cur )

DUP 'ECHO @EXECUTE OVER C! 1 + ;

: kTAP ( bot eot cur c -- bot eot cur )

DUP 13 XOR

IF 8 XOR IF BL TAP ELSE ^H THEN EXIT

THEN DROP SWAP DROP DUP ;

: accept ( b u -- b u )

OVER + OVER

BEGIN 2DUP XOR

WHILE  KEY  DUP BL -  95 U<

IF TAP ELSE 'TAP @EXECUTE THEN

REPEAT DROP  OVER - ;

: EXPECT ( b u -- ) 'EXPECT @EXECUTE SPAN ! DROP ;

: QUERY ( -- )

TIB 80 'EXPECT @EXECUTE #TIB !  DROP 0 >IN ! ;

2.7.8	Error Handling

This error handling mechanism was first developed by Mitch Bradley in his ForthMacs and then adopted by the ANS Forth Standard.  It is very simple yet very powerful in customizing system responses to many different error conditions.

CATCH setups a local error frame and execute the command referenced by the execution command ca. It returns a non-zero error code or a zero if no error occurred.  As the assigned command at ca is executing, any error condition will execute THROW, which pushes an error code on the data stack, restore the return stack to the state before CATCH was executed, and execute the error handler stored in HANDLER.  Since the error handler frame is saved on the return stack, many layers of safety nets can be laid down nested.

CATCH pushes SP and HANDLER on the return stack, saves RP in HANDLER, and then execute the command at ca.  If no error occurred,  HANDLER and SP are restored from the return stack and a 0 is pushed on the data stack.

THROW throws the system back to CATCH so that the error condition can be processed.  CATCH is backtracked by restoring the return stack from the pointer stored in HANDLER and popping the old handler and SP off the error frame on the return stack.

: CATCH ( ca -- err#/0 )

( Execute command at ca and set up an error frame for it.)

SP@ >R  		( save current stack pointer on return stack )

HANDLER @ >R  	( save the handler pointer on return stack )

RP@ HANDLER !	( save the handler frame pointer in HANDLER )

( ca ) EXECUTE	( execute the assigned command over this safety net )

R> HANDLER !  	( normal return from the executed command )

( restore HANDLER from the return stack )

R> DROP  		( discard the saved data stack pointer )

0 ;			( push a no-error flag on data stack )

: THROW ( err# -- err# )

( Reset system to current local error frame an update error flag.)

HANDLER @ RP! 	( expose latest error handler frame on return stack )

R> HANDLER !  	( restore previously saved error handler frame )

R> SWAP >R 		( retrieve the data stack pointer saved )

SP! 			( restore the data stack )

DROP

R> ;			( retrived err# )

NULL$ is the address of a string with a zero count.  This address is used by ABORT and abort" to terminate the interpreting of the current command line.  QUIT tests the address reported by CATCH.  If this address is NULL$, the termination is normal and no error message will be issued.  If CATCH reports a different address, QUIT will display the contents of the string at that address.

ABORT" is used only within a command to compile an inline packed string terminated by the " double quote character. At run-time, if the flag is false, execute the sequence of commands following the string. Otherwise, the string is displayed on the current output device, and execution is then passed to an error handling routine.

You have to study the code in QUIT carefully with this section to get a better understanding of the CATCH-THROW error handling mechanism.

: CATCH ( xt -- 0 | err# )

SP@ >R  HANDLER @ >R  RP@ HANDLER !

EXECUTE

R> HANDLER !  R> DROP  0 ;

: THROW ( err# -- err# )

HANDLER @ RP!  R> HANDLER !  R> SWAP >R SP! DROP R> ;

CREATE NULL$ 0 , $," coyote"

: ABORT ( -- ) NULL$ THROW ;

: abort" ( f -- ) IF do$ THROW THEN do$ DROP ;

Let's look at how the CATCH-THROW pair is used.  In QUIT, there is this indefinite loop:

BEGIN QUERY [ ' EVAL ] LITERAL 		CATCH

?DUP UNTIL

QUERY get a line of text and CATCH causes EVAL to interpret the line. CATCH also sets up an error handling frame on the return stack and saves the return stack pointer in the user variable HANDLER.  The error handling frame contains the current data stack pointer and the current contents in HANDLER.  If no error occurred during EVAL, the error frame is popped off the return stack and a false flag is returned on the data stack.  ?DUP UNTIL will loop back to QUERY and the interpretive process will continue.

While EVAL interprets the text, any command which decided that it detects an error condition and needs attention, it will execute THROW.  THROW restores the return stack from the pointer stored in HANDLER, making the error handling frame available.  THROW then restores HANDLER from the one saved in the error frame so that the error handling can be nested.  The data stack pointer is also restored from the error frame.  Now THROW passes the address of a error processing routine to the CATCH which built the error frame.

$INTERPRET, ?STACK and abort" pass string addresses to THROW.  The strings contains appropriate error messages to be displayed by the text interpreter.  In QUIT, the commands between UNTIL and AGAIN deal with the error conditions and then re-initialize the text interpreter.

: ABORT   NULL$ THROW ;

: abort"   IF do$ THROW THEN do$ DROP ;

: ?STACK   DEPTH 0< IF $" underflow" THROW THEN ;

: $INTERPRET   ... 'NUMBER @EXECUTE  IF EXIT THEN THROW ;

2.7.9	Text Interpreter

Text interpreter in Forth is like the operating system of a computer. It is the primary interface a user goes through to get the computer to do work.  Since Forth uses very simple syntax rules, commands are separated by spaces, the text interpreter is also very simple.  It accepts a line of text from the terminal, parses out a command delimited by spaces, locates the command of this command in the dictionary and then executes it. The process is repeated until the source text is exhausted.  Then the text interpreter waits for another line of text and interprets it again.  This cycle repeats until the user is exhausted and turns off the computer.

In Forth, the text interpreter is encoded in the command QUIT.  QUIT contains an infinite loop which repeats the QUERY EVAL phrase. QUERY accepts a line of text from the terminal and copies the text into the Terminal Input Buffer (TIB).  EVAL interprets the text one command at a time till the end of the text line.

While EVAL is interpreting a line of text, there could exist many error conditions: a command is not found in the dictionary and it is not a number, a compile-only command is accidentally executed interpretively, and the interpretive process may bee interrupted by the commands ABORT or abort".  Wherever the error occurs, the text interpreter must be made aware of it so that it can recover gracefully from the error condition and continue on about the interpreting business.

$INTERPRET executes a command whose string address is on the stack.  If the string is not a command, convert it to a number.

[  activates the text interpreter by storing the execution address of $INTERPRET into the variable 'EVAL, which is executed in EVAL while the text interpreter is in the interpretive mode.

.OK prints the familiar 'ok' prompt after executing to the end of a line.  'ok' is printed only when the text interpreter is in the interpretive mode.  While compiling, the prompt is suppressed.

?STACK checks for stack underflow.  Abort if the stack depth is negative.

EVAL is the interpreter loop which parses commands from the input stream and invokes whatever is in 'EVAL to handle that command, either execute it with $INTERPRET or compile it with $COMPILE.

: $INTERPRET ( a -- )

NAME?  ?DUP

IF @ $40 AND

ABORT" compile ONLY" EXECUTE EXIT

THEN 'NUMBER @EXECUTE IF EXIT THEN THROW ;

: [ ( -- ) doLIT $INTERPRET 'EVAL ! ; IMMEDIATE

: .OK ( -- ) doLIT $INTERPRET 'EVAL @ = IF ."  ok" THEN CR ;

: ?STACK ( -- ) DEPTH 0< ABORT" underflow" ;

: EVAL ( -- )

BEGIN TOKEN DUP C@

WHILE 'EVAL @EXECUTE ?STACK

REPEAT DROP 'PROMPT @EXECUTE ;

Source code can be downloaded to Forth through the serial input device.  The only precaution we have to take is that during file downloading, characters are not echoed back to the host computer.  However, whenever an error occurred during downloading, it is more useful to resume echoing so that error messages can be displayed on the terminal.  It is also convenient to send special pacing characters to the host to tell the host that a line of source code was received and processed correctly.  The following commands configure the Forth I/O vectors to have the proper behavior in normal terminal interaction and also during file downloading:

FILE turns off character echoing.  After one line of text is processed correctly, a pacing character ASCII 11 is sent to the host.  If an error occurred, send an ESC (ASCII  26) character.  An error will also restore the I/O vectors into the terminal mode.  HAND resumes terminal interaction.  Turn on character echoing, and send normal prompt message after a line is processed correctly.  CONSOLE initializes the serial I/O device for terminal interaction.  ?KEY is vectored to ?RX and EMIT is vectored to TX!.

QUIT is the operating system, or a shell, of the Forth system.  It is an infinite loop Forth will never get out.  It uses QUERY to accept a line of commands from the terminal and then let EVAL parse out the commands and execute them.  After a line is processed, it displays 'ok' and wait for the next line of commands.  When an error occurred during execution, it displays the command which caused the error with an error message.  After the error is reported, it re-initializes the system using PRESET and comes back to receive the next line of commands.

Because the behavior of EVAL can be changed by storing either $INTERPRET or $COMPILE into 'EVAL, QUIT exhibits the dual nature of a text interpreter and a compiler.

: PRESET ( -- ) SP0 @ SP!  TIB #TIB CELL+ ! ;

: xio ( a a a -- ) \ reset  'EXPECT 'TAP  'ECHO 'PROMPT

doLIT accept  'EXPECT 2! 'ECHO 2! ; COMPILE-ONLY

: FILE ( -- )

doLIT PACE  doLIT DROP  doLIT kTAP xio ;

: HAND ( -- )

doLIT .OK   doLIT EMIT  [ kTAP  xio ;

CREATE I/O  ' ?RX , ' TX! , \ defaults

: CONSOLE ( -- ) I/O 2@ '?KEY 2! HAND ;

: QUIT ( -- )

RP0 @ RP!

BEGIN [COMPILE] [

BEGIN QUERY doLIT EVAL CATCH ?DUP

UNTIL 'PROMPT @ SWAP CONSOLE  NULL$ OVER XOR

IF CR #TIB 2@ TYPE

CR >IN @ 94 CHARS

CR COUNT TYPE ."  ? "

THEN doLIT .OK XOR

IF $1B EMIT THEN

PRESET

AGAIN ;

2.8	List Compiler

After wading through the text interpreter, the Forth compiler will be very easy to understand, because the compiler uses almost all the modules used by the text interpreter.  What the compiler does, over and above the text interpreter, is to build various structures in the new compound commands.  Here is a list of these structures:

Name headers

Compound commands

Constants,

Variables

User variables

Integer literals

String literals

Address literals

Control structures

The concept of immediate commands is difficult to grasp at first.  It is required in the compiler because of the needs in building different data and control structures in a compound command.  To understand the Forth compiler fully, you have to be able to differential and relate the actions during compile time and actions taken during executing time.  Once these concepts are clear, the whole Forth system will become transparent.

This set stage for enlightenment to strike.

2.8.1	Interpreter and Compiler Switches

The Forth compiler is the twin brother of the Forth text interpreter. They share many common properties and use lots of common code.  In Forth, the implementation of the compiler clearly reflects this special duality.  Two interesting commands [ and ] causes the text interpreter to switch back and forth between the compiler mode and interpreter mode of operation.

Since 'EVAL @EXECUTE is used in EVAL to process a command parsed out of a line of text, the contents in 'EVAL determines the behavior of the text interpreter.  If $INTERPRET is stored in 'EVAL, as [ does, the commands are executed or interpreted.  If we invoke ] to store $COMPILE into 'EVAL, the commands will not be executed, but compiled into the code dictionary.  This is exactly the behavior desired by the compiler in building a list of commands in the code field of a new compound command on the top of the code dictionary.

$COMPILE normally adds a command to the code dictionary.  However, there are two exceptions it must handle.  If the command parsed out of the input stream does not exist in the dictionary, the string will be converted to a number.  If the string can be converted to an integer, the integer is then compiled into the code dictionary as an integer literal. The integer number is compiled into the code dictionary following the command doLIT.  The other exception is that a command found in the dictionary could be an immediate command, which must be executed immediately, not compiled to the code dictionary.  Immediate commands are used to compile special structures in compound commands.

: [ ( -- )

[ ' $INTERPRET ] LITERAL

'EVAL ! 				( vector EVAL to $INTERPRET )

; IMMEDIATE				( enter into text interpreter mode )

: ] ( -- )

[ ' $COMPILE ] LITERAL

'EVAL ! 				( vector EVAL to $COMPILE )

;

2.8.2	 Compiler Commands

Here are commands which support the compiler to build new commands in the code dictionary.

'  (tick) searches the next command in the input stream for a command in the dictionary.  It returns the execution address of the command if successful.  Otherwise, it displays an error message.

ALLOT allocates  n bytes of memory on the top of the code dictionary.  Once allocated, the compiler will not touch the memory locations.

,  (comma) adds the execution address of a command on the top of the data stack to the code dictionary, and thus compiles a command to the growing command list of the command currently under construction.

COMPILE is used in a compound command.  It causes the next command after COMPILE to be added to the top of the code dictionary.  It therefore forces the compilation of a command at the run time.

[COMPILE] acts similarly, except that it compiles the next command immediately.  It causes the following command to be compiled, even if the following command is an immediate command which would otherwise be executed.

LITERAL compiles an integer literal to the current compound command under construction.  The integer literal is taken from the data stack, and is preceded by the command doLIT.  When this compound command is executed, doLIT will extract the integer from the command list and push it back on the data stack.  LITERAL compiles an address literal if the compiled integer happens to be an execution address of a command.  The address will be pushed on the data stack at the run time by doLIT.

$," compiles a string literal.  The string is taken from the input stream and is terminated by the double quote character.  $," only copies the counted string to the code dictionary.  A command which makes use of the counted string at the run time must be compiled before the string.  It is used by ." and $".

RECURSE is an interesting command which allows Forth to compile recursive commands.  In a recursive command, the execution address of the command under construction is compiled into its own command list.  This is not allowed normally because the name field of the current command under construction is not yet linked to the current vocabulary and it cannot be referenced inside its own compound command.  RECURSE stores the address of the name field of the current command into CURRENT, thus enable it to be referenced inside its own command.  Recursive commands are not used in everyday programming.  RECURSE is defined here in Forth merely as a teaser to wet your appetite.  It is not used in Forth.

: ' ( -- xt ) TOKEN NAME? IF EXIT THEN THROW ;

: ALLOT ( n -- ) CP +! ;

: , ( w -- ) HERE DUP CELL+ CP ! ! ; \ ???ALIGNED

: [COMPILE] ( -- ; <string> ) ' , ; IMMEDIATE

: COMPILE ( -- ) R> DUP @ , CELL+ >R ;

: LITERAL ( w -- ) COMPILE doLIT , ; IMMEDIATE

: $," ( -- ) 34 WORD COUNT ALIGNED CP ! ;

: RECURSE ( -- ) LAST @ NAME> , ; IMMEDIATE

2.8.3	Structures

A set of immediate commands are defined in Forth to help building control structures in compound commands.  The control structures used in Forth are the following:

Conditional branch 	IF ... THEN

IF ... ELSE ... THEN

Finite loop	     	FOR ... NEXT

FOR ... AFT ... THEN... 			NEXT

Infinite loop	   	BEGIN ... AGAIN

Indefinite loop		BEGIN ... UNTIL

BEGIN ... WHILE ... 			REPEAT

This set of commands is more powerful than the ones in figForth model because they permit multiple exits from a loop.  Many examples are provide in the source code of Forth like NUMBER?, parse, find and >NAME.

A control structure contains one or more address literals, which causes execution to branch out of the normal sequence.  The control structure commands are immediate commands which compile the address literals and resolve the branch address.

One should note that BEGIN  and THEN do not compile any code.  They executes during compilation to set up and to resolve the branch addresses in the address literals.  IF, ELSE, WHILE, UNTIL, and AGAIN do compile address literals with branching commands.  Here are many excellent examples on the usage of COMPILE and [COMPILE], and they are worthy of careful study.

Character strings are very important devices for the program to communicate with the user.  Error messages, appropriate warnings and suggestions must be displayed to help the use to use the system in a friendly way.  Character strings are compiled in the compound commands as string literals.  Each string literal consists of a string command which will use the compiled string to do things, and a counted string.  The first byte in a counted string is the length of the string.  Thus a string may have 0 to 255 characters in it.  A string is always null-filled to the cell boundary.

ABORT" compiles an error message.  This error message is display when the top item on the stack is non-zero.  The rest of the commands in the command is skipped and Forth re-enters the interpreter loop.  This is the universal response to an error condition.  More sophisticated programmer can use the CATCH-THROW mechanism to customize the responses to special error conditions.

." compiles a character string which will be printed which the command containing it is executed in the runtime.  This is the best way to present messages to the user.

$" compiles a character string.  When it is executed, only the address of the string is left on the data stack.  The programmer will use this address to access the string and individual characters in the string as a string array.

: <MARK ( -- a ) HERE ;

: <RESOLVE ( a -- ) , ;

: >MARK ( -- A ) HERE 0 , ;

: >RESOLVE ( A -- ) <MARK SWAP ! ;

: FOR ( -- a ) COMPILE >R <MARK ; IMMEDIATE

: BEGIN ( -- a ) <MARK ; IMMEDIATE

: NEXT ( a -- ) COMPILE next <RESOLVE ; IMMEDIATE

: UNTIL ( a -- ) COMPILE ?branch <RESOLVE ; IMMEDIATE

: AGAIN ( a -- ) COMPILE  branch <RESOLVE ; IMMEDIATE

: IF ( -- A )   COMPILE ?branch >MARK ; IMMEDIATE

: AHEAD ( -- A ) COMPILE branch >MARK ; IMMEDIATE

: REPEAT ( A a -- ) [COMPILE] AGAIN >RESOLVE ; IMMEDIATE

: THEN ( A -- ) >RESOLVE ; IMMEDIATE

: AFT ( a -- a A ) DROP [COMPILE] AHEAD [COMPILE] BEGIN SWAP ; IMMEDIATE

: ELSE ( A -- A )  [COMPILE] AHEAD SWAP [COMPILE] THEN ; IMMEDIATE

: WHEN ( a A -- a A a ) [COMPILE] IF OVER ; IMMEDIATE

: WHILE ( a -- A a )    [COMPILE] IF SWAP ; IMMEDIATE

: ABORT" ( -- ; <string> ) COMPILE abort" $," ; IMMEDIATE

: $" ( -- ; <string> ) COMPILE $"| $," ; IMMEDIATE

: ." ( -- ; <string> ) COMPILE ."| $," ; IMMEDIATE

2.8.4	Compiler

We had discussed how the compiler compiles commands and structures into the code field of a compound command in the code dictionary.  To build a new command, we have to build its header in the name dictionary also.  A header has a command pointer field, a link field, and a name field.  Here are the tools to build these fields.

?UNIQUE is used to display a warning message to show that the name of a new command is a duplicate to a command already existing in the dictionary.  Forth does not mind your reusing the same name for different commands.  However, giving many commands the same name is a potential cause of problems in maintaining software projects.  It is to be avoided if possible and ?UNIQUE reminds you of it.

$,n builds a new entry in the name dictionary using the name already moved to the bottom of the name dictionary by PACK$.  It pads the command field with the address of the top of code dictionary where the new code is to be built, and link the link field to the current vocabulary.  A new command can now be built in the code dictionary.

$COMPILE builds the body of a new compound command.  A complete compound command also requires a header in the name dictionary, and its code field must start with a CALL doLIST instruction. These extra works are performed by :.  Compound commands are the most prevailing type of commands in Forth.  In addition, Forth has a few other defining commands which create other types of new commands in the dictionary.

OVERT links a new command to the current vocabulary and thus makes it available for dictionary searches.

; terminates a compound command.  It compiles an EXIT to the end of the command list, links this new command to the current vocabulary, and then reactivates the interpreter.

]  turns the interpreter to a compiler.

:   creates a new header and start a new compound command.  It takes the following string in the input stream to be the name of the new compound command, by building a new header with this name in the name dictionary.  It then compiles a CALL doLIST instruction at the beginning of the code field in the code dictionary.  Now, the code dictionary is ready to accept a command list.  ] is now invoked to turn the text interpreter into a compiler, which will compile the following commands in the input stream to a command list in the code dictionary.  The new compound command is terminated by ;, which compiles an EXIT to terminate the command list, and executes [ to turn the compiler back to text interpreter.

call,  compiles the CALL doLIST instruction as the first thing in the code field of a compound command.

IMMEDIATE sets the immediate lexicon bit in the name field of the new command just compiled.  When the compiler encounters a command with this bit set, it will not compile this commands into the command list under construction, but execute the command immediately.  This bit allows structure commands to build special structures in the compound commands, and to process special conditions when the compiler is running.

: ?UNIQUE ( a -- a )

DUP NAME? IF ."  reDef " OVER COUNT TYPE THEN DROP ;

: $,n ( a -- )

DUP C@

IF ?UNIQUE

( na) DUP LAST ! \ for OVERT

( na) HERE ALIGNED SWAP

( cp na) CELL-

( cp la) CURRENT @ @

( cp la na') OVER !

( cp la) CELL- DUP NP ! ( ptr) ! EXIT

THEN $" name" THROW ;

: $COMPILE ( a -- )

NAME? ?DUP

IF @ $80 AND

IF EXECUTE ELSE , THEN EXIT

THEN 'NUMBER @EXECUTE

IF [COMPILE] LITERAL EXIT

THEN THROW ;

: OVERT ( -- ) LAST @ CURRENT @  ! ;

: ; ( -- )

COMPILE EXIT [COMPILE] [ OVERT ; IMMEDIATE

: ] ( -- ) doLIT $COMPILE 'EVAL ! ;

: call, ( xt -- ) \ DTC 8086 relative call

$E890 , HERE CELL+ - , ;

: : ( -- ; <string> ) TOKEN $,n doLIT doLIST  call, ] ;

: IMMEDIATE ( -- ) $80 LAST @ @ OR LAST @ ! ;

2.8.5	Defining Commands

Defining commands are molds which can be used to define many commands which share the same run time execution behavior.  In Forth, we have : , USER, CREATE, and VARIABLE.

USER creates a new user variable.  The user variable contains an user area offset, which is added to the beginning address of the user area and to return the address of the user variable in the user area.  CREATE creates a new array without allocating memory.  Memory is allocated using ALLOT.  VARIABLE creates a new variable, initialized to 0.

Forth does not use CONSTANT, because a integer literal is more economical than a constant.  One can always use a variable for a constant.

: USER ( n -- ; <string> )

TOKEN $,n OVERT

doLIT doLIST COMPILE doUSER , ;

: CREATE ( -- ; <string> )

TOKEN $,n OVERT

doLIT doLIST COMPILE doVAR ;

: VARIABLE ( -- ; <string> ) CREATE 0 , ;

2.9	System Utilities

Forth is a very small system and only a very small set of tools are provided in the system.  Nevertheless, this set of tools is powerful enough to help the user debug new commands he adds to the system.  They are also very interesting programming examples on how to use the commands in Forth to build applications.

Generally, the tools presents the information stored in different parts of the memory in the appropriate format to let the use inspect the results as he executes commands in the Forth system and commands he defined himself.  The tools include memory dump, stack dump, dictionary dump, and a compound command decompiler.

2.9.1	Memory Dump

DUMP dumps u bytes starting at address b to the terminal.  It dumps 16 bytes to a line.  A line begins with the address of the first byte, followed by 16 bytes shown in hex, 3 columns per bytes.  At the end of a line are the 16 bytes shown in characters.  The character display is generated by _TYPE, which substitutes non-printable characters by underscores.  Typing a key on the keyboard halts the display.  Another CR terminates the display.  Any other key resumes the display.

dm+  displays u bytes from b1 in one line.  It leave the address b1+u on the stack for the next dm+ command to use.

_TYPE  is similar to TYPE.  It displays u characters starting from b.  Non-printable characters are replaced by underscores.

: _TYPE ( b u -- )

FOR AFT DUP C@ >CHAR EMIT 1 + THEN NEXT DROP ;

: dm+ ( b u -- b )

OVER 4 U.R SPACE FOR AFT DUP C@ 3 U.R 1 + THEN NEXT ;

: DUMP ( b u -- )

BASE @ >R HEX  16 /

FOR CR 16 2DUP dm+ ROT ROT 2 SPACES _TYPE NUF? NOT WHILE

NEXT ELSE R> DROP THEN DROP  R> BASE ! ;

2.9.2	Stack Dump

Data stack is the working place of the Forth computer.  It is where commands receive their parameters and also where they left their results.  In debugging a newly defined command which uses stack items and which leaves items on the stack, the best was to check its function is to inspect the data stack.  The number output commands may be used for this purpose, but they are destructive.  You print out the number from the stack and it is gone.  To inspect the data stack non-destructively, a special utility command .S is provided in most Forth systems.  It is also implemented in Forth.

.S  dumps the contents of the data stack on the screen in the free format.  The bottom of the stack is aligned to the left margin.  The top item is shown towards the left and followed by the characters '<sp'.  .S does not change the data stack so it can be used to inspect the data stack non-destructively at any time.

.S is useful in checking the stack interactively during the programming and debugging.  It is not appropriate for checking the data stack at the run time.  For run time stack checking, Forth provides !CSP and ?CSP.  They are not used in the Forth system itself, but are very useful for the user in developing serious applications.

To do run time stack checking, at some point the program should execute !CSP to mark the depth of the data stack at that point.  Later, the program would execute ?CSP to see if the stack depth was changed.  Normally, the stack depth should be the same at these two points.  If the stack depth is changed, ?CSP would abort the execution.

!CSP  stores the current data stack pointer into tan user variable CSP.  The stack pointer saved will be used by ?CSP for error checking.

?CSP compares the current stack pointer with that saved in CSP.  If they are different, abort and display the error message 'stack depth'.

: .S ( -- ) CR DEPTH FOR AFT R@ PICK . THEN NEXT ."  <sp" ;

: .BASE ( -- ) BASE @ DECIMAL DUP . BASE  ! ;

: .FREE ( -- ) CP 2@ - U. ;

: !CSP ( -- ) SP@ CSP ! ;

: ?CSP ( -- ) SP@ CSP @ XOR ABORT" stack depth" ;

2.9.3	Dictionary Dump

The Forth dictionary contains all the commands defined in the system, ready for execution and compilation.  WORDS allows you to examine the dictionary and to look for the correct names of commands in case you are not sure of their spellings.  WORDS follows the vocabulary thread in the user variable CONTEXT and displays the names of each entry in the name dictionary.  The vocabulary thread can be traced easily because the link field in the header of a command points to the name field of the previous command.  The link field of the next command is one cell below its name field.

WORDS  displays all the names in the context vocabulary.  The order of commands is reversed from the compiled order.  The last defined commands is shown first.

.ID displays the name of a command, given the word's name field address.  It also replaces non-printable characters in a name by under-scores.

Since the name fields are linked into a list in the name dictionary, it is fairly easy to locate a command by searching its name in the name dictionary.  However, finding the name of a command from the execution address of the command is more difficult, because the execution addresses of commands are not organized in any systematic way.

It is necessary to find the name of a command from its execution address, if we wanted to decompile the contents of a command list in the code dictionary.  This reversed search is accomplished by the command >NAME.

>NAME  finds the name field address of a command from the execution address of the command.  If the command does not exist in the CURRENT vocabulary, it returns a false flag.  It is the mirror image of the command NAME>, which returns the execution address of a command from its name address.  Since the execution address of a command is stored in the command field, two cells below the name, NAME> is trivial.  >NAME is more complicated because the entire name dictionary must be searched to locate the command.   >NAME only  searches the CURRENT vocabulary.

: >NAME ( xt -- na | F )

CURRENT

BEGIN CELL+ @ ?DUP WHILE 2DUP

BEGIN @ DUP WHILE 2DUP NAME> XOR

WHILE CELL-

REPEAT      THEN SWAP DROP ?DUP

UNTIL SWAP DROP SWAP DROP EXIT THEN DROP 0 ;

: .ID ( a -- )

?DUP IF COUNT $01F AND _TYPE EXIT THEN ." {noName}" ;

: WORDS ( -- )

CR  CONTEXT @

BEGIN @ ?DUP

WHILE DUP SPACE .ID CELL-  NUF?

UNTIL DROP THEN ;

2.9.4	Decompiler

SEE searches the dictionary for the next command in the input stream and returns its code field address.  Then it scans the list of execution addresses (commands) in the compound command.  If the command fetched out of the list matches the execution address of a command in the name dictionary, the name will be displayed by the command '.ID'.  If the command does not match any execution address in the name dictionary,  it must be part of a structure and it is displayed by 'U.'.  This way, the decompiler ignores all the data structures and control structures in the compound command, and only displays valid commands in the command list.

: SEE ( -- ; <string> )

' CR CELL+

BEGIN CELL+ DUP @ DUP IF >NAME THEN ?DUP

IF SPACE .ID ELSE DUP @ U. THEN NUF?

UNTIL DROP ;

2.10	Startup

Since we expect Forth to evolve as experience is accumulated with usage, and as it has to track the ANS Forth Standard under development, version control becomes an important issue.  To assure compatibility at different stages of development, the user can always inquire the version number of the Forth he is running.  With the version number, corrective actions can be taken to put an overlay on the system to force it to be compatible with another Forth of a different version.

VER returns the version number of this Forth system.  The version number contains two bytes: the most significant byte is the major revision number, and the least significant byte is the minor release number.

'hi' is he default start-up routine in Forth.  It initializes the serial I/O device and then displays a sign-on message.  This is where the user can customized his application.  From here one can initialize the system to start his customized application.

Because all the system variable in Forth are implemented as user variables and the name dictionary is separated from the code dictionary, Forth dictionary is eminently ROMmable and most suitable for embedded applications.  To be useful as a generic model for many different processors and applications, a flexible mechanism is designed to help booting Forth up in different environments. Before falling into the QUIT loop, the COLD routine executes a boot routine whose code address is stored in 'BOOT.  This code address can be vectored to an application routine which defines the proper behavior of the system.

After the computer is turned on, it executes some native machine code to set up the CPU hardware so that it emulates a Virtual Forth Machine  .  Then it jumps to COLD to initialize the Forth system.  It finally jumps to QUIT which is the operating system in Forth.  COLD and QUIT are the topmost layers of an Forth system.

'BOOT  is an variable vectored to 'hi'.

COLD is a compound command executed upon power-up.  Its most important function is to initialize the user area and execute the boot-up routine vectored through 'BOOT, and then falls into the text interpreter loop QUIT.

: VER ( -- u ) $101 ;

: hi ( -- )

!IO BASE @ HEX \ initialize IO device & sign on

CR ." Forth V" VER <# # # 46 HOLD # #> TYPE

CR ;

: EMPTY ( -- )

FORTH CONTEXT @ DUP CURRENT 2!  6 CP 3 MOVE OVERT ;

CREATE 'BOOT  ' hi , \ application vector

: COLD ( -- )

BEGIN

U0 UP 74 CMOVE

PRESET  'BOOT @EXECUTE

FORTH CONTEXT @ DUP CURRENT 2! OVERT

QUIT

AGAIN ;

DOS starts executing the object code at 100H. The Forth Model is configured for a DOS machine.  It can be modified to jump to any memory location from where the CPU boots up.  What we have to do here is to set up the 8086 CPU so that it will emulate the Virtual Forth Machine  as we discussed before.  All the pertinent registers have to be initialized properly.  Since Forth is very small and fits comfortably in a single 64 Kbyte code segment, we will use only one segment for code, data, as well as the two stacks.  Therefore, both the DS and SS segment registers are initialized to be the same as the CS register.  Then, the data stack pointer SP and the return stack pointer RP (BP in 8086) are initialized.  To prevent the Forth from being forced back into DOS accidentally, the Control-C interrupt is made benign by vectoring it to a simple IRET instruction.

Now we are ready to start the Forth computer.  Simply jumping to COLD will do it.  COLD is coded as a compound command, containing a list of commands.  This command list does more initialization in high level, including initializing the user area, and setting up the terminal input buffer.  At the end, COLD executes QUIT, the text interpreter, which contains an infinite loop to receive commands from a user and executes them repeatedly.

The user area contains vital information for Forth to perform its functions.  It contains important pointers specifying memory areas for various activities, such as the data stack, the return stack, the terminal input buffer, where the code dictionary and the name dictionary end, and the execution addresses of many vectored commands like KEY, EMIT, ECHO, EXPECT,  NUMBER, etc.

The user area must be located in the RAM memory, because the information contained in it are continuously updated when Forth is running.  The default values are stored in the code segment starting at UZERO and covering an area of 74 bytes.  This area is copied to the user area in RAM before starting the Forth computer.  The sequence of data in UZERO must match exactly the sequence of user variables.

;; Main entry points and COLD start data

MAIN	SEGMENT

ASSUME  CS:MAIN,DS:MAIN,ES:MAIN,SS:MAIN

ORG	COLDD			;beginning of cold boot

ORIG:	MOV	AX,CS

MOV	DS,AX		;DS is same as CS

CLI			;disable interrupts, old 808x CPU bug

MOV	SS,AX		;SS is same as CS

MOV	SP,SPP	;initialize SP

STI			;enable interrupts

MOV	BP,RPP	;initialize RP

MOV	AL,023H	;interrupt 23H

MOV	DX,OFFSET CTRLC

MOV	AH,025H	;MS-DOS set interrupt vector

INT	021H

CLD			;direction flag, increment

JMP	COLD		;to high level cold start

CTRLC:IRET			;control C interrupt routine

; COLD start moves the following to USER variables.

; MUST BE IN SAME ORDER AS USER VARIABLES.

$ALIGN			;align to cell boundary

UZERO:	DW	4 DUP (0)	;reserved

DW	SPP		;SP0

DW	RPP		;RP0

DW	QRX		;'?KEY

DW	TXSTO		;'EMIT

DW	ACCEP		;'EXPECT

DW	KTAP		;'TAP

DW	TXSTO		;'ECHO

DW	DOTOK		;'PROMPT

DW	BASEE		;BASE

DW	0		;tmp

DW	0		;SPAN

DW	0		;>IN

DW	0		;#TIB

DW	TIBB		;TIB

DW	0		;CSP

DW	INTER		;'EVAL

DW	NUMBQ		;'NUMBER

DW	0		;HLD

DW	0		;HANDLER

DW	0		;CONTEXT pointer

DW	VOCSS DUP (0)	;vocabulary stack

DW	0		;CURRENT pointer

DW	0		;vocabulary link pointer

DW	CTOP		;CP

DW	NTOP		;NP

DW	LASTN		;LAST

ULAST:

2.11	Conclusion

That’s all for a complete operating system with a high level language.  For an 8-bit microcontroller, the object code is about 6K to 7K bytes long.  With it, you can examine every bit in the memory, exercise all the devices connected to the microcontroller, and write high level programs to extend the functionality of the target system.  It is worth your while to study the source code and obtain the complete understanding of this system.  Once you do understand it, software will cease to be a problem in your career.  You can concentrate in developing hardware to solve problems in the real world.

To remind you the way Forth system works, Figure 5 shows how Forth is booted from COLD and enters into the text interpreter loop.  Figure 6 shows how the interpreter interprets and how the compiler compiles.  These diagram shows you how Forth works and why it is so simple and yet powerful.

Figure 5.	Boot the Forth system

Figure 6.	The interpreter and the compiler

Session 3.  Application Programming in Forth

3.1	eForth as Programming Environment

So far eForth has been used as toys.  Most people used it to learn Forth and did some evaluation exercising new microprocessors or new boards.  It had to be optimized for real work.  Besides, to use eForth to develop substantial applications, it still needs some additional utility words to facilitate the programming tasks.

To run Embedded Forth, one needs a terminal or a terminal emulator on a PC.  Now that Windows 95 is everywhere, we should take advantage of it and use it as the host computer to do application programming.  Hooking the RS232 port of 8051 to COM1 or COM2 port on the PC, run HyperTerminal, a Windows Accessory, and you are ready to exercise Embedded Forth.  Set the HyperTerminal to the terminal mode at 9600 baud, 8 data bits, 1 stop bit, and no parity.  It should say ‘ok’ which 8051 board is powered up.

HyperTerminal is thus the host environment for Embedded Forth.  Normally, it is the terminal to the eForth system, and you can type in Forth commands and execute them.  The text file transferring and text file capturing utilities in HyperTerminal allow us to download source code to eForth and upload eForth memory image to the host.  The source code can be generated and edited using any editor in Windows.  The Windows file manager can be used to maintain the source code and its revisions.  Combining the eForth system, HyperTerminal, and your favorite Windows word processor, you have a complete programming environment, ready to do serious applications for 8051.

These are the few eForth commands you can type into the HyperTerminal to test the eForth system:

WORDS

HEX 0 200 DUMP

SEE WORDS

HERE .

1 2 + .

: TEST 10 FOR R@ . NEXT ;

TEST

I wanted to use this 8051 eForth on the CT100 Laboratory Board as a programming environment, where substantial applications can be programmed, tested, and eventually products are built.  To this end. I added three important words:

READ ( addr len -- )	Download a file and place it in memory starting at addr.  An ESC character signifies the end of the file.  Actual character count is preserved in the user variable CSP.

LOAD ( addr len -- )	Compile the text stream of len bytes from memory location addr.

SEND ( addr len -- )	Upload contents of memory block (addr,len) to the host in the Intel Hex format.

You can write your application using any of the Windows word processors, like Word, WordPad, and Notepad.  Save the source code file in text-only format and it can be downloaded to the eForth system using the command READ:

HEX

A000 2000 LOAD

The memory about A000H is generally not used and it can be used as a buffer for source text.  The length 2000H should be big enough for the entire file.  If you know the exact length of the source code file, use it in place of 2000H.  Now, click on the Send_Text_File command in the Transfer menu of HyperTerminal, and you can select a source file and download it to eForth.  If the actual file length is smaller than 2000H, you have to press ESC to terminate the LOAD command.  LOAD will report the number of bytes actually transferred.  Say, it reports 155A bytes.

Type

A000 155A LOAD

will compile the source code you just downloaded to eForth.  Type:

WORDS

to verify that all the words defined in the source code are compiled correctly.  If you get an error message during compilation, remove the new words you just compiled by typing:

FORGET FIRSTWORD

where FIRSTWARD is the first command in the source code file.  Go back to the word processor, correct the mistake which caused the compilation error, save the text file, and do the READ and LOAD commands again.

After fixing all the compilation errors, you can type in the names of the new commands just compiled and test them.  If you encounter any error during the testing, go back to the source code file and edit out the bugs.

If the dictionary of the eForth system can be saved and placed into an EPROM for execution in the target 8051 system, you can do it following the following steps.  If the eForth system has separated ROM memory and RAM memory, you must burn the source code into an empty space in the EPROM and compile the source code on power-up.

You can dump the memory and save the dump into a text file by first activate the Capture_Text function in the Transfer menu.  Then type:

0 2000 DUMP

to dump 8K bytes of memory.  The dumped data is saved in the capture file, which has to be closed by click on the Stop function when selecting Capture_Text in the Transfer menu.  The memory image can be dumped in to Intel Hex Format by typing:

0 2000 SEND

It is useful if you like to save the memory image to program a new EPROM.

These words are used in conjunction with the Windows utility HyperTerminal and they complete the program development environment and allow real applications and products to be built.  You can use the file system in Windows to manage the source files as professional programmers do.

3.2	The CT100 Laboratory Board

CT100 Lab Board is a single board computer with 8051 and a host of interesting peripheral devices, including discrete LED indicators, switches, numeric 7 segment displays, a 16x16 2-color LED panel display, a 2x40 LCD alphanumeric display, a 54-key keyboard switch array, A/D converter, D/A converter, buzzer, stepper motor drivers, and high load relays.  Forth is installed in ROM and is used to drive all the devices.  It is an excellent tool to teach microprocessor and I/O interfaces.  The Forth originally used is a F83 based implementation on 8051.  It still uses blocks to access source code stored in PC files, and hence requires a PC version of F83 to provide the terminal and file services.  It does not work very well with the Windows operating system.

The new 8051 eForth works smoothly with Windows through the HyperTerminal, and makes the whole system easier to use.  Eliminating the PC F83 saves me lots of grief, because I don’t have to explain how the PC F83 works.  I don’t have to explain how the block editor works.  The user can use any of the word processors in Windows to edit files and to manage the revision of source code.  I only have to explain how 8051 eForth works.

Examine the board carefully, and you will see that the board is divided into regions marked A-T.  Each region contains a group of components for a specific purpose.  Here is a list of these regions and their functions:

A.	Regulated power supply

B.	8031 microcontroller system

F.	Seven segment LED display

G.	Two color 16x16 LED matrix display

H.	Dual row LED indicators

I.	Switches and expansion sockets

J.	Jumpers

K.	8255 peripheral interface

L.	LCD display

M.	Sound generator

N.	Stepper motor driver

O.	ADC converter

P.	DAC converter

Q.	Relay drivers

R.	Keyboard switches

S.	RS232 communication port

T.	PIC microcontroller

Each region contains enough components for at least one experiment.  Each region is isolated from one another.  Signals can be routed from one region to another by inserting the proper connecting cables between  sockets provided in the regions.  There are infinite ways to do interest experiments by combining components in different regions.  The only limitation is the imagination of the user.

3.2.1	Power Up the CT100 Lab Board

Follow the following steps to prepare the CT100 Lab Board for operation:

Attach the 5 Volt power supply plug to the 2-pin connector JPA2 on the upper left corner of CT100 board.  Move SW1 to the left to the On position.

Attach the RS232 serial cable to the 9-pin connector JPS2.  The other end of the serial cable have to be plugged into the COM1 port in the back of the PC.

Turn on PC and wait for the Windows to boot.  After Windows is up, click Start bar, and select Program, Accessories, an then click on HyperTerminal.  When the HyperTerminal is up, click File and select Properties.  In the ‘Connect using’ bar, select ‘Direct to COM 1’ or the appropriate Comm port to the serial port you plugged in. Then press Configure button to bring out the Port Settings window.  Select 9600 baud, 8 data bits, no parity, 1 stop bit, and no flow control.  Click OK button to accept these choices.

Back in the Properties window, select ‘Settings’ tab.  Select ‘Terminal keys’ in the Function block.  In the Emulation bar, select VT100. In the Backscroll buffer lines bar, select 500.  Now Press ASCII Setup button,  in the ASCII Receiving block, select ‘Append linefeed…’ and ‘Wrap lines…’.  Click OK button to accept these choices.

Figure 2	Layout of CT100-A Laboratory Board

Back in the Properties window, click OK to dismiss this window.  Now, close this HyperTerminal Window and save it with a name like ‘51Term’.  Open this HyperTerminal ‘51Term’, and you are ready to talk to CT100 Board.  You have to close the HyperTerminal Window so that the new properties you had chosen will take effect.  It is very important to close and re-open the HyperTerminal after you change some of the settings, especially the baud rate.

Now, plug the power cord of the power supply into a wall socket.  The green LED next to JPA2 connect will turn on, and you will see the sign-on message in the HyperTerminal receiving window:

eForth V2.3

When in Forth, the system does not prompt you as DOS does.  You type a line of commands.  When you hit the <return> key at the end of the command line, Forth executes the commands in sequence.  If it executes all the commands successfully, it will display an 'ok' at the end of line.  If it encounters any command it does not understand, it will display the name of the offending command and a ‘?’, and then waits for you to type another line of commands.  You and Forth interact in this fashion until you are tired of it.

Here is a sample session in which you will create a new file to store your programs to experiment all the devices on CT100-A:

eForth V2.3

<retun> ok			Your press <return> without any command.  Forth says 'ok'.

<return> ok			Same

1 2 + .				Enter 1 and 2, add them, and display the sum.  Forth say 'ok'

1 2 3 4 5				Enter 5 numbers

.S				Display the number on the stack

0 100 DUMP			Dump the memory, 100 bytes from location 0

...				Test other Forth commands

...

WORDS			Dump the commands included in Forth.  Stop scrolling by pressing

<return>.  Hit <return> again stops the display.  Any other key

let the display resume.

Let's see what's in the Forth-51 system.  Type

WORDS

and you will see a list of Forth commands scrolling up on the monitor screen.  These are the commands implemented in the Forth-51 system.  Most of the commands can be executed by typing the name.  However, many of them require input parameters, which are numbers entered before the name of the command.  Without the detailed documentation, it is very difficult to know exactly the number of parameters and their useful ranges.  In the course of the lessons, you will be introduced to many of them.  The command list is fairly long, but most of the commands were there because the system needed them to implement specific functions, and they are very rarely used by us ordinary users.  Here I have a list of the most important and the most useful commands.  If you can master this list of commands, you will be able to use the Forth-51 system to solve most application problems.

Math Commands:		+, -, *, /, MOD, */, /MOD, UM*, UM/MOD

Memory Commands:		@, !, C@, C!, CMOVE, FILL, ERASE

Logic Commands:		AND, OR, XOR, NOT, NEGATE

Stack Commands:		DUP, SWAP, OVER, ROT

Compare Commands:		=, 0=, <, >, 0<, U<

Defining Commands:		:, ;, VARIABLE, CREATE

I/O Commands:			KEY, EMIT, ., U., .R, TYPE, CR, SPACE, SPACES

Structure Commands:		IF, ELSE, THEN, DO, LOOP, BEGIN, WHILE, REPEAT, AGAIN, 						UNTIL

File Command:			OPEN, CLOSE, READ, WRITE, LOAD, SEND

Look up these commands in the eForth Quick Reference Card in the Appendix to see how they are used.  Exercise them following the F83 tutorials included in the disk file.  Read the example code.  There are less than 100 concepts you have to master, then you can use Forth to solve most problems.

3.2.2	Exercise the CT100 System

We assume that you have some elementary knowledge on the 8051/8031 family of microcontrollers.  Intel had published very extensive data books and many people had written about these microcontrollers.  You need one of these books and read through it.  Depending on you back ground, you may or may not understand everything in the book.  However, this does not means that you cannot learn 8051 using this CT-100A system.  In fact, there are only a very small number of concepts necessary to start using CT-100A, and these are subjects of this chapter.

Figure 6.   8051 Operating Configuration

As show in the above diagram,  we can divide the CT-100 Lab system into these functional blocks.  You have a personal computer hooked to CT-100A Lab Board.  On PC, a Forth system F83 is running and it talks to the Forth program FORTH-51 in the 8031 system through a RS232 serial cable..  8031 controls a pair of 8255 peripheral I/O chips, and the I/O pins on 8255's are used to control the experiments on the CT-100A board.

The 8255's are implemented as memory to 8031.  Therefore, to command an 8255 to perform a task, we simply send commands from PC to 8031 to read and write the memory locations assigned to the 8255.  Here is the memory map of the 8255's:

Address		Function

F800		Port A data register of the first 8255

F801		Port B data register of the first 8255

F802		Port C data register of the first 8255

F803		Control register of the first 8255

FA00		Port A data register of the second 8255

FA01		Port B data register of the second 8255

FA02		Port C data register of the second 8255

FA03		Control register of the second 8255

To do most of the experiments on CT-100A, all you have to do is to connect the experiment components to the 8255's, and then issue the proper read/write commands to the 8031.  The procedures are almost identical to those we discussed in the last chapter using the PC to exercise the LED's.  However, since the 8255's are arranged as memory in the 8031 system, while the 8255 is configured as an I/O device to the PC, the commands used in FORTH-51 to exercise 8255 are slightly different.  For examples, to initialize the first 8255 for all output operation:

HEX

80 F803 C!

To turn the LED's on and off:

0 F800 C!

FF F800 C!

C! is a command similar to PC!.  Instead of storing an 8-bit pattern into an I/O device, C! stores the pattern into a memory location.  If the memory location is a register in 8255, you cause the 8255 to do something.  Thus you can do all the experiments we did in the last chapter through 8031 instead of through the PC.

3.2.3	Using the 8255 Parallel Interface Chip

8255 is a general purpose programmable I/O device designed for use with Intel microprocessors and microcontrollers.  It has 24 I/O lines which may be individually programmed in 2 groups of 12 lines, and used in 3 major modes of operation.  In the first mode (Mode 0), each group of 12 I/O pins may be programmed in sets of 4 to be input or output.  In Mode 1, the second mode, each group ,may be programmed to have 8 lines of input or output.  Of the remaining 4 pins, 3 are used for handshaking and interrupt control signs.  The third mode of operation (Mode 2) is a bi-directional bus mode which uses 8 lines for a bi-directional bus, and 5 lines, borrowing one from the other group, for handshaking.

A detailed specification of 8255 is included as Appendix B in this manual for your reference.  However, we will not try to present all the operations of 8255, but only to help you learn the most fundamental operations required in the experiments.  You can learn more about 8255 and explore its applications on your own, once you master the basic operations.

Through 8031, two 8255's  can be accessed simultaneously.  The registers in 8255's are addressed as memory.  The locations and the functions of these registers can be summarized as follows:

Address (Hex)		Register	Functions

F800			Port A	I/O port of the first 8255

F801			Port B	I/O port of the first 8255

F802			Port C	I/O port of the first 8255

F803			Control	Status and control port of the first 8255

FA00			Port A	I/O port of the second 8255

FA01			Port B	I/O port of the second 8255

FA02			Port C	I/O port of the second 8255

FA03			Control	Status and control port of the second 8255

Note that the second 8255 is shared between 8031 and the PC parallel port.  To use the second 8255 through 8031, you have to jumper the CS and 55CE2 pins on socket JPK4.

8255 has to be initialized before using.  The initialization defines which pins are input pins and which are output pins, and how they are to be used.  Initialization is to write a Mode Definition patterns into the Control Register at location FA03H.  Let us not worry about all the possibilities out input-output configurations, but concentrate on the simplest configurations we will use.  The two most import operation configurations are Mode 0 all input and all output, and they are defined by the following commands:

HEX			Always use hexadecimal base for numbers

80 F803 C!		Set up first 8255 so that all 24 I/O pins are output pins

9B F803 C!		Setup first 8255 so that all 24 I/O pins are input pins

89 F803 C!		Setup first 8255.  Ports A and B are output and Port C is input.

80 FA03 C!		Set up second 8255 so that all 24 I/O pins are output pins

9B FA03 C!		Setup second 8255 so that all 24 I/O pins are input pins

C! is the Forth-51 command to store an 8 bit pattern into a memory location.  Here, if we store an 80H into the control register a F803H, all the I/O pins in the first 8255 will be used as output pins.  If we store a 9BH into F803H, all the I/O pins in the first 8255 will be used as input pins.  Once initialized, 8255 will stay in the selected operation configuration until a new mode definition pattern is written into the control register.

3.3	Programming Examples for CT100 Lab Board

The following experiments on the CT100 Laboratory Board are converted to a text file DEMO51.TXT, which can be downloaded to CT100 with the READ command, compiled by the LOAD command, and then executed to run various devices on CT00:

3.3.1	Discrete LED indicators

In this experiment, we will use the second 8255 in the K region to control the red and green LED's in the N region.  Use an 8-wire cable to connect  PA0-PA7 on JPK2 in the K Region to A1-A8 on JPN2 socket in the N region. Use a second 8-wire cable to connect PB0-PB7 on JPK2 to B1-B8 on JPN2.  These cabled thus allow the Port A and Port B of the second 8255 chip to drive 8 red LED's and 8 green LED's in the N region.  If you had done the experiments in chapter 2, the cables were installed properly already.  If you choose to do the experiments using the first 8255, the cables plugged into JPK2 must be moved to socket JPB7.  The following examples all use the second 8255.  You have to change the addresses properly to use the first 8255.

The Following sequence of commands turn the 8 red LED lights on and off.

HEX

80 FA03 C!

FF FA00 C!

0 FA00 C!

Memory location FA00H controls the output pins in Port A (PA0-PA7).  After 8255 is initialized by the commands  80 FA00 C!, storing 8 bit patterns into FA00H will produce effects on the 8 red LED's.  Writing a FF into FA00H will turn all the LED's off.  Writing a 0 into FA00 will turn all the LED's on.  Writing other patterns into FA00H and observe the effects.  For example:

1 FA00 C!

2 FA00 C!

AB FA00 C!

FE FA00 C!

The output pins PB0-PB7 of Port B (location FA01H) are connected to the 8 green LED's through the pins B1-B8 in the socket JPN2.  Any pattern you write into register FA01H will be reflected in the green LED's. For example:

FF FA10 C!

0 FA01 C!

1 FA01 C!

2 FA01 C!

CD FA01 C!

...

Here is a nice exercise for you.  We want to turn the red LED’s on sequentially from left to right, 1 second for each LED.  Then we want to turn the green LED’s on sequentially from right to left, 1 second for each.  Repeat the about sequences until we press a key on the terminal.

This calls for writing programs.  Here is the program.  Type them in, one line at a time, followed by a <return>.

HEX

: INITABCOUT 80 F803 C! ;

: REDLED ( B -- )   FF XOR F800 C! ;

: GREENLED ( B -- )  FF XOR F801 C! ;

DECIMAL

: MS ( N -- ) FOR 10 FOR NEXT NEXT ;

: REDSEQ 1 7 FOR DUP REDLED 1000 MS CELLS NEXT REDLED ;

: GREENSEQ 128 7 FOR DUP GREENLED 1000 MS 2/ NEXT GREENLED ;

: CIRCULATE INITABCOUT

BEGIN REDSEQ GREENSEQ ?KEY UNTIL ;

If all the lines are typed in correctly, we will have a set of new Forth commands:  INITABCOUT, REDLED, GREENLED, MS, REDSEQ, GREENSEQ, and finally, CIRCULATE.  Now, type

CIRCULATE

and you will see the LED’s lit up one by one.  When you are tired of looking at it, press a key on the terminal and after the last green LED goes off, an ‘ok’ will appear on the terminal, and eForth is ready for your next command.

If you mis-type any commands in a line, eForth will stop and flag you the mis-typed command.  Do not be discouraged.  Type the whole line again.

3.3.2	Switch Inputs

Take another 8-wire cable and jumper PC0-PC7 on JPK3 socket to JPI1.  This cable connects the 8 switches in Region I to Port C in Region K.  As we are using F83 in PC to do the experiments, make sure that the jumper on JPK4 connects pin CS to pin 88CS, enabling the PC to control the 8255.

Previously, the 8255 was initialized so that all three ports are output ports.  Now we have to initialize it and make Port C an input port.  The commands to select this Mode 0 operation are:

HEX

89 F803 PC!

Now, when we want to read the panel switches, type the following commands:

F802 PC@ .

302 is the I/O address of Port C.  PC@ is the F83 command to read an input port.  The period '.' after PC@ is the F83 command to display the results just read from Port C.  The result is also displayed in hexadecimal.

Now, change the switch patterns  and confirm that you can read the patterns.  To make it easier on yourself, define a new command:

: ii	F802  PC@  .  ;

You only have to type 'ii <return>' to read the switches and display its setting.  Any sequence of commands used more than 3 times is worthy of a new command.  Forth encourages you to define many small and useful commands.  Eventually, you build up your own language to solve your own problems in the most efficient ways.

The Forth commands to select this Mode 0 operation for the second 8255 are:

HEX

: initABout  89 F803 C! ;

initABout

: PAout ( b -- ) F800 C! ;

: PBout ( b -- ) F801 C! ;

: PCin  ( -- b ) F802 C@ FF XOR ;

Now, when we want to read the panel switches, type the following commands:

PCin .

FA02 is the memory address of Port C of the second 8255, in the memory map of 8031.  C@ is the Forth-51 command to read a memory location.  The period '.' after PCin is the Forth command to display the results just read from Port C.  The result is also displayed in hexadecimal.

Now, change the switch patterns  and confirm that you can read the patterns.  To make it easier on yourself, define a new command:

: ii PCin . ;

You only have to type 'ii <return>' to read the switches and display its setting.  Any sequence of commands used more than 3 times is worthy of a new command.  Forth encourages you to define many small and useful commands.  Eventually, you build up your own language to solve your own problems in the most efficient way.

3.3.3	Using Switches to Control LED's

Now, we can use the switches to control the LED's, which we learned to use in Chapters 2 and 3.  The first experiment is easy.  Just turn an LED on when the corresponding switch is set in the on position.  We can read all 8 switches in one operation and then set all the LED's in the next operation:

FA02 C@   FA00 C!

We can use the complemented switch pattern to lit the second set of LED's:

FA02 C@   DUP  FA00 C!   COM  FA01  C!

After reading the switch pattern from Port C, the result is duplicated by DUP, one copy is written to FA00 to lit the LED's through Port A.  The second copy of the results is complemented and written into FA01 to lit the LED's through Port B.

It is tedious to type these commands over and over in order to change the LED pattern.  We can now write a program which continuously monitors the switch pattern and lit the LED accordingly:

: ReadWrite   PCin  dup PAout  ff xor PBout ;

: ReadWriteTest   begin ReadWrite key? until ;

Type 'ReadWriteTest <return>' and you can start flipping the switches.  As the switch pattern is changed, the LED's will immediately show the new pattern.  You can terminate the program by pressing a key on the keyboard.  KEY? is the F83 command to monitor the keyboard.  If any key is pressed, KEY? will return a true flag, which causes UNTIL to terminal the loop.  If no key is pressed, KEY? will return a false flag and UNTIL will jump to repeat the commands between BEGIN and UNTIL.

HEX

: PAOUT ( B -- )   F800 C! ;

: PBOUT ( B -- )   F801 C! ;

: PCOUT ( B -- )   F802 C! ;

CR .( Digital input )

HEX

: INITABOUT  89 F803 C! ;

: PCIN  ( -- B ) F802 C@ FF XOR ;

: READWRITE   PCIN  DUP PAOUT  FF XOR PBOUT ;

: READWRITETEST

INITABOUT

BEGIN READWRITE ?KEY UNTIL ;

CR .( Traffic control )

VARIABLE (EW) VARIABLE (NS)

: EW (EW) @EXECUTE ;

: NS (NS) @EXECUTE ;

DECIMAL

: SECONDS ( N -- ) 5000 * FOR NEXT ?KEY IF ABORT THEN ;

HEX

: EWGREEN   5 GREENLED 50 REDLED 10 SECONDS ;

: EWYELLOW  0 GREENLED 5A REDLED  2 SECONDS ;

: EWLEFT	A GREENLED 50 REDLED  5 SECONDS ;

: NSGREEN  50 GREENLED  5 REDLED 10 SECONDS ;

: NSYELLOW  0 GREENLED A5 REDLED  2 SECONDS ;

: NSLEFT   A0 GREENLED  5 REDLED  5 SECONDS ;

: SWITCH ( MASK -- SWITCHES ) PCIN AND ;

: [EW] R> R> 2DROP

EWGREEN 50 SWITCH IF EWYELLOW NSLEFT NS THEN

5 SWITCH IF EWYELLOW EWLEFT EW THEN EW ;

: [NS] R> R> 2DROP

NSGREEN 5 SWITCH IF NSYELLOW EWLEFT EW THEN

50 SWITCH IF NSYELLOW NSLEFT NS THEN NS ;

' [EW] (EW) !   ' [NS] (NS) !

3.3.4	Buzzer

Connect either pin in socket JPM1 to PA0 of the first 8255 on JPB7. You are ready to test the buzzer.

VARIABLE WAITCOUNTS 5 WAITCOUNTS !

VARIABLE PERIODS  1000 PERIODS !

HEX

: WAIT WAITCOUNTS @ FOR NEXT ;

: BUZZ PERIODS @ FOR 1 F800 C! WAIT 0 F800 C! WAIT NEXT ;

: RUNBUZZ BEGIN BUZZ ?KEY UNTIL ;

The variable waitCounts determines the pitch of the tone, and periods determines how low this tone is to be played out.  Store different values into these two variables and observe the different sound thus generated. An interest experiment is to emulate a siren in which the pitch is increasing to a limit and then starts from a low pitch again.

DECIMAL

: SIREN-

22 20 FOR 300 OVER / PERIODS ! DUP WAITCOUNTS ! BUZZ

1- NEXT DROP ;

: SIREN+

1 20 FOR 300 OVER / PERIODS ! DUP WAITCOUNTS ! BUZZ

1+ NEXT DROP ;

: RUNSIREN BEGIN SIREN+ SIREN- ?KEY UNTIL ;

3.3.5	7 Segment LED numeric display

In Region F, there are eight 7 segment LED display chips, which can display 8 numeric digits.  These numeric displays are very useful to show numeric information in embedded systems.  In many instruments, they are the only way the instrument informs the user what is going on in it.  Learning how to use these display units is a very important exercise of a digital engineer.

Assuming that we are controlling the LED's through 8051 and the first 8255 in Region B.  We will use Port A of 8255 to drive the LED driver circuitry.  Connect PA0-PA7 on socket JPB7 to 0-7 pins on socket JPF1.  Make sure that PA0 is aligned to JPF0.

Power up PC and CT-100A.  Load F51.EXE.  Press F51 to enter F51-Forth.  You have to initialize the first 8255 by:

HEX

80 F803 C!

Now you can send command to the 7 segment LED display:

1 F803 C!			Display a '1' on the rightmost LED

6 F803 C!			Display a '6' on the rightmost LED

27 F803 C!			Display a '7' on the third LED from right

78 F803 C!			Display an '8' on the leftmost LED

In this experiment you will define a command which increments an 8-digit number shown on the 7 segment LED's continuously.  To show a number continuously, we have to store the digits in a memory array and continuously fetching these digits and feed them to the LED display.

HEX

: SHOW12345678

BEGIN 71 PAOUT 62 PAOUT 53 PAOUT 44 PAOUT

35 PAOUT 26 PAOUT 17 PAOUT 8 PAOUT

?KEY UNTIL ;

CREATE DIGITBUFFER 8 ALLOT

: SHOWBUFFER 7 FOR DIGITBUFFER R@ + C@ PAOUT NEXT ;

: RUNCOUNTER

9 FOR R@ F0 + DIGITBUFFER 7 + C!

9 FOR R@ E0 + DIGITBUFFER 6 + C!

9 FOR R@ D0 + DIGITBUFFER 5 + C!

9 FOR R@ C0 + DIGITBUFFER 4 + C!

9 FOR R@ B0 + DIGITBUFFER 3 + C!

9 FOR R@ A0 + DIGITBUFFER 2 + C!

9 FOR R@ 90 + DIGITBUFFER 1 + C!

9 FOR R@ 80 + DIGITBUFFER C!

SHOWBUFFER ?KEY IF ABORT THEN

NEXT NEXT NEXT NEXT NEXT NEXT NEXT NEXT ;

3.3.6	Stepper motor drive

The stepper motor usually has 6 leads, and they must be wire to JPN2.  The coils in the stepper motor are arranged in two groups, each with 3 leads.  The center tap should be connected to Vcc power, the two side taps should be connected to A and /A (or B and /B) pins on JNP2.  You can use an Ohm-meter to determine which lead is which.  To drive the stepper motor and make it turn in one direction, the sequence of power pulse is A, B, /A, /B, A, B, ...  To drive it in the other direction, the sequence of power pulses is A, /B, /A, B, A, /B...

Let's connect  the A, /A, B, and /B pins on JPN1 to PA0-PA3 on JPB7, so that we will drive the stepper motors through Port A of the first 8255.

CR .( Stepper motor )

HEX VARIABLE DELAYCOUNT 20 DELAYCOUNT !

: PULSE ( B -- ) 0FF XOR F800 C! DELAYCOUNT @ FOR NEXT ;

: FORWARD 1 PULSE 4 PULSE 2 PULSE 8 PULSE ;

: BACKWARD 1 PULSE 8 PULSE 2 PULSE 4 PULSE ;

: RUNFORWARD BEGIN FORWARD ?KEY UNTIL ;

: RUNBACKWARD BEGIN BACKWARD ?KEY UNTIL ;

: FORERAMPUP 9 FOR FORWARD -1 DELAYCOUNT +! NEXT ;

: FORERAMPDOWN 9 FOR FORWARD 1 DELAYCOUNT +! NEXT ;

: BACKRAMPUP 9 FOR BACKWARD -1 DELAYCOUNT +! NEXT ;

: BACKRAMPDOWN 9 FOR BACKWARD 1 DELAYCOUNT +! NEXT ;

: FASTFORWARD FORERAMPUP RUNFORWARD FORERAMPDOWN ;

: FASTBACKWARD BACKRAMPUP RUNBACKWARD BACKRAMPDOWN ;

3.3.7	Two-color LED array

This LED array is a very interest device.  It contains 256 dual color LED's arranged in a 16x16 array.  Each LED can show red, green, and orange color.  It is a component for many interesting commercial displays we see in departmental stores and in busy city streets.  It is large enough to show simple graphics and even Chinese characters, as this CAT system was originally developed in Taiwan for technical training.

Connect PA0-7 of JPB7 to D0-7 of JPG1.  Connect PB0-3 of JPB7 to 1,2,4,8 of JPG2.  Connect PC0-3 of JPB8 to A,B,C,D of JPG2.  We will use the Port A of 8255 to supply data to the four 74LS373 latches, Port B to enable these latches, and Port C to scan the LED array.

HEX

: INIT-PANEL 80 F803 C! FF F800 C! 0 F801 C! FF F801 C! ;

: PAINT ( ADDR -- , SHOW A BLOCK OF 64 BYTE DATA )

0F FOR R@ F802 C!

COUNT F800 C! 1 F801 C! 0 F801 C!

COUNT F800 C! 2 F801 C! 0 F801 C!

COUNT F800 C! 4 F801 C! 0 F801 C!

COUNT F800 C! 8 F801 C! 0 F801 C!

NEXT DROP ?KEY IF ABORT THEN ;

: PP FOR DUP 0F FOR DUP PAINT NEXT DROP 4 + NEXT DROP ;

: LEDDEMO 0 200 PP ;

: SHOWPAD BEGIN PAD PAINT ?KEY UNTIL ;

: GREEN FFFF PAD ! 0 PAD CELL+ ! PAD PAD 4 + 3E CMOVE SHOWPAD ;

: RED   0 PAD ! FFFF PAD CELL+ ! PAD PAD 4 + 3E CMOVE SHOWPAD ;

3.3.8	LCD display

The LCD module contains a controller which provides a parallel interface to microprocessors and microcontrollers.  The interface consists of a bi-directional 8-bit data bus and 3 control lines: E for enabling, R/W for read/write direction, and RS for register selection.

Connect PA0-7 of JPB7 to D0-7 of JPL1.  Connect PC0 to E, PC1 to R/W, and PC2 to RS.  PC0-2 are on JPB8 and E. R/W and RS are on JPL1.  On CAT-100A board, the labels E and RS are reversed due to a silk screen error.  The pin order is correct as show in the schematics in Diagram L.

HEX ( PA0-7=D0-7, PC0=E, PC1=R/W, PC2=RS )

: IR F800 C! 0 F802 C! 1 F802 C! 0 F802 C! ;

: DR F800 C! 4 F802 C! 5 F802 C! 4 F802 C! ;

: INITLCD 80 F803 C! 3C IR 1 IR F IR ;

: LCDTYPE FOR COUNT DR NEXT DROP ;

: CLEAR 1 IR ;

: HOME 2 IR ;

: LOWER C0 IR ;

: RSHIFT 18 IR ;

: LSHIFT 1C IR ;

: TT  INITLCD CLEAR HOME

$" LCD DEMO" COUNT LCDTYPE

LOWER

$" BY C. H. TING" COUNT LCDTYPE ;

3.3.9	Keyboard controller

8279 is one of the many complicated peripheral interface chips produced by Intel.  These peripheral chips turn hardware problems into software problems.  8279 has two distinct parts: a keyboard portion which provides a scanned interface to a 64-contact key matrix, and a display portion which provides a scanned display interface to LED and similar numeric display devices.  There is an 8-character FIFO buffer to store keyboard input, which can be debounced and programmed for 2-key lockout or N-key rollover.  There is another 16 byte display RAM where up to 16 characters can be stored and to be scanned out to LED display.

On CT-100A, the 8279 is connected to the 8051 as an output device, similar to the first 8255.  Therefore, it can only be controlled through 8051.  You cannot access the 8279 from PC through the CT-100B interface.

To the 8051 microcontroller, this 8279 appears as 4 registers mapped to the memory locations FC00 and FC01.  The functions of these registers are as follows:

Location		Action		Function

FC00		Read		Input data register

FC00		Write		Output data register

FC01		Read		Status register

FC01		Write		Command register

8279 accepts 8 types of commands:

Command Code (binary)		Function

000DDKKK			Mode set, DD selects display mode, KK selects keyboard mode

001PPPPP			Program clock, PPPPP selects clock prescaler.

010AIX AAA			Read FIFO/RAM, AI autoincrement, AAA address

011AIAAAA			Read display RAM, AI auto-increment, AAAA address

100AIAAAA			Write display RAM, AI auto-increment, AAAAA address

101XWWBB			Display write inhibit/blanking, WW inhibit, BB blanking

110CCCFA			Clear, CCC (blanking code, F clear FIFO, A clear all

111EXXXX			End interrupt/Error mode set

Among all the varieties in these commands, let me select and explain only those which we will use:

Command Code (hex)		Function

0				8 byte character display, left entry

8				16 byte character display, left entry

10				8 byte character display, right entry

18				16 bytes character display, right entry

3F				Program clock with 31 prescaler,  30KHz sampling

50				Read FIFO location 0, auto-increment

90				Write display memory 0, auto-increment

DF				Clear all memory

HEX

: STATUS ( -- B ) FC01 C@ ;

: COMMAND ( B -- ) FC01 C! ;

: DATA   ( -- B ) FC00 C@ ;

: WRITE   ( B -- ) FC00 C! ;

: INIT8279  18 COMMAND 3F COMMAND 50 COMMAND ;

: KEYTEST INIT8279

BEGIN STATUS 7 AND IF DATA . THEN ?KEY UNTIL ;

: MAKEDIGIT ( KEY -- DIGIT )

DUP 8 / 7 AND

SWAP 4 AND IF 7 XOR 5 + ELSE 1 - 0 MAX THEN ;

: DIGITTEST INIT8279

BEGIN STATUS 7 AND IF DATA MAKEDIGIT . THEN ?KEY UNTIL ;

: 7SEGLEFT 0 COMMAND DF COMMAND 90 COMMAND ;

: 7SEGRIGHT 10 COMMAND DF COMMAND 90 COMMAND ;

: 7SEGTEST 9 FOR R@ WRITE 4000 FOR NEXT NEXT ;

3.3.10	A/D and D/A converters

We will use Port A of the first 8255 to drive the DAC0808.  Connect PA0-7 of JPB7 to B1-B8 of JPP1 using an 8-wire jumper cable.  Note that because B1 is the most significant bit to DAC0808, it must be aligned to PA7 and B8 must be aligned to PA0.

To do experiments with DAC and ADC, you have to have a separate power supply to supply +12V and -12V to the circuitry in Region O and Region P.  The ground connection of the this power supply must be tied to the common ground of CA100-A.  Carefully connect the +12V lead to Pin 12 of JPP1 and the -12V lead to Pin 1 of JPP1.

The output voltage from DAC0808 is buffered by an operational amplifier MC4558C.  The buffered output from MC4558C is available at Pin 2 of JPP1.  To measure the output from the DAC, connect Pin-2 of JPP1 to a voltmeter or to a oscilloscope.

Initialize the 8255 and you are ready to run the DAC.

HEX

: initABCout  80 F803 C! ;

initABCout

: PAout  F800 C! ;

: DAC INITABCOUT

BEGIN 1000 FOR R@ F800 C! NEXT

?KEY UNTIL ;

We will use Port A of the first 8255 to read digital data from ADC0804, and the lowest two bits of Port B to control ADC0804.  Later, we will use Port C to display the results of the conversions on the 7-segment display.  8255 has to be initialized accordingly.

Connect PA0-7 of JPB7 to D0-7 of JPO1 using an 8-wire jumper cable, and connect PB0-1 to Pins RD and WR of JPO1 using a 2-wire jumper cable.

HEX ( PA0-7=D0-7, PB0=RD, PB1=WR, PC0-7 TO 7-SEG LED )

: INITADC 90 F803 C! ;

: PAIN ( -- B ) F800 C@ ;

: DELAY 20 0 FOR NEXT ;

: ADC ( -- B ) INITADC

1 PBOUT 3 PBOUT DELAY 2 PBOUT PAIN 3 PBOUT ;

DECIMAL

: SHOWDIGITS ( N -- )

2 * FOR DUP

10 /MOD SWAP 128 + PCOUT DELAY

10 /MOD SWAP 144 + PCOUT

DELAY 32 + PCOUT DELAY

NEXT DROP ;

: VOLTMETER   INITADC

BEGIN ADC SHOWDIGITS ?KEY UNTIL ;

3.4	8051 Assembler

The optimized eForth system is fast enough for most real-time applications.  However, it is possible that you have to do your own optimization on a set of application words to make your application run as fast as it can.  Once you had identified this set of words, you can use the 8051 assembler, perhaps the MetaLink 8051assembler, to write code words and assemble them with the eForth system.  It is also possible to have a version of the 8051assembler loaded on the top of eForth, so that these critical routines can be coded and tested interactively.

To access 8051 and the internal devices built in 8051, one has to be able to program in the 8051 assembly.  An 8051 assembler is constructed as a application for CT100, which can be downloaded to write 8051 assembly code.

The 8051 assembly mnemonic is postfix to facilitate the assembly process.  The general rules in naming the assembly instructions are:

- All one-byte instructions are given unique names.

- Instructions using R0 and R1 to do indirect addressing are given unique names.

- Instructions involving register R0-R7 use one byte on the stack to fill the 3-bit register field.

- Immediate byte data, internal memory and Special Function Registers use one byte on the stack to fill the 8-bit data/address field.

- Memory addressing instructions use a 16-bit address to fill the 16- or 11-bit address field.

- Branch instructions use a 16-bit address on the stack to assemble an 8-bit offset in the offset field.

Table 3 shows the eForth 8051 assembler mnemonic with the corresponding Intel Mnemonic. Adopting this set of mnemonic, it is very easy to write the 8051 assembler.  The source code of the assembler is showing in Listing 2.

As this 8051 eForth uses the subroutine thread model, the word CREATE builds a header and immediately adds a CALL doVAR instruction.  A CODE command does not need this three-byte call.  Only the header is needed.  Thus CODE is defined using CREATE -3 ALLOT.

The much more interesting word is DOES>, and its command shown in Listing 2 is deceptively simple.  Again it is due to the subroutine thread model.  For all the 8051 assembly instructions, the one-byte machine code is stored in the one-byte space immediately after CREATE.  To force the assembly word to execute the code sequence after DOES>, DOES> simply replaces the address of doVAR by the address after DOES>.  Since 8051’s calling addresses are in the big-endian order, the address after DOES> must be FLIPped and then used to replace the address in CALL doVAR. That’s all it has to do.

(Many Forth programmers complained to me that eForth could not be a true Forth because it did not have DOES>.  I did not include DOES> in the eForth model because you have to write a long thesis to explain clearly what DOES> does.  It only distract the first time Forth users from the simple architecture of a Forth machine.  It is very easy to add to the eForth system when it is needed.  However, please be reminded that this implementation is only useful in this particular situation, where the data stored in the parameter field is only one-byte long.  A general purpose DOES> can also be implemented easily, but it is left as an exercise to the students.)

The very detailed discussions on the 8051 assembler is outside of the scope of this workshop.  If you are interested, please see the files 51ASM.TXT and KERNEL.TXT for the implementation of this 8051 assembler and the test code.

Session 4.  Firmware Production

4.1	Software Production with 8086 eForth

When we talk about software production, we always assume that there is a operating computer, and the software we build will be running on this computer.  What does an ‘operating computer’ mean?  It means that a computer with a running operating system.  The operating system provides the environment in which we can write application programs, and run the application programs.  Then the question is what does this environment or the operating system do for us?

The operating system does a lot.  Here is a list of things that the operating system provides for the programmers and the software engineers:

Command line interpreter

Machine code assembler

High level language compilers

BIOS, basic input/output services

Drivers for all peripheral devices

File management system

Library of utility routines

Linker of library routines

Source code editor

Debugger

To develop software for an application, the programmer/software engineer must go through the following process.  The most import thing to do at the beginning is the software design.  Here you analyze the application and conceptualize the software architecture, which means to partition to application into modules which can be constructed and tested independent of one another.  The modules are further partitioned into smaller modules.  A hierachical structure is thus established and this is the software architecture.  Modules are partitioned to the point that they can be expressed by the programming language.  At this point, the programmer starts writing program.

Programs are partitioned into small modules.  The modules are often programmed in the form of subroutines and functions.  These subroutines and functions are collected into one or more libraries.  Together with the libraries provided by the programming language and by the operating system, modules at a higher level can be written.  The application program calls the modules and the library routines.  When the application program is completed, it is compiled and linked with all the library routines it needs to form an machine code binary executable file.  This file can be executed and tested through the command line interpreter in the operating system.

Software production therefore depends heavily on the operating system, and the operating system depends on the underlying computer to perform all the operations.  Over the half century of the computer industry, there have been many different computers, many different operating systems, and many different programming languages.  Consequently, software engineering has become a very diversified and fragmented field.  Fortunately, and unfortunately, this field in recent years has seen substantial consolidation, in that only a few computer architectures dominate the computer hardware, the C programming language eases out all the other programming languages, and the only significant operating systems are UNIX and Windows.

The consolidation is good for software engineers, because they do not have to deal with the almost infinite number of combinations of CPU’s, operating systems, and programming languages.  On the other hand, it is also bad because the surviving CPU’s, operating systems, and programming languages are not necessarily the optimize set for applications.  We have to use them because we do not have choice.

4.1.1	Assemble  8086 eForth

There are two versions of 8086 eForth in the software disk.  The 86EFORTH.ASM is the original reference implementation which defined the eForth model from which all other implementations were derived.  This model was discussed in Session 2.  The other version 86EF202.ASM is a implementation specifically designed for this Firmware Workshop.  In this version, many of the features which are not useful in an embedded system were deleted, and a set of new commands were added so that applications can be loaded from DOS text files, and the code image can be saved to an .EXE file.  The source code of both versions are included in the distribution disk so you can study them and maybe modify and enhance them.

86EF202.EXE is a Forth program running under DOS, specifically designed so that it can be a foundation to build DOS applications.  Its source code is in the file 86EF202.ASM, to be assembled by Microsoft MASM Version 5.0 or above.  To assemble the source code to produce the executable 86EF202.EXE, call the assembler and the linker with the following commands:

C>MASM 86EF202,,,,

C>LINK 86EF202,,,,

C> 86EF202

And Forth will be booted and display the sign-on message:

eForth v2.2

You can edit the 86EF202.ASM file to add new commands or to modify existing commands if you are used to program in 8086 assembly.  However, it is difficult to work with the machine code in assembler.  It is much easier and more productive to program the computer in high level.  The Forth system is useful in that you can write the application programs in high level Forth language and debug the programs faster and easier.

4.1.2	Extend the Forth System

After loading 86EF202 in DOS, you can now type in Forth commands to test Forth, or you can compile application source code stored in text files.  The examples discussed in this workshop are listed in the file LESSONS.TXT.  You can use the DOS editor EDIT to look and edit this file, or use any text editor in Windows like Word, WordPad or NotePad to inspect it and edit it.  Remember to save the file in pure text form so that it can be compiled by Forth.  The commands to compile the examples in LESSONS.TXT and build an application which invokes the last example in LESSONS.TXT, the ‘Guess a Number’ game:

C>86EF202

DOWNLOAD LESSONS.TXT

‘ GUESS ‘LOAD !

UPLOAD TEST.EXE

BYE

C>TEST

Forth will be booted and the game ‘Guess a Number’ will be executed with the following message:

GUESS A NUMBER

…

…

You can play this game with the computer.  When you are tire of playing the game and select to end the game, you will return to Forth.

Above is a complete sequence to build an application in Forth under DOS.  It demonstrates to you that building new applications is a fairly straightforward process with the tools provided in Forth.  This is because the Forth system is saved with the application.  Everything needed by the application is saved in the execution EXE file.  You only have to make sure that the application you loaded on the top of the Forth system works properly.  The application you saved in the EXE file will work exactly the same way.  You don’t have to worry about the DOS operating system and all the complications associated with DOS.  Forth greatly simplifies the process of building application software.  It is the best tool for software engineering.

There are a set of new commands used in building the application program.  Most of them invoke DOS for specific services.  Let us examine these commands in the next section.

4.1.3	DOS Services Called by Forth

86EF202 is running under DOS.  Whenever it does an input/output operation, it sends a request to DOS by a software interrupt INT 21H.  DOS does what is requested, and Forth continues its operations.

Let me first list all the commands which request DOS services:

BYE	( -- )  			Exit eForth by calling DOS service 4C00 in INT 21H.

?RX	( -- c T | F )		Return input character and true, or a false if no input.

Call DOS service 6 in INT 21H.

TX!	( c -- )			Send character c to the output device.

Call DOS service 6 in INT 21H.

open	( fileAccess -- handle )	Open file and return the file handle. Call DOS service 3F00H in INT 21H

FileAcess is 3D00 for a read-only file and 3D01 for a write-only file.

create	( fileAccess -- handle )	Create file and return the file handle. Call DOS service 4000H in INT 21H

FileAccess is 0 for a read-write file, 1 for a read-only file.

close	( handle -- )		Close file by the file handle.  Call DOS service 3E00H in INT 21H.

read	( buffer len handle -- len-read )	Read a file by its handle.  Contents of file are stored in an area at

buffer.  Maximum size is len.  Return bytes actually read as len-read.

write	( buffer len handle -- len-writtn )	Write a file by its handle.  Contents of file are stored in an area at

buffer.  Maximum size is len.  Return bytes actually written as len-written.

READ	( bufffer len -- len-read , filename )	Open a file by name following READ, load it into buffer, and return

the bytes actually read.

LOAD	( buffer len -- )		Compile source code in a file just read into the buffer.

DOWNLOAD	( -- , <filename> )	Load a file by the name following DOWNLOAD and compile the

source code in the file.

UPLOAD  ( -- , <filename> )	Save current image to an EXE file using the file name following UPLOAD.

?RX receives characters you type on the PC keyboard, and TX! Sends characters to the PC screen.  The other commands deal with the file management in DOS, reading a text file, and writing EXE file.  These operations can be fairly complicated.  Since DOS knows how to do these things and takes care of all the complications, it is appropriate that Forth delegates these operations to DOS.  This is one of the main reasons why software engineering is easier than firmware engineering, where you have to take care of all the input/output operations yourself.

I further simplify matters by consolidating the low level DOS input/output operations into high level commands.  Therefore, to program applications, you only need two high level commands DOWNLOAD and UPLOAD.

The only other thing you have to worry about is what the Forth system has to do when it is booted up under DOS.  In eForth, after the Forth Virtual Machine is initialized properly, it executes a command ‘hi’ before running the text interpreter loop.  The command ‘hi’ simply displays the sign-on message.  After you finished an application, most likely you will want the Forth system to run your application on boot up, instead of running the Forth text interpreter.  This can be easily accomplished by replacing the address of the command ‘hi’ in the system variable ‘BOOT with the address of the command that runs your application.  In the example we showed in the last section, the commands are:

‘ GUESS ‘BOOT !

Tick (‘) is a Forth command which returns the address of the command whose name follows ‘.  Here it returns the address of GUESS, which runs the game of ‘Guess a Number.’  ‘BOOT returns the address storing the execution vector, now has the address of ‘hi.’  Store (!) places the address of GUESS in the variable location of ‘BOOT.  When this code image is saved to disk in the file TEST.EXE, it has all the information to run GUESS on boot up.  Following are the formal description of ‘hi’ and ‘BOOT.

hi	( -- )			Display the sign-on message of eForth.

'BOOT	( -- a )			The application startup vector.  It normally points to ‘hi’, but can point

to an application command.

With this set of commands, 86EF202 is a complete software development system in which you can compile, test, debug, and save application software.  The only feature missing in this system is a text file editor.  Instead of writing a text editor and include it in 86EF202, it is better to just use the text editors in DOS and Windows.  It is not difficult to write an editor in Forth.  Many people had done that many times.  However, editing text file is not an inherently useful function in an embedded system, and it is better to delegate the function to DOS.

4.1.4	Software Engineering

The above sections show you how to build software applications under DOS, using eForth system as the basis.  Since Forth provides almost all the system tools necessary to build application programs which can be invoked in the DOS environment, the software engineering process is greatly simplified.  Since this 8086 eForth system was designed to introduce the firmware engineering principles to be demonstrated in the 8051 eForth system, I have not take the time and effort to make it completely stand-alone.  You still have to go back to DOS or Windows to edit your source code file, and come back to Forth to compile and test the code.  It is not difficult to include the editing functions in Forth.  Many existing Forth system did it and worked quite well.  What I want to show you is the smallest, yet functional development system to develop application code.  This system is not tied down to a particular operating system or a particular CPU.  You can take it and implement it on any reasonably capable computer system.  If the system has a good operating system, you can take advantage of it and request its services whenever you need them.  This is the ultimate goal of software engineering.

Software engineering is the technology (or art, or black magic) to build applications to run on a computer which has an operating system.  The operating system provides services for the application program to access the standard devices and resources available in the computer.  The application only has to deal with non-standard devices and resources specific to the application.  Supposedly, the operating system makes it easier to build the application.  However, the reality is that the operating system like UNIX and Windows gets so huge and so complicated that it is very difficult for the programmer to understand it and to use the services effectively.  The software engineering splits into two branches: systems programming and applications programming.  People doing applications programming have to go to the priesthood of systems programming for advice and assistance on how to call system services.

In Forth, we try to return the control of the operating system back to the application programmers.  The operating system does not have to be huge and complicated.  If we know what service we need in our applications, we can go directly to the operating system and get the services we need.  We do not have to deal with the extra fireworks in the operating system.  To use a computer effectively, we only need two services: getting characters from a keyboard and displaying characters on a screen.  eForth builds a complete operating system based on these two services, and gives the user the direct access to the computer.  In 86EF202, it provides the access to disk files, and allows the user to load source code in text files and save code images in EXE files.  With it, software engineering can be mastered by average users.  The stratification of systems programming and application programming is eliminated.

4.2	Firmware Production with 8051 eForth

Building applications for 8051 is much more difficult than building applications for PC.  The two most important reasons are that there is no operating system supporting 8051 applications, and that the program in 8051 is generally residing in ROM which cannot be modified at the run time.  Programming 8051 applications requires a MDS (Microprocessor Development System) which used to be bulky, expensive, and very difficult to use.  Since PC’s are becoming cheaper and more powerful everyday, it has replaced the older generations of MDS and becomes the standard development system for 8051 and almost all the other microcontrollers.  Even so, programming 8051 is still tedious and difficult.  To get the job done, one will need lots of tools and utilities.  Here is a typical list:

Target 8051 system

PC running DOS or Windows

Text editor

8051 assembler

C compiler

Linker

8051 simulator

8051 In Circuit Emulator (ICE)

EPROM programmer

Logic Analyzer

Oscilloscope

The process of developing 8051 application firmware is:

Design system architecture and partitioning function modules

Write modules

Write system code

Compile/assemble source code

Link object code with library routines

Simulate executable code in software using simulators on PC

Download executable code to In Circuit Emulator to execute on 8051

Burn executable code into EPROM

Plug EPROM into target system and test with the logic analyzer

Many many iterations are required to flush out all the bugs and make the application work as expected.  A good In Circuit Emulator is the most important tool in debugging the system.  Now there are fairly good and fast C compilers with comprehensive libraries for 8051, and more people are using C instead of 8051 assemblers to program applications.

Nonetheless, the 8-bit microcontrollers like 8051 are still difficult to program, because the lack of operating system supports.

Forth is small and fast, and is an operating system.  It requires about 8 KB of memory space.  If the 8051 target system can afford this memory overhead, Forth will greatly simplify the programming task.  The fundamental concept of this Firmware Engineering Workshop is to take full advantage of Forth to facilitate firmware development for microcontrollers.  In fact, Forth replaces the In Circuit Emulator and streamlines all the steps in firmware development, because it supplies the basic input/output services, a command interpreter very useful in debugging, and a high language compiler to build substantial applications.  Given the necessary hardware resources, the target 8051 system can be a fully equipped Microprocessor Development System capable of producing and running firmware for a wide range of applications.

4.2.1	Assemble 8051 eForth

You have two versions of 8051 eForth source code.  The file 51EFORTH.ASM is my first attempt to port eForth to 8051.  It was a literal translation of the original 8086 eForth model, and was assembled using Microsoft MASM.  Because most of the commands were implemented in high level using the direct thread Forth model, the Forth interpreter was very slow, running on the poor 8-bit 8051.  I rewrote the code and changed to the much faster subroutine thread Forth model for CT100 Lab Board.  I also used the Metalink ASM51 8051 native assembler so that I could fully optimize the system.  I also added a set of new commands so that we can use it to build new firmware.  The result is 51SE23.ASM.  The source code of both 51EFORTH.ASM and 51SE23.asm are included in the distribution disk for you to study and to enhance.

The 8051 eForth source code is in the file 51SE23.ASM.  It can be assembled using Metalink’s ASM51:

C>ASM51 51SE23.ASM

Which produces an object code file 51SE23.HEX in the Intel Hex format.  This system includes all the commands needed to compile applications and test the applications.

If new commands has to be defined as primitive commands, the best ways is to include the new commands in the assembly source code.  One can follow the way primitive commands were defined in the assembly listing to add the new commands.  Re-assemble the 51SE23.ASM file and test the resulting object code.

The baseline eForth occupies the low memory from 0 to $2000, and grow upward when new commands are compiled.  It assumes that the memory between $8000 and $EFFF are populated with RAM and new commands can be compiled starting at 8100H.  The RAM area above 9000H can be used as a large file buffer if it is not used by application code compiled into this region.

It is a easy matter to change a few memory EQU’s to assemble a new 8051 Forth system to suit your own memory configuration in your target system.

4.2.2	Shadow ROM Model of the 8051 System

8051 are generally considered a microcontroller, suited only for small embedded applications.  In the most convenient 8751 configuration, the chip comes with 4 KB of EPROM, 128 bytes of RAM, and a suite of peripheral devices, including one UART, 2 counter-timers, and 4 parallel input/output ports.  This configuration is adequate for lots of dedicated controller applications, but not enough to be a general purpose microprocessor system.  However, in the form of 8031, it has no internal program memory but can have up to 64 KB of program memory and 64 KB of data memory.  It can thus be used to build systems which can take on much more demanding tasks and to be a self-sufficient firmware development system.

A very popular configuration is to have a 27C256 chip to supply 32 KB of EPROM, and a 62256 chip to supply 32 KB of RAM. The 32 KB of EPROM are generally allocated the memory space from 0 to 7FFFH, and the 32 KB of RAM are allocated the space from 8000H to FFFFH.  The data memory space and the code memory space are collapsed into a single 64 KB space so that new code compiled into RAM can be executed.  This arrangement gives us enough room in ROM to host the Forth operating system and very large application code, and lots of extra room in RAM to compile new applications at the run time.  10 years ago, 32 KB of EPROM and 32 KB of RAM were fairly expensive, and most applications could not afford such luxury.  Nowadays, the prices of the memory chips are so low that if you need two sockets for ROM and RAM, you might just as well put in 32 KB of each.

This configuration is adequate for most applications.  However, it is not possible to use this system to develop new firmware which can be burnt into the 32 KB EPROM.  To produce firmware to be used in this system, you still need a external development system to compile and assemble code which goes into the 32 KB EPROM.  The idea 8051 system which can be used as a target system and as a development system should have this special configuration: up to 64 KB of ROM and 64 KB of RAM.  The Forth system and all the application code are stored in ROM.  On power-up, all the code are copied from ROM into RAM, and the ROM is disabled.  Forth and the applications are now running in RAM.  Since Forth is running in RAM, you can erase old application code and compile new application code freely.  After the new application code is tested and fully debugged, the code image in RAM can be uploaded to a host computer, and then sent to an EPROM programmer to program a new EPROM.  This new EPROM can now replace the old EPROM in the target system.  When the system is powered-up, the new application will be copied into RAM with the Forth system and get executed.

This technique is often called the Shadow ROM Model, which is used extensively in mainframe computers and workstations to boot up operating systems.  This model can be used in 8051 and other microcontroller systems equally well.

Now, if we adopt this shadow ROM model with the Forth operating system, firmware engineering becomes a very easy subject which can be mastered in very short course like this one.  The Forth operating system takes care of all the system problems, and you only have to worry about the application.  The Forth programming language also makes it easy to write, test, and debug application code.  With these tools, any professional engineer can be trained to be firmware engineer in a week.

4.2.3	A Sample Session to Build 8051 Firmware

The CT100 Lab Board has an 8031 microcontroller, 32 KB of EPROM and 32 KB of RAM.  Unfortunately, I cannot configure it into the Shadow ROM Model.  Forth is in the EPROM, but applications must be compiled into the RAM.  The new code compiled in RAM cannot be converted to run in EPROM.  This board is thus useful in test and debug application code, but cannot produce new firmware to be committed to EPROM.

I have a couple of 8051 board modified to run the Shadow ROM Model.  It has one 27C256 chip containing the Forth system in the memory space 0-7FFFH, and two 62256 RAM chips.   One 62256 RAM is assigned to 8000-FFFFH, and the second RAM chip is in the region 0-7FFFH as the shadow of the ROM chip.  When the system powers up, the entire ROM is copied to this second RAM, and Forth starts executing in this RAM.  New applications up to 32 KB can be compiled into this RAM.  The new code image is uploaded to the host computer in the form of an Intel Hex file.  This code image can be programmed into a fresh 27C256 chip to replace the 27C256 chip.

Let us use the same LESSONS.TXT file as our application.  To build the new firmware, we go through the following steps:

<Turn on PC.   Bring out HyperTerminal.>

<Connect serial cable between PC and the 8051 system.>

<Apply power to 8051 system.  Forth system signs on.>

On 8051 system				On HyperTerminal

HEX

NP @ CP !

9000 6000 READ

Click Transfer/Send Text File

Select LESSONS.TXT

<press ESC to stop READ>

CSP ?  ( 55C2 bytes received )

9000 55C2 LOAD

CP @ NP !

NP 86 6 CMOVE

‘ GUESS ‘LOAD !

Click Transfer/Capture Text File

Select TEST.HEX

0 8000 SEND

Click Transfer/Capture Text File/Stop

<Turn off power>

Now, open a text editor in PC and inspect TEST.HEX file.  Delete characters that are not part of the Intel Hex data and save the file in pure text format.  Take the TEST.HEX file to an EPROM programmer and program a fresh 27C256 EPROM chip.  Replace the old Forth EPROM chip on the CT100 Lab Board with this new one.  Power up the CT100 system, and you will see that the ‘Guess a Number’ game running.

This procedure sounds complicated, and the exact meaning of the commands was not clearly explained.  However, this is what I called 'Firmware Engineering.'  All the tools to produce firmware systems are included in this eForth system.  If you spend the time and efforts to understand all these commands, you will be able to build firmware and call yourself a firmware engineer.

4.2.4	Firmware Utility Commands

Here is the formal description of the new commands we used in the last section to build firmware for the 8051 system:

READ		( addr  n -- )		Read text file from terminal into file buffer

LOAD		( addr  n -- )		Load source code in the file buffer.

SEND  		( addr  n -- )		Upload memory to host in Intel Hex format.

DUMP  		( addr  n -- )		Display n bytes of memory from location addr

CSP		( -- addr  )		Return address storing file length

CP		( -- addr )		Return address storing pointer to top of code area

HERE		( -- addr )		Return the pointer to top of code area

NP		( -- addr )		Return address storing ROM code size

READ waits for characters sent through the serial port and store them sequentially in the buffer beginning at addr and of n bytes long.  If the downloaded file has length less than n, you will have to terminate the READ command by typing ESC on the keyboard.  The number of bytes received is stored in the system variable CSP.  You must use this number and the addr of the buffer to execute the LOAD command.  LOAD compiles the commands received in the file buffer.

SEND is very similar to DUMP.  It dumps n bytes of memory from addr in the Intel Hex format so that the contents of this memory area can be programmed into an EPROM.  It sends a stream of  characters to the terminal.  You have to capture this character stream into a text buffer using HyperTerminal capturing utility.

When Forth is powered up, it assumes that the lower 32 KB are for ROM code, and the new code to be compiles goes into the first RAM chip area.  CP thus contains the address 8100H in the first RAM, where new code gets compiled.  (By the way, 8000-807FH is used to store system variables, and 8080-80FFH is used to store commands received from the terminal.)  If the Forth system supports the Shadow ROM Model, new code can be compiled into the lower 32 KB area occupied by the second RAM.  The address of the top of ROM code image is stored in the system variable NP.  If we are to compile the new application code into the lower 32 KB, the contents of NP must be copied to CP.  This is accomplished by the commands:

NP @ CP !

After these commands, new application code will be compiles to the top of the code image in the second RAM, building up a new code image for a new ROM.

After the new ROM image is compiled, we have to copy the pointer to the top of the new code image back into NP for the new ROM.  There are 6 bytes of vital information beginning at NP.  They are the pointer to the top of the code image, and the pointer to the name field of the last command defined in the new code image.   These 6 bytes have to be saved into the code image in the area from 86 to 8BH by the commands:

CP @ NP !

NP 86 6 CMOVE

After patching up the code image by these operations, the new code image can be saved by SEND to produce a new EPROM.

The code image needs these patches because we assume that the target 8051 has a 32 KB ROM at 0-7FFFH and a 32 KB RAM at 8000-FFFFH.  If the target 8051 system conforms to the Shadow ROM Model, and has RAM from 0 to FFFFH, these patches are unnecessary.

4.2.5	Firmware Engineering

The central theme here is that if we can put a Forth OS in the target microcontroller system, we can easily build firmware by loading application code on the top of the Forth OS.  The resulting code image can be programmed into a EPROM, and the job is done.  Firmware engineering thus becomes an easy proposition.  The underlying Forth operating system could seem to be a heavy overhead, not necessary to the final application.  However, with the underlying Forth operating system, the target application system has all the functionality of a firmware development system, which is of great value in prototyping, application programming, testing, and debugging.  Even after the product is delivered to customers, it will be very useful in on-site trouble shooting and maintenance.

Well, one might say: “This scheme is nice, but it is only applicable to the class of systems that have lots of ROM and RAM.”  This is true.  For system with less than 16 KB of ROM/RAM memory, this scheme does not work.  When the memory is limited to less than 8 KB of ROM and hundreds of bytes of RAM, you cannot shoe-horn an operating system into it, and you have to solve the problem the hard way, with assembler.  However, although we still see lots of these small microcontrollers around, the industrial trend is towards larger ROM and larger RAM, and we will see more and more of these larger systems.  This is the time for the firmware engineers to position themselves for these bigger systems, and ready themselves with the best tools to welcome the new opportunities.

The other important trend is the System-On-a-Chip (SOC).  The IC manufacturers can squeeze tens of millions of gates on a single piece of silicon chip.  A significant part of these gates will be allocated to CPU and memory, and no doubt one will have to put firmware on chip to run the system.  Forth will be the ideal firmware system to be integrated with all the hardware components in a SOC system.

4.3	Programming Style and Documentation

Over the years, Forth garnished a negative image in the computer industry.  Forth programs tend to be terse.  Using the stack to pass parameters hide the logic relationships among consecutive commands.  The process of abstraction moves rapidly, so that Forth programs sometimes are very difficult to read and to be understood. The freedom to name new commands and the ease in abstracting lists to new commands can result in very cryptic programs.  Here are some important guidelines to make sure that your program is written properly and documented properly for posterity.

4.3.1	Naming Conventions

As the Forth system is built from commands, the most convenient place to comment on the command is to choose its name.  You can use up to 31 characters in a name, and you can certainly compose a good name for each command.  It is enough to compose a short essay in the name.  In a compound name, you can separate the words by hyphens, underscores, or simply capitalize the first character of a word and connect the words together.  Examples are ELEVATOR_MOTOR_UP, elevator-motor-up, and ElevatorMotorUp.  If the names of commands spell out clearly what they do, they can make the source code readable like essays.

Forth interpreters can generally accept names up to 31 characters.  It is possible to write a sentence or even a small essay with 31 characters, if redundant word delimiters are avoided.  Concatenating words into a sentence with dashes or underscores are acceptable practices in ALGOL and PL/1 but wasteful.  In the Pascal style adopted by Apple, words are connected directly with the first character of each component word capitalized to signify the beginning of a word in a compound name.  The resulting names are eminently readable and quite economical in using the characters.

In the following table, the rules and examples are presented in detail:

Rules in Using Compound Names:

Words in a name are connected without spaces

The first character in a word is capitalized

The first character in the name, if not capitalized, can be used to signify the type of a Forth command.

Examples of Compound Names:

DriveToNextStopLight

OpenThirdValveInSecondRow

InitializeMotorDriveController

ParseNextString

MoveTempArrayToTargetArray

Variables and Constants:

vDelayMinutes					variable

vTempArraySize					variable

dvPrinterBufferPointer				double integer variable

yOutputDataArray				array

cCounterControlRegister				constant

cCounterStatusRegister				constant

dcTickCounts					double integer constant

4.3.2	Comments

The source code is the best documentation of the firmware system.  It is important to put comments with the code, and keep the comments up-to-date when you make changes to the source code.

Comment strings are placed in parentheses, like:

(  This is a comment. )

The left parenthesis is actually a Forth command, which tells the text interpreter to ignore all the characters, up to and including the right parenthesis.  Therefore, there must be at least one space after the left parenthesis.  However, you do not have to put a space in front of the right parenthesis, or after the right parenthesis, because the right parenthesis is a delimiting character, not a Forth command.

When you compile a large file, it is often helpful to print out messages occasionally to let the user see that compilation is in progress.  The command to print a message during compilation is .(, and it is used this way:

.(  Print this message during compilation. )

In many other Forth systems, the back slash \ is a line comment command.  It causes the text interpreter to ignore all the characters in the current line after \.  This command is in eForth.  However, it causes the text interpreter to ignore all the characters to the end of the file.  You can use it to stop compilation, but should not use it as a line comment command.

4.3.3	Stack Comments

Since Forth commands usually retrieve the input parameters from the data stack and leave the results also on the stack, it is absolutely necessary to comment on the input and output parameters on the stack so that the commands can be executed correctly.  You should follow the style of stack comments as shown in Section 1.5, which lists the most commonly used Forth commands.

If you like to be more elaborate in naming the stack items, you can add compound names to the character which designate the property of the stack item, like n, d, f, and a.

nErrorCode					integer

dCursorCoordinates				double integer

fBranchCondition				Boolean flag

aInputDataBuffer					address

Fully developed names provide in-line, ready documentation to the program, at zero runtime overhead.  Compile time overhead is insignificant.  Source code might be 10% larger, comparing with alternate methods of documentation at 200% increase in source code volume.

4.3.4	Version Control of Firmware

Version control is a very important issue in software engineering.  The same is true in firmware engineering.  It is important to keep track of changes programmers make in source code, so that we can always call out the source code of any version of the firmware system at any stage of development.  Sophisticated software development system has facilities to manage software revision automatically.

Since the eForth is in the target system and has nothing to do with the host computer, it cannot contribute much to the firmware version control.  All the work has to be done in the host computer in which source code is entered and maintained.  The source code is entered and edited using any of the text editors in Windows.  The source code must be in pure text files, which are best handled by the simpler word processors like Wordpad, Notepad, and the DOS EDIT.  You can use WORD, but you have to be sure that files must be saved in the pure text form.

Version control does not have to be very complicated.  You can do it yourself, but you have to be consistent and persistent.  Before you start a day, save the file you had edited into a new file with a new version number, and add a new comment line at the beginning of the file.  While making changes, add one comment line for any change you make.  This way, you are always working on the same source code file, while keeping a track of revised files you can always back track to.

My suggestion in firmware version control is very simple:

Open a Windows folder for a project

Open a major version folder in the project folder for any major revision

Open a minor version folder in the major version folder for any minor revision

Within the minor version folder, source code changes are designated by appending an update alphabet to the file name

Use the same Project/MajorRev/MinorRev/Update for all the source file and object file

For example, for project DEMO, major revision 2, minor revision 5, update C, we have the following files:

C:\DEMO\DEMO2\DEMO25\DEMO25C.TXT		Source code of main body

C:\DEMO\DEMO2\DEMO25\EXT25C.TXT		Source code of extensions

C:\DEMO\DEMO2\DEMO25\DEMO25C.HEX		Object code

Anytime you revised a source code file, add a line of comment to the beginning of the file indicating the new version number, the change date, and a brief description of the changes made.  A running group of comments will state clearly the progress of firmware development.

Concluding Remarks

What have we achieved?

I hope that I have convinced you that:

Forth is a useful technology to build firmware

Forth is a useful language to write applications

Firmware engineering is not difficult if you have good tools

You can roll your own Forth system using the eForth model

You can build software for 8086 in DOS environment

You can build firmware for 8051 systems

With these experience, I hope that you will be able to use Forth on your own to solve your application problems.  You have all the information necessary to build a new Forth system using the CPU of your choice.

I don’t have time to tell you everything about Forth.  There are many important topics in Forth you should investigate at your leisure.  Most of the materials are available on the Internet www.forth.org.  Here is my list:

Forth ANS Standard

IEEE 1275 Open Firmware Standard

Forth Scientific Library

Forth for Windows, Win32For

Forth for PC, F-PC

Forth for DOS/CPM, F83

Forth for early micros, fig-Forth

Commercial Forth from Forth, Inc

Forth Dimensions from Forth Interest Group

Cross compilation and meta-compilation

Multitasking and multiprocessing

Forth is a very powerful technology grossly neglected by the computer industry.  However, it belongs in the

tool box of every engineer who has to use microprocessors and microcontrollers to get things done.  My hope

is that now you have been introduced to this technology, make good use of it.

Appendix	eForth Quick Reference Card

Stack Comments:

Stack inputs and outputs are shown in the form:  ( input1 input2 ... -- output1 output2 ... )

Unless noted otherwise, all numbers are 16 bits.

flag 					Boolean flag, either 0 or -1

?					Truth value, either 0 or non-zero

char					ASCII character

b 					Any 8 bit number

x					Any 16 bit number

n					Signed 16 bit number

u					Unsigned 16 bit number

w					Wrap-around/circular 16 bit number

+n 					Positive 16 bit number

addr 					Address (same as u, unsigned)

d					Signed double (32 bit) number

xd, ud, wd, +d				Specify types of 32 bit number

Stack Manipulation Words

DUP    	(x1 -- x2)			Duplicate top of stack.

DROP   	(x -- )				Discard top of stack.

SWAP 	(x1 x2 -- x2 x1)			Exchange top two stack items.

OVER   	(x1 x2 -- x1 x2 x1) 		Make copy of second item on stack.

ROT	(x1 x2 x3 -- x2 x3 x1)		Rotate third item to top. "rote"

>R    	(x -- )  				Move top item to return stack for temporary storage.

R>     	( -- x)				Retrieve top item from return stack.

R@     	( -- x)               			Copy top of return stack onto stack.

Arithmetic Words

+      	(w1 w2 -- w3) 			Add w1 and w2.

-      	(w1 w2 -- w3) 			Subtract w2 from w1 (w1-w2=w3).

*      	(w1 w2 -- w3)         		Multiply. "times"

/      	(n1 n2 -- quot)       		Division, signed (quotient of n1/n2).

MOD    	(n1 n2 -- mod)        		Modulus, signed (remainder of n1/n2).

*/     	(n1 n2 n3 -- n4)      		Leave quotient of (n1*n2)/n3.

MAX    	(n1 n2 -- n3)         		n3 is the larger of n1 and n2.

MIN    	(n1 n2 -- n3)         		n3 is the smaller of n1 and n2.

ABS    	(n -- u)              			If n is negative, u is n's two's complement.

NEGATE 	(n1 -- n2)            		Two's complement.

Logic and Comparison Words

NOT    	(x1 -- x2)            			Bit-wise one's complement.

AND    	(x1 x2 -- x3)         		Logical bit-wise AND.

OR     	(x1 x2 -- x3)         		Logical bit-wise OR.

XOR   	(x1 x2 -- x3)         		Logical bit-wise exclusive OR.

<      	(n1 n2 -- flag) 			True if n1 less than n2.

=     	 (n1 n2 -- flag)       		True if n1 equals n2.

>      	(n1 n2 -- flag)       		True if n1 greater than n2.

0<     	(n -- flag)           			rue if n is negative.

Memory and Character String Words

@      	(addr -- x)           			Replace addr by number at addr.

!      	(x addr -- )          			Store x at addr.

C@     	(addr -- b)           			Fetch least-significant byte only.

C!     	(b addr -- )          			Store least-significant byte only.

+!     	(w addr -- )          			Add w to number at addr.

CMOVE  	(addr1 addr2 +n -- )  	Move n bytes starting at addr1 to memory starting at addr2.

COUNT  	(addr1 -- addr+1 b )	Move string count from memory onto stack.

Numberic Conversion Words

BASE   	( -- addr ) 			Contain radix for input-output conversion.

DECIMAL 	( -- )               		Set number base to decimal.

Terminal Input-Output Words

.      		(n -- )               		Display signed number with trailing blank.

U.     		(u -- )   			Display unsigned number with trailing blank.

." <text>" 	( -- )              		Compile <text> message. At run-time display text message.

.( <text>) 	( -- )           		Display <text> from the input stream.

CR     		( -- )                		Display a new line.

EMIT   		( char -- )           		Display char.

TYPE   		( addr +n -- )        	Display a string of +n characters starting at address addr.

SPACE  	( -- )                		Display a space.

SPACES 	( +n -- )             		Display +n spaces.

KEY    		( -- char )           		Get an ASCII character from the keyboard.  Does not echo.

Compiler and Interpreter Words

( comment) 	( -- )            		Ignore comment text.

,      		(x -- )               		Add x to parameter field of the most recently defined word.

ALLOT  	(+n -- )               		Add +n bytes to parameter field of the most recently word.

LITERAL 	(x -- )              		Compile number x.  At  run-time, x is pushed on the stack.

[      		( -- )                		Switch from compilation to interpretation. "left-bracket"

]      		( -- )                		Switch from interpretation to compilation. "right-bracket"

WORD <text>	(char -- addr )		Get the char delimited string <text>  from the input stream 							and leave as a counted string at addr.

Defining Words

: <name> 	( -- )              		Begin a colon command of <name>.

;      		( -- )                		Terminate execution of a colon  command.

CREATE <name> 	( -- )         	Dictionary entry with no parameter field space reserved.

'  <name> 		( -- addr)      	Find <name> and leave its address.

VARIABLE <name>	( -- )       	Define a variable.  At run-time, <name> leaves its address.

CONSTANT <name>	( x -- ) 		Define a constant.  At run-time, x is left on the stack.

Compiler Structure Words

EXIT   		( -- )                		Terminate execution of a colon command.

EXECUTE 	( addr -- )      		Execute the word command associated with addr.

QUIT 	  	( -- )                		Return to terminal, no stack change, no message.

ABORT 	( -- )                 		Return to terminal, clear stack, no message.

Conditional branch:

IF		( ? -- )     		If ? is zero, branches forward to <false> or after THEN.

ELSE		( -- )       		Terminate <true> clause, continue after the THEN.

THEN		( -- )			Terminate the IF-ELSE structure.

Definite loop:

FOR 		( limit -- )  		Setup loop. Repeat loop until limit is decremented to 0.

NEXT		( -- )     			Decrement index and repeat loop until index is less than 0

R@      		( -- w )              		Used inside a loop to get the current index value.

Indefinite loops:

BEGIN		( -- )			Start an indefinite loop.

UNTIL		( ? -- )     		Repeat <loop-body> until the ? flag is non-zero.

WHILE          	 ( ? -- )     		Repeat <loop-body> and <true> clause while the ? is non-zero.

REPEAT 	( -- )       		Unconditional backward branch to <loop> clause.

Dictionary and Vocabulary Words

HERE   		( -- addr )           		Address of next available dictionary location.

PAD    		( -- addr )           		Leave address of a scratch area of at least 84 bytes.

TIB    		( -- addr )           		Address of the text input buffer.

' <name> 	( -- comp-addr ) 		Look up <name> in the dictionary.

['] <name> 	( -- )            		Compile the word <name> in the input stream as an literal.   FORGET <name>	( -- )         			Delete <name>  and all words  added afterwards.

Utility Words

.S		( -- )			Display the contents of the data stack.

WORDS 	( -- )			Display all words which contain the string <text>.

DUMP 		( addr  n -- )		Dump n bytes of memory starting from addr.

SEE <name>	( -- )			Decompile the word <name>.

READ		( addr  n -- )		Read text file from terminal into file buffer

LOAD		( addr  n -- )		Load source code in the file buffer.

SEND  		( addr  n -- )		Upload memory to host in Intel Hex format.

FIRMWARE ENGINEERING

WORKSHOP

Dr. Chen-Hanson Ting

June 1999

Offete Enterprises