Mandelbrot Plots

Chen-Hanson Ting

About every 10 years, I would feel the urge to visit Mandelbrot plots, to see how the computer doing the plots.  In 1992, while we were promoting FPC, I tried to do the plots using 16 bit integers, 32 bit integers and floating point numbers.  At the time, I had a 386 PC running at 50 MHz.  It took about a minute to do a 640x480 plot on a VGA monitor.  The plots were impressive, but it was painfully slow, and not very interesting.  I was then hoping that the computer would get faster so I can interact with the plots.

In 2001, I started using F# and ported much of the tutorials on FPC to F#.  Mandelbrot plots were also ported to F#, and I used many of the plots as background graphics for the web site www.offete.com.  PC's then were much faster, but it was still slow, and took 10-20 second to do one plot.  Now, my Dell PC has a 1.5 GHz CPU, and it is time to see whether we can run the plots interactively.  I tried to push my PC to its limit by optimizing the assembly code in the Floating Point Unit (FPU), and also getting Windows XP operating system to do its best in rendering the plots.  Furthermore, I thought it is time to use the mouse to control the plotting.

The plotting routine in F# now does one plot in a second.  It is now interesting that I can explore the Mandelbrot sets almost interactively.  In another 10 years, I hope that I can build a 3D Mandelbrot landscape simulator, so that I can fly an airplane to explore all the sceneries in Mandelbrot landscape.

1. Mandelbrot Plot

Mandelbrot plots are based on the iterative equations on a complex number (x,y), starting at (x0,y0):

(xn+1,yn+1) = (x0,y0) + (xn,yn)2

The real part is:

xn+1 = x0 + xn2 - yn2

The imaginary part is:

yn+1 = y0 + 2xn*yn

When (xn,yn) is outside of the circle of radius 2.0, it diverges quickly.  Iteration is stopped when (xn,yn) is outside this circle, or when it reaches 127.  Each point (x,y) has a unique n between 0 and 127 associated with it.  Assign a color base on n to this point and plot all the points in color forms a Mandelbrot plot.

2. Color Palette

Coloring the pixels in a Mandelbrot plot is a simple yet complicated matter.  We have seen lots of impressive plots in black and white.  A flexible scheme of coloring will allow the user to explore the rich color space available on a PC.  Windows allows pixels to be represent in 24 bit color, with 8 bits assigned to red, green and blue.  These 24 bits occupy the lower 24 bits in a 32 bit word as 00000000rrrrrrrrggggggggbbbbbbbb in binary.

I chose a palette with 1024 entries, trying to represent contiguous colors in a rainbow.  The palette code and the RGB code are listed in the following table:

| Palette Code | Color | Red | Green | Blue |
| --- | --- | --- | --- | --- |
| 0 | Black | 0 | 0 | 0 |
| 256 | Red | 255 | 0 | 0 |
| 384 | Yellow | 255 | 255 | 0 |
| 512 | Green | 0 | 255 | 0 |
| 640 | Cyan | 0 | 255 | 255 |
| 768 | Blue | 0 | 0 | 255 |
| 896 | Purple | 0 | 255 | 255 |
| 1023 | White | 255 | 255 | 255 |

As the iteration number n is between 0 and 127, only 128 colors are used.  To assign different colors to n, I use two variables: COLOR contains the number of colors used, and BASE_COLOR contains the initial palette color to pick that many colors in the palette table.  The colors selected are evenly distributed in the palette table.

COLORS defines the number of colors used to paint a plot.

MORE_COLORS doubles number of colors.

LESS_COLORS halves number of colors.

HIGHER_COLORS moves BASE_COLOR to a higher color.

LOWER_COLORS moves BASE_COLOR to a lower color.

3.  External Representation of Coordinates

Iterative computations are carried out inside the Floating Point Unit FPU, and coordinates are represented by Intel 80 bit floating point number format.  However, coordinates and coordinate increments external of FPU are represented by 64 bit double integers.  The decimal point is placed between bits 59 and 60.  The sign bit is bit 63, and we have 4 bits to represent integer values from -8 to +7.  Following are some of the floating numbers and their external representations:

| Floating Number | External Representation in Hex |
| --- | --- |
| 7.0 | 7000,0000,0000,0000 |
| 4.0 | 4000,0000,0000,0000 |
| 2.0 | 2000,0000,0000,0000 |
| 1.0 | 1000,0000,0000,0000 |
| 0.0 | 0000,0000,0000,0000 |
| -1.0 | F000,0000,0000,0000 |
| -2.0 | E000,0000,0000,0000 |
| -4.0 | C000,0000,0000,0000 |
| -8.0 | 8000,0000,0000,0000 |

Coordinates are loaded into FPU as double integers, and right shifted by 60 binary bits to become actual floating numbers.

I was concerned that 64 bit precision in the external representation is not sufficient to support internal 80 floating numbers.  However, extensive testing showed that for Mandelbrot plots, numbers are rounded off at about 40 bits of precision, because 80 bits of internal precision tends to round off bits beyond 40 binary bits after squaring.  Hence the double integers are plenty wide to represent coordinates in Mandelbrot plots.

FLOAT_X0 and FLOAT_Y0 are used to store (x,y) coordinates of the center pixel in a Mandelbrot plot.

FLOAT_X and FLOAT_Y are initialized to point to (x,y) coordinates at the upper left corner of a Mandelbrot plot.  They are incremented in the x direction to plot a horizontal line and incremented in the y direction to start plotting the next vertical line.

FLOAT_D is the incremental value in x and y directions between neighboring pixels.

4.  Floating Point Computation

FPU is a classic FORTH engine with 8 levels of parameter stack.  I used 7 level to perform all iterations to compute the pixel value n.  Minimizing stack level used in computation tends to optimize the computation.  The computation is fully commented by listing the stack usage.

code xy ( -- n )

lea     ebp, -4 [ebp]

mov     [ebp], ebx

finit

fild    dword justification

fild    qword float_x

fscale

fstp    st(1)

fild    dword justification

fild    qword float_y

fscale

fstp    st(1)                  ( x0 y0 )

mov     ecx, max_iteration

mov     ebx, ecx

fld     st(1)                   ( x0 y0 x )

fld     st(1)                   ( x0 y0 x y )

@@1:    fld     st(1)                  ( x0 y0 x y x )

fmul    st, st                  ( x0 y0 x y x*x )

fld     st(1)                   ( x0 y0 x y x*x y )

fmul    st, st                  ( x0 y0 x y x*x y*y )

fld     st(1)                   ( x0 y0 x y x*x y*y x*x )

fadd    st, st(1)               ( x0 y0 x y x*x y*y x*x+y*y )

ficomp  dword four              ( x0 y0 x y x*x y*y )

fstsw   ax

sahf

jnb     @@2                     ( x*x+y*y=<4 ? )

fsubp   st(1), st               ( x0 y0 x y x*x-y*y )

fld     st(2)                   ( x0 y0 x y x*x-y*y x )

fmul    st, st(2)               ( x0 y0 x y x*x-y*y xy )

fadd    st, st                  ( x0 y0 x y x*x-y*y 2xy )

fadd    st, st(4)               ( x0 y0 x y x*x-y*y y0+2xy )

fstp    st(2)                   ( x0 y0 x y0+2xy x*x-y*y  )

fadd    st, st(4)               ( x0 y0 x y0+2xy x0+x*x-y*y )

fstp    st(2)                   ( x0 y0 x0+x*x-y*y y0+2xy )

loop    @@1

@@2:    sub     ebx, ecx

ret

end-code

5.  FORTH Code and Keyboard Interface

FORTH words at the highest level of Mandelbrot plots are listed in the following table, with keys assigned to these words:

| Key | FORTH Word | Functions |
| --- | --- | --- |
|  | MANDELBROT | Start Mandelbrot plotting loop |
|  | XY | Mandelbrot iteration on at pixel at (FLOAT_X,FLOAT_Y) |
|  | XX | Mandelbrot plot centered at (FLOAT_X0, FLOAT_Y0) with coordinate increment in FLOAT_D. |
| 0 | XXX | Plot an initial Mandelbrot plot. |
| r | RRR | Shift Mandelbrot plot right by 48 pixels |
| l | LLL | Shift Mandelbrot plot left by 48 pixels |
| u | UUU | Shift Mandelbrot plot up by 48 pixels |
| d | DDD | Shift Mandelbrot plot down by 48 pixels |
| = | +++ | Expand a Mandelbrot plot by a factor of 2 |
| - | --- | Shrink a Mandelbrot plot by a factor of 2. |
| 1 | scene_1 | Point of interest. |
| 2 | scene_2 | Point of interest. |
| 3 | scene_3 | Point of interest. |
| 4 | scene_4 | Point of interest. |
| z | RED | Change palette to shades of red. |
| x | YELLOW | Change palette to shades of yellow. |
| c | GREEN | Change palette to shades of green. |
| v | CYAN | Change palette to shades of cyan. |
| b | BLUE | Change palette to shades of blue. |
| n | VIOLET | Change palette to shades of violet. |
| ] | VIOLET | Decrease base_color. |
| [ | VIOLET | Increase base_color |
| } | VIOLET | Decrease color |
| { | VIOLET | Increase color |
| { | VIOLET | Increase color |
| ESC |  | Terminate Mandelbrot plotting loop. |

Note that ZXCVBN are 6 keys in the lower left corner of the keyboard, and they are assigned to the six rainbow colors in that order.

6.  Mouse interface

When PC was slow, it took a long time to do a Mandelbrot plot and the user interface was not very important.  Any key sequence to start plotting was adequate.  As PC are now fast enough, how to control the plotting becomes an issue.  The keyboard interface discussed in the last section is OK if a plot would take more than a second.  When we can make a plot in less than a second, a mouse interface is more intuitive than a keyboard interface.

I would like to have these 4 mouse function:

| LButtonDown | Hold down left button records mouse position to (mouse_x,mouse_y) |
| --- | --- |
| MouseMove | Drag a plot around by holding down left button and move mouse |
| RButtonDown | Expand the plot around cursor by click right mouse |
| LButtonDoubleClick | Shrink the plot around cursor by double click left mouse |

However, for some strange reasons, double clicking does not work on F# , and I have to use the control key to replace double clicking.  To shrink a plot, you have to hold down control key, and then hold down left mouse button and move mouse slight.  (See canvasE.f for mouse interface code.)

7.  Conclusion

On my no brand name desktop PC, which has an 800 MHz AMD Duron processor and 640 Mbytes of memory, it takes 2 seconds to do a Mandelbrot plot.  On my Dell Inspiron B120 laptop, which has an Intel Celeron M 1.4 GHz processor and 504 MB of RAM, it takes 1 second to do a plot.  Last Christmas I bought a new laptop for my wife.  It is a Dell Inspiron 1545, with a Pentium Dual Core T4300 processor at 2.1 GHz and 4.0 GB of RAM.  Running the Mandelbrot loop, the response is almost instantaneous.  I cannot determine how long it takes to do a plot.  Its speed must be in the order of 0.1 second.  This is what I call an interactive Mandelbrot machine.